COMMISSION’S DIRECTIVES

EARLIER DIRECTIVES

| Directive No. | Directive Description | Status |
| --- | --- | --- |
| 14. | The Commission directs TSDISCOMs to take steps for the installation of prepaid smart meters with latest technology for all interested consumers. | - In GO MS No.1, Dt:03.01.2016, Energy (Budget) Department, Govt. of Telangana, it was decided that all Govt. Departments should have prepaid meters at their own cost w.e.f 1st April,2016. - As per the above GO, TSNPDCL purchased 18812 prepaid meters for installation of meters to Govt.services and 15035 meters are fixed till now. - In the reference F No:20/9/2019-IPDS.GoI/MoP, DT:20.07.2021 Govt.of India has conveyed new scheme for implementation of “Revamped Distribution Sector Scheme-A Reforms based Results linked Scheme” with the objective of improving the quality and reliability of power supply to consumer. The scheme aim is to reduce AT&C losses. Smart metering is also covered under this scheme. - After sanction of the above scheme, fixing of smart meters will be taken up. |
| 15. | The Commission directs TSDISCOMs to submit a time bound action plan for replacement of existing meters with prepaid smart meters with two way communication in the interest of revenue realisation of TSDISCOMs. | - In the reference FNo:20/9/2019-IPDS.GoI/MoP, DT:20.07.2021 Govt.of India has conveyed new scheme for implementation of “Revamped Distribution Sector Scheme-A Reforms based Results linked Scheme” with the objective of improving the quality and reliability of power supply to consumer. The scheme aim is to reduce AT&C losses. Smart metering is also covered under this scheme. - After sanction of the above scheme, fixing of smart meters will be taken up. |

NEW DIRECTIVES

| Directive No. | Directive Description | Status |
| --- | --- | --- |
| 2. | The Commission directs TSDISCOMs to take strict measures to reduce the distribution losses and submit the quarterly report on the measures taken. The TSDISCOMs are also directed to ensure timely availability of quarterly energy audit reports in public domain. | The following measures are taken for reduction of T&D losses. - TSNPDCL installed 1385 No.s 2/1MVAR Capacitor banks for improvement of power factor and reduction of losses. The performance of these capacitor banks are being monitored daily from Corp.Office. Further, 87No.s 11KV 2MVAR and 20No.s 11KV 1MVAR Capacitor banks tender process completed and erection will be taken up shortly. - 449 No.s 600KVAR line capacitor banks are erected on over loaded 11KV feeders. - Power factor of 33KV feeders and at LVs of 33/11KV Substations are being monitored regularly for improvement Power factor and for further reduction of losses. - Further, the detailed project report (DPR) for reactive power compensation by installation and commissioning of 259 No.s 2 MVAR Capacitor banks at 33/11KV Sub Stations and 1569 No.s 600KVAR line Capacitor banks has submitted to National Load Despatch Centre for considering of grant from Power System Development Fund(PSDF) vide Lr.No:CGM/Projects/GM(P)/ DE(P-2)/ADE(p3)/F.PSDF/DNo.L1742/21, DT:19.01.2022. - In addition to the above, intensive inspections are being conducted regularly on high loss 11KV feeders for reduction of losses by attending exceptional, booking of theft cases, providing of seals to the meters etc., - For reduction of technical losses 11KV feeder segregation, erection of new lines 33/11KV Substations, new DTRs, load balancing etc., are being done regularly by monitoring the loads on 33KV and 11KV feeders. - New AGL services are being released in TSNPDCL only after providing necessary infrastructure i.e LT lines,11KV lines and Distribution Transformers and also by ensuring fixing of 2/3KVAR capacitor at consumer AGL pump sets. - Special drive is also being conducted for fixing of 2/3KVAR capacitors at Agl.pump sets whereever they are not available for reactive power compensation and efficient utilization of AGL pumpsets. - With the above, the T&D losses of the company are being reduced and the details of the losses are as follows: Sl No; Financial Year; % of Losses incl. EHT (Company Losses). 1; 2018-19; 9.90. 2; 2019-20; 9.04. 3; 2020-21; 9.02. 4; 2021-22; 8.76. 5; 2022-23; 8.14. - As per the Energy Audit Regulations for conduct of Energy Audit in Electricity Distribution Companies,2021, Dt:27.10.2021, TSNPDCL has submitted Quarterly and Annual reports as per the timeline, to Bureau of Energy Efficiency. The Energy Audit reports of TSNPDCL is uploaded in website up to Q1 of FY 2023-24. |
| 5 | The Commission directs TSDISCOMs to achieve 100% Agricultural DTR metering within a period of twelve (12) months and to furnish the quarterly progress on the status of implementation in this regard. | As per the Hon’ble Commission’s directive in RST Order for FY 2023-24, the licensee is directed to achieve 100% Agricultural DTR metering within a period of twelve (12) months. In this regard, the licensee submits that it is a policy matter and requires huge investment for installation of 2,32,526 meters for AGL DTRs an amount of Rs.378.55 Crs. is required and for maintenance of these meters an amount of Rs.136.16 Crs. on yearly basis is required. |