### Appendix 2: Power Purchase Forecast

### Power Purchase Forecast

#### Basis of estimating the quantity and cost of Power Purchased

This sub-section discusses the methodology and assumptions used for estimating the quantum and corresponding cost of power purchase of the Licensee for the second half of the financial year ending March 31, 2008 and for the financial year ending March 31, 2009.

With the implementation of Multi-Buyer Model (MBM) in the state from June 9, 2005, each discom has been allocated a certain share of the generating stations contracted by APTransco.  Non-conventional Energy sources have been allocated to the discoms based on their locations while the two mini-power plants LVS and Sri Vathsa have been allocated to EPDCL. The allocation percentages for the different discoms are as follows:

| S.No. | Name of the Distribution Company | Allocation Percentage |
| --- | --- | --- |
| 1 | EPDCL | 16.70% |
| 2 | SPDCL | 22.90% |
| 3 | CPDCL | 43.48% |
| 4 | NPDCL | 16.92% |

In the following paragraphs the capacities and availabilities of all the generating sources have been described. The actual energy availability in MU for each discom is simply the total generation availability for each source (except NCEs and the mini-power plants) multiplied by the percentage allocation as per MBM. The energy availability of NCEs for each discom has been shown separately.

#### Energy Availability

The NCE capacities are allocated to Discoms as follows on the basis of their location.

| NCE DISCOM - wise Allocation (Energy Availability - MUs) H2 2007-08 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| S. No. | Station Name | EPDCL | SPDCL | CPDCL | NPDCL | Total |
| 1 | NCE - Bio-Mass | 85 | 322 | 111 | 76 | 594 |
| 2 | NCE – Bagasse | 94 | 93 | 33 | 129 | 349 |
| 3 | NCE - Municipal Waste to Energy | 0 | 17 | 18 | 0 | 35 |
| 4 | NCE - Industrial Waste based power project | 46 | 0 | 6 | 0 | 51 |
| 5 | NCE – Wind Power | 0 | 2 | 43 | 0 | 45 |
| 6 | NCE - Mini Hydel | 11 | 23 | 35 | 12 | 81 |
| 7 | NCE – NCL Energy Ltd | 2 | 2 | 4 | 2 | 10 |
| Total Availability (DISCOM - wise) |  | 237 | 460 | 250 | 218 | 1165 |

The availability of power from non-conventional sources is shown below:

| NCE DISCOM - wise Allocation (Energy Availability - MUs) 2008-09 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| S. No. | Station Name | EPDCL | SPDCL | CPDCL | NPDCL | Total |
| 1 | NCE - Bio-Mass | 171 | 682 | 223 | 153 | 1229 |
| 2 | NCE – Bagasse | 189 | 234 | 66 | 260 | 748 |
| 3 | NCE - Municipal Waste to Energy | 0 | 34 | 37 | 0 | 71 |
| 4 | NCE - Industrial Waste based power project | 98 | 0 | 32 | 0 | 130 |
| 5 | NCE – Wind Power | 0 | 4 | 145 | 0 | 149 |
| 6 | NCE – Mini Hydel | 25 | 47 | 74 | 23 | 169 |
| 7 | NCE – NCL Energy Ltd | 3 | 5 | 9 | 3 | 20 |
| Total Availability (DISCOM - wise) |  | 486 | 1006 | 585 | 439 | 2517 |

#### Energy Requirement

Based on the availability shown above and the energy requirement from all the Discoms, the actual energy to be purchased has been projected as follows:

| DISCOMS | H2 2007-08 | 2008-09 |
| --- | --- | --- |
|  | MUs | MUs |
| EPDCL | 4,729 | 9,813 |
| SPDCL | 7,309 | 14,733 |
| CPDCL | 13,676 | 29,012 |
| NPDCL | 5,307 | 10,047 |
| Total | 31,021 | 63,605 |
| Total Energy Requirement (after APTransco and PGCIL Losses) | 32,745 | 66,751 |
| 2007-08 H1 Purchase | 28,868 |  |
| Annual Purchase | 61,613 | 66,751 |

#### Dispatch from Generating Stations:

The Dispatches from various Generators duly considering the allocation (16.92% of the Sector allocation) to NPDCL is as below:

NPDCL

| Station | H2- 2007-08 - Dispatch | 2008-09 - Dispatch |
| --- | --- | --- |
|  | MUs | MUs |
| APGenco – Thermal | 2,000.01 | 4,134.57 |
| APGenco – Hydel | 600.28 | 1352.15 |
| CGS (SR) | 825.03 | 1804.97 |
| NTPC Simhadri | 655.57 | 1236.01 |
| NTPC – ER | 264.11 | 487.59 |
| Other SEBs | 509.82 | 615.41 |
| IPPs | 457.08 | 1159.00 |
| APGPCL | 22.32 | 60.16 |
| Others | 217.88 | 439.31 |
| Total | 5,552.08 | 11,289.16 |

#### Summary of Power Purchase for Current Year 2007-08 and Ensuing Year 2008-09

Based on the availability, requirement and costs for each source, the summary power purchase cost for the entire Sector is projected as follows:

Power Purchase Cost for the State (FY 2007-08):

| FY 07-08 Projection (Current Year) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station | Power Purchase | Fixed Cost | Variable Cost | Other Costs | Total Costs |
|  | MUs | Rs. Crs. | Rs. Crs. | Rs. Crs. | Rs. Crs. |
| APGenco – Thermal | 21,293.12 | 1,763.72 | 2,574.27 | 75.25 | 4,413.23 |
| APGenco – Hydel | 9,515.94 | 0.00 | 224.00 | 0.00 | 224.00 |
| CGS (SR) | 12,863.33 | 717.72 | 1,349.74 | 108.72 | 2,176.19 |
| NTPC Simhadri | 7,208.95 | 458.72 | 776.63 | 18.16 | 1,253.51 |
| NTPC – ER | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Traders/ Naphtha | 3,605.16 | 0.00 | 2,248.42 | 0.00 | 2,248.42 |
| IPPs | 5,759.80 | 765.10 | 660.46 | 0.00 | 1,425.56 |
| APGPCL | 243.11 | 9.22 | 27.78 | 0.00 | 37.00 |
| Others | 2,018.49 | 40.84 | 591.39 | 0.00 | 632.23 |
| Total | 62,507.90 | 3,755.32 | 8,452.70 | 202.13 | 12,410.15 |

Power Purchase Cost for the State (FY 2008-09):

| FY 08-09 Projection (Ensuing Year) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station | Power Purchase | Fixed Cost | Variable Cost | Other Costs | Total Costs |
|  | MUs | Rs. Crs. | Rs. Crs. | Rs. Crs. | Rs. Crs. |
| APGenco – Thermal | 24,435.98 | 2,049.15 | 2,959.01 | 101.08 | 5,109.24 |
| APGenco – Hydel | 7,991.43 | 0.00 | 220.83 | 0.00 | 220.83 |
| CGS (SR) | 13,549.37 | 892.80 | 1,388.99 | 102.32 | 2,384.11 |
| NTPC Simhadri | 7,305.00 | 459.54 | 816.11 | 20.57 | 1,296.22 |
| NTPC – ER | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Traders/ Naphtha | 3,637.16 | 0.00 | 2,546.01 | 0.00 | 2,546.01 |
| IPPs | 6,849.91 | 855.27 | 778.07 | 0.00 | 1,633.34 |
| APGPCL | 355.55 | 12.66 | 37.32 | 0.00 | 49.98 |
| Others | 2,627.00 | 40.61 | 774.29 | 0.00 | 814.91 |
| Total | 66,751.39 | 4,310.03 | 9,520.64 | 223.96 | 14,054.64 |

#### Discom to Discom transaction

These transactions primarily arise from the need to minimize the power purchase cost for the sector in AP through a centralized merit order dispatch. The centralized merit order dispatch results in dispatch of a station based on its variable cost, with the lower variable cost stations dispatched ahead of the higher variable cost stations. This results in dispatch of a station even though a particular Discom (say Discom 1), who has a share in it, may not need the energy but the energy is utilized by some other Discom (say Discom 2). In such a case, as far as the generator is concerned, the payment for the dispatched energy needs to be borne by Discom 1. Therefore, the Discom 1 needs to recoup the expense from the user Discom (Discom 2). In this case, the Discom 1 is said to be “contributing” to the energy pool.

For the ARR filing 2008-09 the Licensees have taken 2007-08 Tariff Order Commission approved rate of Rs.2.2783 / unit for D-to-D Transactions.

The Table below shows the cost of energy for each DISCOM prior to and after the D-to-D energy transfers within the required quantum of energy for the current year:

Estimated D-D Transactions for Current Year

| DISCOM | Allocation from total energy (MU) | Energy Purchase Required (MU) | Surplus / Deficit (MU) | Power Purchase Cost before D-to-D transfer (Rs. Crs.) | Power Purhcase Cost after D-to-D transfer (Rs. Crs.) |
| --- | --- | --- | --- | --- | --- |
| APEPDCL | 10,422.10 | 9,635.46 | 786.64 | 2,087.33 | 1,908 |
| APSPDCL | 14,466.49 | 14,299.26 | 167.23 | 2,888.35 | 2,850 |
| APCPDCL | 26,324.69 | 27,702.79 | (1,378.10) | 5,129.80 | 5,444 |
| APNPDCL | 10,399.58 | 9,975.35 | 424.23 | 2,040.55 | 1,944 |
| Total | 61,613 | 61,613 | (0.00) | 12,146 | 12,146 |

The Table below shows the cost of energy for each DISCOM prior to and after the D-to-D energy transfers within the required quantum of energy for the ensuing year:

Estimated D-D Transactions for Ensuing Year

| DISCOM | Allocation from total energy (MU) | Energy Purchase Required (MU) | Surplus / Deficit (MU) | Power Purchase Cost before D-to-D transfer (Rs. Crs.) | Power Purhcase Cost after D-to-D transfer (Rs. Crs.) |
| --- | --- | --- | --- | --- | --- |
| APEPDCL | 11,305.01 | 10,298.69 | 1,006.31 | 2,411.43 | 2,182 |
| APSPDCL | 15,690.88 | 15,461.66 | 229.22 | 3,342.02 | 3,290 |
| APCPDCL | 28,466.35 | 30,447.22 | (1,980.87) | 5,930.37 | 6,382 |
| APNPDCL | 11,289.16 | 10,543.82 | 745.34 | 2,370.81 | 2,201 |
| Total | 66,751 | 66,751 | - | 14,055 | 14,055 |

For a particular Discom when the Discom-to-Discom transaction results in a cost (due to drawal), the cost is included in the power purchase cost. However, when it becomes a revenue (due to contribution), the revenue is treated as non-tariff income for the supply business as by nature this is an income from trading in power (albeit within the state between discoms).

Based on the principles described above, for APNPDCL, the cost of power purchased from all sources including D-to-D transactions is as follows:

|  |  | NPDCL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | H1 CY | H2 CY | CY | EY |
| Power Purchase from Generators | (in MUs) | 4,847.50 | 5,552.08 | 10,399.58 | 11,289.16 |
| Power Purchase from Generators | (in Rs. Crores) | 784.04 | 1,256.52 | 2,040.55 | 2,370.81 |
| Pool Transaction increment/ (decrement) | (in MUs) | 474.18 | (49.95) | 424.23 | 745.34 |
| Pool Transaction income/ (expenditure) | (in Rs. Crores) | 108.03 | (11.38) | 96.65 | 169.81 |

Appendix 3: Power Procurement Cost

# Power Procurement Cost for Current Year Second Half and Ensuing Year

#### Basis of estimating the quantity and cost of Power Purchased

This section discusses the methodology and assumptions used for estimating the quantum and corresponding cost of power purchase of the Licensee for the second half of the financial year ending March 31, 2008 and for the financial year ending March 31, 2009.

With the implementation of Multi-Buyer Model (MBM) in the state from June 9, 2005, each DISCOM has been allocated a certain share of the generating stations contracted by APTransco.  Non-conventional Energy sources have been allocated to the DISCOMs based on their locations while the two mini-power plants LVS and Sri Vathsasa have been allocated to EPDCL. The allocation percentages for the different DISCOMs are as follows:

| S.No. | Name of the Distribution Company | Allocation Percentage |
| --- | --- | --- |
| 1 | EPDCL | 16.7 % |
| 2 | SPDCL | 22.9 % |
| 3 | CPDCL | 43.48 % |
| 4 | NPDCL | 16.92 % |

In the following paragraphs the capacities and availabilities of all the generating sources have been described. The actual energy availability in MU for each DISCOM is simply the total generation availability for each source (except NCEs and the mini-power plants) multiplied by the percentage allocation as per MBM. The energy availability of NCEs for each DISCOM has been shown separately.

## Capacity break-up of major generating sources

APGENCO

The table below shows the installed capacities of the Thermal and Hydel generating stations of APGENCO as on 30-9-2007 including share in the interstate projects.  The DISCOMs purchase the entire generation of APGENCO under the terms of the PPA with the generator.

| Source | Installed Capacity (MW) |
| --- | --- |
| - THERMAL |  |
| Kothagudem-A | 240.0 |
| Kothagudem-B | 240.0 |
| Kothagudem-C | 240.0 |
| Kothagudem-D | 500.0 |
| Ramagundam-B | 62.5 |
| VTPS | 1,260.0 |
| RTPP-I | 420.0 |
| *RTPP-II | 420.0 |
| TOTAL THERMAL | 3382.5 |
| HYDEL |  |
| Interstate projects: |  |
| Machkund, Orissa (AP share 70%) | 84.0 |
| T.B. Station, Karnataka (AP share 80%) | 57.6 |
| State projects: |  |
| Donkarayi | 25.0 |
| Upper Sileru | 240.0 |
| Lower Sileru | 460.0 |
| Srisailam right bank PH | 770.0 |
| Srisailam left bank PH | 900.0 |
| Nagarjunsagar | 815.6 |
| Nagarjunsagar right canal PH | 90.0 |
| Nagarjunsagar left canal PH | 60.0 |
| Nizam Sagar | 10.0 |
| Pochampadu | 27.0 |
| Penna Ahobilam | 20.0 |
| Mini hydro | 12.6 |
| Singur | 15.0 |
| TOTAL HYDEL | 3586.8 |
| TOTAL APGENCO | 6969.3 |

* Note : The 2nd 210 MW unit of RTPP-II is likely to come under commercial   operation from Jan’08

CENTRAL GENERATING STATIONS

Southern region:

The Licensee has Power Purchase Agreements with Central Generating Stations of Southern Region to purchase power from NTPC (SR), NTPC (SR) Stage-III, NTPC -Talcher-II, Neyveli Lignite Corporation Ltd (“NLC”), Madras Atomic Power Station (“MAPS”) and Kaiga Atomic Power Station (“KAPS”). The allocated and unallocated share of the DISCOMs in the total capacity of the stations is provided below:

| Name of the Station | Capacity | AP Share (including un-allocated power) |  |
| --- | --- | --- | --- |
|  | MW | MW | % |
| NTPC-(SR) | 2100 | 735.25 | 35.01 |
| NTPC-(SR) STAGE - III | 500 | 183.86 | 36.77 |
| NTPC-TALCHER-II | 2000 | 430.3 | 21.52 |
| NLC TS II STAGE-I | 630 | 126.81 | 20.13 |
| NLC TS II STAGE-II | 840 | 219.23 | 26.1 |
| NLC TPS-I EXP | 420 | 0 | 0 |
| MAPS | 440 | 47.09 | 10.7 |
| KAIGA 1 & 2 | 440 | 148.32 | 33.71 |
| KAIGA 3 & 4 | *374 | 123 | 32.89 |
| TOTAL | 7744 | 2013.77 | 26 |
| NTPC-SIMHADRI | 1000 | 1000.00 | 100.00 |
| GRAND-TOTAL (CGS) | 8744 | 3013.77 | 34.47 |

Note: Out of 440 MW installed capacity of KAIGA 3 & 4, 66 MW is kept as Un- allocated and not allocated to any state. KAIGA unit-4 is expected to come under commercial operation in May’08.

IPPs

The following IPPs are under commercial operations in the State:

- 216MW gas-based plant at Jegurupadu by GVK Industries (“GVK”);

- 208MW gas-based plant at Kakinada by Spectrum Power Generation Ltd.,

- 351.49 MW gas-based plant at Vijaywada by Lanco Kondapalli Power Ltd (“Lanco Kondapalli”).  .

- 220 MW gas based plant at Samalkota, East Godavari District by M/s. Reliance Power Ltd. (formerly M/s.BSES).

AP Gas Power Corporation Ltd (“APGPCL”): Joint Sector

APGPCL is a joint sector gas-based power project. The allocation of power from this project is in proportion to the equity share capital of participating industries.  The total installed capacity of the project along with the DISCOMs share is as given below:

| Source | Installed Capacity (MW) | APDISCOMs Share (MW) | APDISCOMs Share (%) |
| --- | --- | --- | --- |
| Stage I | 100 | 16 | 16 |
| Stage II | 172 | 42.8 | 25.0 |
| Total | 272 | 58.8 | 21.62 |

Inter-State purchases

PTC, NTPC  V V N L, TPTCL

The Licensee proposes to procure power from Power Trading Corporation, NTPC Vidyut Vyapara Nigam Ltd., Tata Power Trading Company Ltd, Reliance Energy Trading Ltd. Adani etc., on need basis.

Non-Conventional Energy (NCE) Sources

The installed capacities of NCE projects in the state for FY 2008 and FY 2009 are as follows:

| Type of Project | FY 2008 (MW) | FY 2009 (MW) |
| --- | --- | --- |
| Bio Mass Power Projects | 192.75 | 192.75 |
| Bagasse Cogeneration Projects. | 162.7 | 204.7 |
| Wind Power Projects | 74.50 | 114.5 |
| Mini Hydel Power Projects | 44.45 | 54.95 |
| Industrial Waste Based Power Projects | 18 | 27.66 |
| Municipal Waste Based Power Projects | 12.6 60 | 12.74 |
| TOTAL | 505.06 | 607.3 |

Mini-Power Plants

APTransco had entered into adhoc agreement with LVS (36.8 MW) for purchase of power as per the direction of Supreme Court and had a PPA with Sri Vathsa (17.202 MW) power plant. These have been allocated to EPDCL. Licensee proposes power purchase from CPPs of VSP, NBFAL, Sponge Iron India Ltd, Heavy Water Plant etc., during the year 2008-09 on need basis at the rates approved by the Commission.

### Expected major capacity additions during FY 2009

| S. No. | Station Name | Type | Fuel | Capacity (MW) |
| --- | --- | --- | --- | --- |
| 1 | #### *GVK Extension Project | #### IPP | #### Gas | 220 |
| 2 | *Vemagiri Power Generation | IPP | Gas | 370 |
| 3 | *Konaseema EPS Oakwell Power Ltd | #### IPP | #### Gas | 445 |
| 4 | *Gautami | #### IPP | #### Gas | 464 |
| 5 | VTPS-IV | APGENCO | Coal | 500 |
| 6 | Jurala | APGENCO | Hydel | 78 |

The licensee expects new capacity additions of about 1499 MW from new IPPs subject to the supply of gas by GAIL.

Generation of 200 to 230 MW is expected from Jan’08 to Aug’08 from Vemagiri Power Generation by utilization of diverted gas from LANCO, Kondaplli.

BASIS OF ESTIMATION OF POWER AVAILABILITY FOR FY 2008 H2 AND FY 2009

#### APGENCO

Thermal:

The power availability for the second half of FY 2008 has been estimated by the Licensee based on actual performance for the first half of FY 2008 and the expected plant performance and maintenance schedules of APGENCO for the remaining six months of FY 2008.

| APGENCO Thermal ( Energy Availability- MUs) |  |  |  |
| --- | --- | --- | --- |
| Sl No | Station Name | FY 2008 H2 | FY 2009 |
| 1 | VTPS- I | 1486 | 3042 |
| 2 | VTPS- II | 1582 | 2890 |
| 3 | VTPS- III | 1577 | 3016 |
| 4 | VTPS-IV | 0 | 505 |
| 5 | RTPP-I | 1577 | 3033 |
| 6 | RTPP-II | 955 | 2853 |
| 7 | KTPS-A | 825 | 1704 |
| 8 | KTPS-B | 899 | 1680 |
| 9 | KTPS-C | 899 | 1729 |
| 10 | KTPS-D | 1800 | 3606 |
| 11 | RTS-B | 219 | 379 |
|  | Total | 11819 | 24437 |

Hydel:

The availability from Hydel plants of APGENCO for the second half of FY 2008 been estimated taking into consideration the inflows in to the reservoirs of Srisailam and Nagarjuna Sagar owing to good rainfall in the Krishna catchment areas during the monsoon.

The projection for ensuing year is based on the expected reservoir levels at end of current year and the assumption of a normal rainfall in the ensuing year.

The following table shows the projected availability for FY 2008 H2 and FY 2009:

| APGENCO Hydel ( Energy Availability-MUs) |  |  |  |
| --- | --- | --- | --- |
| Sl No | Station Name | FY 2008 H2 | FY 2009 |
| 1 | MACHKUND PH AP Share | 182 | 365 |
| 2 | TUNGBHADRA PH AP Share | 82 | 135 |
| 3 | USL | 319 | 450 |
| 4 | LSR | 689 | 1105 |
| 5 | DONKARAYI | 92 | 97 |
| 6 | SSLM (Right Bank) | 436 | 1449 |
| 7 | NSPH | 692 | 1708 |
| 8 | NSRCPH | 178 | 155 |
| 9 | NSLCPH | 89 | 71 |
| 10 | POCHAMPAD PH | 98 | 87 |
| 11 | NIZAMSAGAR PH | 3 | 11 |
| 12 | PABM | 5 | 8 |
| 13 | MINI HYDRO&OTHERS | 14 | 13 |
| 14 | SINGUR | 4 | 7 |
| 15 | LOWER JURALA | 9 | 146 |
| 16 | SSLM LCPH | 657 | 2185 |
|  | Total | 3549 | 7992 |

#### CGS

NTPC- Southern region: The estimate of availability from the CGS of the Southern Region for the second half of FY 2008 is based on the actual performance for the first half of FY 2008 and the expected performance for the second half of FY 2008.

The total power availability estimate from CGS for FY 08 H2 and FY 09 is as stated below:

| Central Generating Stations ( Energy Availability-MUs ) |  |  |  |
| --- | --- | --- | --- |
| Sl No | Station Name | FY 2008 H2 | FY 2009 |
| 1 | NTPC- (SR) | 2591 | 5139 |
| 2 | NTPC-(SR) STAGE - III | 651 | 1278 |
| 3 | NTPC –TALCHER-II | 1561 | 2882 |
| 4 | NTPC- SIMHADRI | 3875 | 7305 |
| 5 | NLC TS II STAGE- I | 389 | 828 |
| 6 | NLC TS II STAGE- II | 686 | 1440 |
| 7 | NPC-MAPS | 106 | 260 |
| 8 | NPC-KAIGA 1 & 2 | 365 | 817 |
| 9 | NPC- KAIGA 3 & 4 | 88 | 906 |
|  | TOTAL | 10312 | 20855 |

#### APGPCL

The Licensee has not projected any power available from the unutilized capacity of other shareholders in APGPCL in the 2nd half of FY 2008 as it is in the nature of infirm power.

As per the agreement with APGPCL, unutilized energy in stage-I has to be purchased by AP DISCOMs.

| APGPCL Allocated Capacity (Energy Availability- MUs ) |  |  |  |
| --- | --- | --- | --- |
| Sl No | Station Name | FY 2008 H2 | FY 2009 |
| 1 | APGPCL I - Allocated capacity | 43 | 64 |
| 2 | APGPCL II - Allocated capacity | 89 | 291 |
|  | Total | 132 | 355 |

#### IPPs

The availability of power from the generating stations of GVK, Spectrum, Lanco Kondapalli and BSES has been projected as declared by the IPPs.

| IPP's (Energy Availability-MUs ) |  |  |  |
| --- | --- | --- | --- |
| Sl No | Station Name | FY 2008 H2 | FY 2009 |
| 1 | GVK | 565 | 1124 |
| 2 | Spectrum | 632 | 1220 |
| 3 | Lanco Kondapalli (Gas) | 1063 | 2121 |
| 4 | Reliance | 440 | 881 |
|  | Total | 2700 | 5346 |

The projections for FY 2008 H2 and FY 2009 are based on existing gas supply position.

The availability of power from generating stations of GVK Extension, Vemagiri, Gautami and Konaseema has been projected based on the availability of gas from Jan 09. Expected operational PLF for new IPPs will be 47% for the first three months and fully operational from there onwards. Based on the projections, the energy availability from the New IPPs is given below

| New IPPs | FY 2009 (projected) All figures in MU |
| --- | --- |
| #### GVK Extension Project | 263 |
| Vemagiri Power Generation Ltd | 391 |
| Gautami Power Ltd | 468 |
| Konaseema EPS Oakwell Power Ltd. | 382 |
| Total | 1504 |

#### Availability from stations with DISCOM Specific Allocations

The NCE projects and the two mini power plants are not allocated on pro-rata basis but allocated to specific DISCOMs based on their locations. The availability from these sources for each DISCOM is as shown below:

##### Mini Power Plants:

| Mini-Power Plants Allocated to EPDCL ( Energy Availability-MUs) |  |  |  |
| --- | --- | --- | --- |
| Sl No | Station Name | FY 2008 H2 | 2008-09 |
| 1 | Srivathsa | 54 | 110 |
| 2 | LVS | 0 | 0 |
|  | Total | 54 | 110 |

| Sl No | Station Name | EPDCL | SPDCL | CPDCL | NPDCL | Total ( Type-wise) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | NCE - Bio-Mass | 85 | 322 | 111 | 76 | 594 |
| 2 | NCE - Bagasse | 94 | 93 | 33 | 129 | 349 |
| 3 | NCE - Municipal Waste to Energy | 0 | 17 | 18 | 0 | 35 |
| 4 | NCE - Industrial Waste based power project | 46 | 0 | 6 | 0 | 51 |
| 5 | NCE - Wind Power | 0 | 2 | 43 | 0 | 45 |
| 6 | NCE - Mini Hydel | 11 | 23 | 35 | 12 | 81 |
| 7 | NCE - NCL Energy Ltd | 2 | 2 | 4 | 2 | 10 |
|  | Total Availability ( DISCOM-wise) | 237 | 460 | 250 | 218 | 1165 |

##### Non-Conventional Energy (NCE) Sources

Energy availability in MU for FY 2008 H2

| Sl No | Station Name | EPDCL | SPDCL | CPDCL | NPDCL | Total ( Type-wise) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | NCE - Bio-Mass | 171 | 682 | 223 | 153 | 1229 |
| 2 | NCE - Bagasse | 189 | 234 | 66 | 260 | 748 |
| 3 | NCE - Municipal Waste to Energy | 0 | 34 | 37 | 0 | 71 |
| 4 | NCE - Industrial Waste based power project | 98 | 0 | 32 | 0 | 130 |
| 5 | NCE - Wind Power | 0 | 4 | 145 | 0 | 149 |
| 6 | NCE - Mini Hydel | 25 | 47 | 74 | 23 | 169 |
| 7 | NCE - NCL Energy Ltd | 3 | 5 | 9 | 3 | 20 |
|  | Total Availability ( DISCOM-wise) | 486 | 1006 | 585 | 439 | 2517 |

Energy availability in MU for FY 2009

#### Losses external to APTransco system

The losses external to the APTransco system are estimated to be 4.17 % for the H2 07-08 and FY 2008-09. This is applicable for procurement of power from Central Generating Stations.

#### Summary

A summary of the source wise current estimate of availability for FY 2008 H2 and FY 2009 is presented below.

| Summary Energy Availability Table- MUs |  |  |  |
| --- | --- | --- | --- |
| Sl No | Sector | FY 2008 H2 | FY 2009 |
| 1 | APGENCO - Thermal | 11819 | 24436 |
| 2 | APGENCO - Hydel | 3549 | 7991 |
| 3 | CGS (SR) | 10312 | 20855 |
| 4 | APGPCL | 132 | 355 |
| 5 | IPPs | 2700 | 6850 |
| 6 | NCE | 1166 | 2517 |
| 7 | Mini Power Plants | 54 | 110 |
| 8 | Bilateral Purchases | 3013 | 3637 |
|  | Total | 32745 | 66751 |

## Power purchase tariffs

### APGENCO

A fixed cost of Rs. 1503.69 crores for FY 2008 has been fixed by APERC and the same has been considered in the ARR. In addition, Rs.322 crores towards fixed cost for RTTP Stage has been considered on prorated operational months i.e (Rs 147.583 Crore). The total fixed cost is thus Rs.1663.72 crores. In addition a provision of Rs 100 Cr is made in the tariff order for FY 2007-08 due to likely financial impact for determination of tariff for FY 2007-08 and 2008-09 pending finalization of OP 27/06 and OP 4/07 filed by APGENCO. As per the orders dt 16-10-07 in IA No. 11/07 in OP 4/07 of APERC, the  payment of Rs 100 Cr is being made monthly on  proportionate basis.

The station-wise variable rates for APGENCO thermal plants (except VTPS-IV) that have been adopted for second half of FY 2008 and full FY 2009 are as per  the Table 8 of APERC Tariff order 2007-08 for Retail Supply . VTPS-IV rate is taken as 130 Ps./Kwh based on rate furnished by APGENCO.

| Station | Variable rate (Ps./kWh) |
| --- | --- |
| VTPS (I, II, III) | 125.83 |
| VTPS-IV | 130.00 |
| RTPP-I | 162.96 |
| RTPP-II | 147.00 |
| KTPS (A, B, C) | 99.53 |
| KTPS- D | 101.18 |
| RTS-B | 120.89 |

VARIABLE COSTS FOR SLBPH

APERC vide orders Dt 16-10-07 in IA No.11/07 in OP No.4/07 directed to pay the SLBPH energy Charges (Conventional Mode) monthly as per the methodology indicated in Para 475 & 476 of Tariff order 2004-05 subject to year ending adjustment.

For Srisailam Laft Bank powerhouse (SSLBPH), weighted average variable cost of Rs. 1.06/unit is considered (Non-Monsoon- Rs.1.63/unit, Monsoon-Rs. 0.64/unit). June to October is considered as monsoon and November to May as Non-Monsoon period. However, the above rates are for estimation purpose only and the actual payments will be made as per the methodology specified in paragraphs 475 to 477 of Tariff Order 2004-05 and reiterated in paragraph 375 of Tariff order 2005-06 .

### NTPC-TALCHER -II (2000 MW)

CERC vide order dt.20-9-2005 approved provisional fixed charges of Rs.990.7265 Crs. w.e.f. 1-8-2005.  Fixed charges payable by APDISCOMs for FY 2008 H2 and FY 2009 are estimated based on revised allocation of 21.515% w.e.f. 22-04-2007.  The variable rate for  Sep’ 2007 has been considered for projecting the variable charges for  H2 2008  and FY 2009.

- Total capacity charges of 2000 MW TALCHER STAGE-II = Rs. 990.7265 Crs

- Capacity charges payable by APDISCOMS for 430.3 MW entitlement = Rs.213.2 Crs.

- Variable rate = Rs.0.74/kwh

### NTPC (SR) (2100 MW).

CERC vide order dt.30-6-2006 approved fixed charges of Rs.479.71 Crs. for 2008-09 w.e.f. 1-4-2004.  Fixed charges payable by APDISCOMs for 2008-09 are estimated based on revised allocation of 35.01 % w.e.f. 07-04-2007.  The variable rate for Sep’ 2007 has been considered for projecting the variable charges for FY 2008 H2 and FY 2009.

- Total capacity charges of 2100 MW TALCHER STAGE-II (2008-09) = Rs. 479.71 Crs

- Capacity charges payable by APDISCOMS for 430.3 MW entitlement = Rs.167.95 Crs.

- Variable rate = Rs.1.05/kwh

NTPC (SR) STAGE-III (500 MW)

CERC vide order dt.15-10-2007 approved fixed charges of Rs.263.31 Crs. for 2008-09 w.e.f. 25-03-2005.  Fixed charges payable by APDISCOMs for 2008-09 are estimated based on revised allocation of 36.77 % w.e.f. 07-04-2007.  The variable rate for Sep’ 2007 has been considered for projecting the variable charges FY 2008 H2 and FY 2009.

- Total capacity charges for entire station = Rs. 263.31 Crs

- Capacity charges payable by APDISCOMS for 183.86 MW entitlement = Rs.96.82 Crs.

- Variable rate = Rs.1.14/kwh.

### NTPC- SIMHADRI

The fixed costs of NTPC (Simhadri) are projected based on CERC order dt.22-09-2006 and variable charges projected are based on Sep’ 2007 bill.

Fixed cost for FY 2009: Rs. 459.54 Crs

Variable rate : Rs. 1.12/kwh

NLC TS II

CERC vide order dt.23-03-2007 determined final tariff for 2001-04. Tariff for 2004-09 is yet to be decided by CERC, the Tariff  for FY 2004 and subsequent Tariff revision on account extra duty of Rs. 50/Tonne on coal w.e.f  01.08.2007 forms the basis for projecting costs for 2008-09.

Stage –I

Fixed cost based on the CERC order = Rs. 116.37 Crs/year.

Fixed cost payable by APDISCOMs for 126.81 MW entitlement = Rs. 23.38 Crs/year

Variable rate 		              = 125.1 Ps/Kwh

Stage –II

Fixed cost based on the CERC order = Rs. 177.49 Crs/year.

Fixed cost payable by APDISCOMs for 218.4 MW entitlement = Rs. 46.25 Crs/year

Variable rate = 125.1 Ps/kwh

MADRAS ATOMIC POWER STATION (MAPS) (440 MW):

Government of India, Department of Atomic Energy (Power Section) notified the tariff for supply of power from MAPS vide ‘Tariff Notification ‘dated 22.09.2006. As per the notification the tariff effective from 1.4.2006 is fixed at 181.18 Ps/Kwh. The tariff is subject to fuel and Heavy water adjustment charges. Taking into account the revisions subsequent to 1.4.2006 in insurance and in the rates of fuel and Heavy water, the tariff applicable for September 2007 is 187.08 Ps/Kwh and the same has been adopted for arriving at projected costs for H2 of current year and ensuing year. The share of AP in MAPS was revised to 10.7 % w.e.f. 07.04.2007. . Fixed cost/ unit has been arrived and the same has been incorporated in projections. The fixed cost considered is at Rs. 0.7129/unit and variable cost at Rs. 1.1584/unit.

KAIGA ATOMIC POWER STATION 1 &2  (KAPS) (440 MW):

On the request of AP, MOP/ GOI restored share of KAIGA 1 & 2 to AP on 28.12.07 and allocated 29.01 % share. As per latest revision w.e.f 07.04.07, the share of AP is 33.71 %. Government of India, Department of Atomic Energy (Power Section) notified the tariff for supply of power from KAIGA 1 &2 vide ‘Tariff Notification ‘dated 21.09.2005. As per the notification, the tariff effective from 1.7.2005 is fixed at 279.5 Ps/Kwh. The tariff is subject to fuel and Heavy water adjustment charges. Taking into account the revisions subsequent to 1.7.2005 in insurance and in the rates of fuel and Heavy water, the tariff applicable for September 2007 is 290.55 Ps/Kwh and the same has been adopted for arriving at projected costs for H2 of current year and ensuing year. Fixed cost is considered at Rs. 1.896/unit and variable cost at Rs. 1.0095/unit.

KAIGA ATOMIC POWER STATION 3 & 4  (KAPS) (440 MW):

Unit 3 (220 MW) of KAIGA Atomic Power Station was declared under commercial operation w.e.f 06.05.07. Unit 4 (220MW) is expected to come under commercial operation shortly. Pending finalization of Tariff for KAIGA 3 & 4 by Department of Atomic Energy, rates of KAIGA 1 & 2 are being adopted for making payments for supply of power from KAIGA 3 & 4. The KAIGA 1 & 2 tariff applicable for September 2007 has been adopted for arriving at projected costs for H2 of current year and ensuing year. The share of AP in KAIGA 3 & 4 is 32.89 %.  Out of 440 MW Installed Capacity of KAIGA 3 & 4, 66 MW is kept as Un- allocated and not allocated to any state. Fixed cost considered for projection is at Rs. 1.896/unit and variable cost at Rs. 1.0095/unit.

Other Costs for CGS (SR)

The Other Costs for CGS (SR) have been calculated based on the previous year’s cost that the Licensees incurred.

CGS (SR) incentive:

Incentives for CGS (SR) for 2007-08 and 2008-09 are arrived at based on CERC notification dt.26-3-04.

Total Incentive payable   :-Incentive shall be payable at a flat rate of 25.0 Ps/kwh for ex-bus scheduled energy corresponding to scheduled generation in excess of ex-bus energy corresponding to target PLF (Target PLF for NTPC Stations -80%, for NLC– 75 % and for MAPS & KAIGA Stations there is no incentive).

As per CERC orders, the incentive amount payable to NTPC & NLC is to be shared by the beneficiaries in the ratio of  energy scheduled beyond target PLF.

### APGPCL

The costs for purchase from APGPCL are as per projections given by APGPCL:

| Cost component |  |
| --- | --- |
| Demand Charges (Rs./KVA) |  |
| Stage-I | 75 |
| Stage-II | - |
| Stage-I |  |
| Fixed cost (Rs per kWh) | 0.52 |
| Variable cost (Rs per kWh) | 1.14 |
| Stage –II |  |
| Fixed cost (Rs per kWh) | 0.32 |
| Variable cost (Rs per kWh) | 1.03 |

There are no demand charges for Stage II of the generating station under the terms of the Memorandum of Understanding of April 1997.

### IPPs

GVK: - JEGURUPADU POWER PROJECT

The fixed cost is fully recoverable at 68.5% PLF. A variable rate has been computed by the Licensee based on the information provided in the bill of September 2007 drawn by the generator. Based on a calorific value of gas of 9642 kcal/scm, a station heat rate of 2,000 kcal/kWh and auxiliary consumption of 3 percent, average gas cost of Rs 4526/1000 scm, (From Sept’2007 bill) the variable cost as per the PPA formula is 96.79 Ps/kWh, which has been adopted for the cost computations.

The capital cost of this plant is Rs.816 crores. The fixed cost is Rs. 155.95 Crores per annum for FY 2008 and Rs.128.97 Crores for FY 2009. The fixed cost includes foreign exchange variations payable by APDISCOMs to the company as per the provisions of Power Purchase Agreement. The actual fixed cost as settled by the Licensee may be different from the estimates as presented above on account of the monthly Foreign Exchange Rate Variation (FERV).  The Licensee submits to the Hon’ble Commission to allow the Licensee to subsequently claim the change in fixed cost on account of FERV to the DISCOMs.

Deemed/Notional generation claims will be payable to the company up to 85% PLF as per the incentive formulae provided in the PPA.

SPECTRUM

The fixed cost is fully recoverable at 68.5% PLF. A variable rate has been computed by the Licensee based on the information provided in the bill of September 2007 drawn by the generator.  Based on the calorific value of 9711 kcal/scm, (From Sept’07 bill) station heat rate of 2,000kcal/kWh and auxiliary consumption of 3 percent, the variable cost works out to Rs 0.972 kWh, which has been adopted for the cost computations, with cost of gas as Rs. 4578 /1000scm including transportation charges.

A fixed cost of Rs 142.27 Crores has been adopted for FY 2008 and Rs 144.91 crores for FY 2009.  The fixed cost is inclusive of foreign exchange variations payable by APDISCOMs to the company as per the provisions of Power Purchase Agreement.

The actual fixed cost as settled by the Licensee may be different from the estimates as presented above on account of the monthly Foreign Exchange Rate Variation (FERV).  The Licensee submits to the Hon’ble Commission to allow the Licensee to subsequently claim the change in fixed cost on account of FERV from the DISCOMs.

Deemed/Notional generation claims will be payable to the company up to 85% PLF as per the incentive formulae provided in the PPA.

Incentives for GVK and Spectrum

Computations of incentives for GVK and Spectrum have been carried out based on the formula provided in the PPAs.  The PLF used for estimating incentives is based on the generator’s projections of plant performance for FY 2009.

GVK

Incentive payment 	= Equity x (PLF - 68.5).x. 0.00525

Equity 	    	= Rs 244 .8 Crores

Projected incentive for FY 2008: Rs.21.21 Crs.

SPECTRUM

Incentive payment 	= Equity x (PLF-68.5)x0.004 (IF PLF > 68.5 < 80.5)

Incentive payment 	= Equity x (PLF - 68.5).x. 0.005 (if PLF > 80.5 and < 85.5 )

Incentive payment 	= Equity x (PLF - 68.5).x. 0.006 (if PLF > 85.5 )

Equity 		    	= Rs 117.92 Crores

Projected incentive for FY 2009 : Rs.9.73 Crs.

LANCO KONDAPALLI

The fixed charges are fully recoverable at 80% PLF

Based on the the weighted average cost of gas, including transportation charges, ie Rs.5312.6 per 1000 SCM., gross calorific value of 9423 kcal/SCM, station heat rate of 1900 kcal/kWh and auxiliary consumption of 3%, the variable cost works out to Rs 1.104 kWh, which has been adopted for the cost computations (for FY 2008 and FY 2009) using the formula

Estimated fixed cost for H2 of FY 2008	:	Rs.168.16 Crs.

Estimated fixed cost for FY 2009		:	Rs.289.73 Crs.

RELIANCE POWER LTD (BSES)

The Fixed charge is fully recoverable at 85% PLF. Normative auxiliary consumption of 2.5% has been assumed under normal operations.

Based on the GCV of 9741.5583 Kcal/scm , Gas cost of Rs.4543 per 1000 scm, Station heat rate of 1900 Kcal/Kwh, Auxiliary consumption of 2.5%, the variable cost works out to Rs.0.909 Kwh

Based on the formula provided in the PPA, the fixed cost works out to Rs. 76.051 Crs. for second half of FY 2007-08.  The fixed cost for FY2008-09 is calculated as Rs.150.30 Crs.

### VEMAGIRI POWER GENERATION LIMITED

Vemagiri Power generation Ltd amended PPA on 2-5-2007 with modification in fuel definition making fuel as natural gas only. The COD of the project was declared on 16-09-2006. The project is awaiting for natural gas supply. Energy availability for FY 2008 H2 is not projected due to the non-availability of gas to the project. The energy availability for FY 2009 is based on expected additional gas availability. The fixed cost per unit is 94 Ps /Kwh . Based on GCV of oil at 9600 Kcal/SCM, basic price of reliance gas at US $ 4.2/MMBTU, Station Heat Rate at 1850 Kcal/Kwh, Auxiliary consumption at 3 %, the variable rate works out to 157 Ps/Kwh. Based on the availability of gas, the fixed cost has been projected on pro rata of Rs 36.8 Crore for the FY 08-09.

### GVK  EXTENSION POWER PROJECT

Energy availability for FY 2008 H2 is not projected due to the non-availability of gas to the project. The energy availability for FY 2009 is based on expected additional gas availability. The fixed cost per unit is 91 Ps /Kwh considering USD rate at Rs. 40/$ . Based on GCV of oil at 9600 Kcal/SCM, basic price of reliance gas at US $ 4.2/MMBTU, Station Heat Rate at 1850 Kcal/Kwh, Auxiliary consumption at 3 %, the variable rate works out to 157 Ps/Kwh. Based on the availability of gas, the fixed cost has been projected on pro rata of Rs 24.68 Crore for the FY 08-09.

### GAUTAMI POWER PROJECT

Energy availability for FY 2008 H2 is not projected due to the non-availability of gas to the project. The energy availability for FY 2009 is based on expected additional gas availability. The fixed cost per unit is 94 Ps /Kwh considering USD rate at Rs. 40/$.  Based on GCV of oil at 9600 Kcal/SCM, basic price of reliance gas at US $ 4.2/MMBTU, Station Heat Rate at 1850 Kcal/Kwh, Auxiliary consumption at 3 %, the variable rate works out to 157 Ps/Kwh. Based on the availability of gas, the fixed cost has been projected on pro rata of Rs 43.98 Crore for the FY 08-09.

### KONASEEMA EPS OAKWELL POWER LTD

Energy availability for FY 2008 H2 is not projected due to the non-availability of gas to the project. The energy availability for FY 2009 is based on expected additional gas availability. The fixed cost per unit is 94 Ps /Kwh considering USD rate at Rs. 40/$.  Based on GCV of oil at 9600 Kcal/SCM, basic price of reliance gas at US $ 4.2/MMBTU, Station Heat Rate at 1850 Kcal/Kwh, Auxiliary consumption at 3 %, the variable rate works out to 157 Ps/Kwh. Based on the availability of gas, the fixed cost has been projected on pro rata of Rs 35.90 Crore for the FY 08-09.

MINI POWER PLANTS

### LVS POWER LTD (36.8 MW)

LVS is a Mini Power Plant originally contemplated for sale of power to third parties.  The Commission directed the developer to sell energy to APTRANSCO.

However the company was directed to back down the plant from 3.4.2003 onwards due to increase in variable cost.  During the back down period fixed charges payable to the company from 11.10..07 to 10.10.2008 is Rs.28.62 Crs. and fixed charges payable from 11.10.2008 to 10.10.2009 is Rs.27.78 Crs.

### SRIVATHSA POWER PROJECTS LTD (17.202 MW)

It is also a Mini power plant originally contemplated for sale of power to third parties. However, as directed by the Commission, the developer has agreed to sell the energy to APTRANSCO withdrawing the court case against APERC order passed in O.P.No.70-A/2001, Dt. 4.5.2001 and after series of negotiations developer furnished their proposals with a capital cost of Rs.56.88 crores. Fixed cost for 5th year (2007-08) is Rs.1.66 per unit and Variable cost during 5th year is Rs.1.211 per unit, 73% PLF at generator terminals.  The fixed cost for 2nd half of 2007-08 is Rs. 6.303 Crores. For the year 2007-08, fixed cost of Rs.1.119 unit and variable cost of Rs.1.211 unit has been adopted.  The annual fixed charges work out to Rs.12.83 Crs for 2008-09.

### NON CONVENTIONAL ENERGY (NCE) SOURCES:

The Commission as per its Order in R.P.84/2003 in O.P.-1075.2000 dated March 23, 2004 has announced the tariff applicable to NCE projects with effect from 1.4.2004. Subsequently, the Commission has revised the incentive in these projects for generation above threshold PLF and tariff for Mini Hydel projects in its orders on review petitions filed by NCE developers/associations against orders dt.20.3.2004.  The NCE developers approached Hon’ble High Court on the Commission’s orders. Hon’ble High Court issued interim directions allowing APTRANSCO to implement the revised tariff in addition to payment of 50% of the differential amount between the old rate and revised tariff. Hon’ble High Court in June’ 2005 disposed the cases filed by NCE developers, giving liberty to the Developers to file appeals before Appellate Tribunal for Electricity, New Delhi.

The NCE developers filed appeals before Appellate Tribunal for Electricity .The Appellate Tribunal vide orders dt. 02.06.06 set aside the orders dt. 20.03.04 of APERC with a direction to APTRANSCO/DISCOMS to continue purchase of energy from NCE projects at a Tariff paid prior to the commission orders.

APTRANSCO/DISCOMS filed appeals before Hon’ble Supreme Court against the Tribunal order dt.02.06.06. The Hon’ble Supreme Court passed interim orders in the appeals and directed its office to issue notice to NCE developers. In the light of Hon’ble Supreme Court orders, the DISCOMS are continuing payment of 50 % of differential amount in addition to revised Tariff to NCE developers.

The GOAP filed an implead petition before Hon’ble Supreme Court to clarify its stand in the case. The Hon’ble Supreme Court has permitted the implead petition. The appeals are yet to be heard.

## ENERGY REQUIREMENT

Based on the availability shown above and the energy requirement from all the DISCOMs, the actual energy to be purchased has been projected as follows:

| DISCOMS | FY 2008 H2 | FY 2009 |
| --- | --- | --- |
|  | MU | MU |
| EPDCL | 4,729 | 9,813 |
| SPDCL | 7,309 | 14,733 |
| CPDCL | 13,676 | 29,012 |
| NPDCL | 5,307 | 10,047 |
| Total | 31,021 | 63,605 |
| Total Energy Requirement (after APTRANSCO and PGCIL Losses) | 32745 | 66751 |
| FY 2008 H1 Purchase | 28868 |  |
| Annual Purchase (Net of Sales) | 61613 | 66751 |

## SUMMARY OF POWER PURCHASE FOR YEAR 2007-08 AND ENSUING YEAR 2008-09.

Based on the availability, requirement and costs for each source, the summary power purchase cost for the Sector is projected as follows:

| FY 07-08 Projection (Current Year) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station | Power Purchase | Fixed Cost | Variable Cost | Other Costs | Total Costs |
|  | MUs | Rs. Crs. | Rs. Crs. | Rs. Crs. | Rs. Crs. |
| APGENCO - Thermal | 21293 | 1764 | 2574 | 75 | 4413 |
| APGENCO - Hydel | 9516 |  | 160 | 0 | 160 |
| CGS | 12863 | 718 | 1350 | 109 | 2176 |
| NTPC Simhadri | 7209 | 459 | 777 | 18 | 1253 |
| NTPC - ER | 0 | 0.00 | 0.00 | 0 | 0.00 |
| IPPs | 5760 | 765 | 660 | 0 | 1425 |
| APGPCL | 243 | 9 | 28 | 0 | 37 |
| Others | 5,624 | 41 | 2,840 | 0 | 2,881 |
| Total | 62,507.90 | 3,755.32 | 8,452.70 | 202.13 | 12,410.15 |

| FY 08-09 Projection (Ensuing Year) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station | Power Purchase | Fixed Cost | Variable Cost | Other Costs | Total Costs |
|  | MUs | Rs. Crs. | Rs. Crs. | Rs. Crs. | Rs. Crs. |
| APGENCO - Thermal | 24436 | 2049 | 2959 | 101 | 5109 |
| APGENCO - Hydel | 7991 |  | 221 | - | 221 |
| CGS | 13549 | 893 | 1389 | 102 | 2384 |
| NTPC Simhadri | 7305 | 460 | 816 | 21 | 1297 |
| NTPC - ER | - | - | - | - | - |
| IPPs | 6850 | 855 | 778 | - | 1633 |
| APGPCL | 356 | 13 | 37 | - | 50 |
| Others | 6264 | 41 | 3320 | - | 3361 |
| Total | 66751 | 4311 | 9520 | 224 | 14055 |

During the overflowing of reservoirs in the monsoon period, part of the energy has been sold to other licensees. The summary of sales and revenue is shown below

| Particulars | FY 2007-08 |  | FY 2008-09 |  |
| --- | --- | --- | --- | --- |
|  | Quantity | Revenue | Quantity | Revenue |
|  | Mus | Rs. Crs. | Mus | Rs. Crs. |
| UI sales | 683.10 | 173.55 | 0.00 | 0.00 |
| Inter State Sales | 211.94 | 90.56 | 0.00 | 0.00 |
| Total | 895.04 | 264.12 | 0.00 | 0.00 |

This quantum of energy towards sales has been shown with sign difference in the availability/ Dispatch sheet (4.1/ 4.2). Gross power purchase cost from all sources i.e. SGS, CGS,UI purchases, Inter state purchases and Pool purchases has been projected in the sheet 1.4 ( for both regulated and non regulated business).Whereas any revenue from sale of power has been projected in the sheet 1.4 (b) ( sales to Licensees) i.e. UI sales, inter state sales and pool sales.