# Eintroduction

This report presents the estimated cost of service for the Central Power Distribution Company Limited (APCPDCL), which is one of the distribution components of APTRANSCO for the test year starting on April 1, 2005 and ending on March 31, 2006.  The objective of this report is to classify into demand, energy and customer components in that year to its customers.

The steps involved in the analysis are:

- Forecast the energy and peak demand requirements for the power system in the test year;

- Forecast the energy and peak demand requirements at the transmission-distribution interface in that year;

- Forecast the energy and peak demand requirements for each customer category for that year;

- Estimate the costs of providing the energy and peak demand required for each customer category; and

- Classifying and allocating the above costs to APCPDCL customers at the retail level by voltage levels with in each customer category.

The forecasts of energy and peak demand requirements and losses are provided by the DISCOM. The costs are provided by the M/s.KPMG as part of the ARR filing and financial modelling. Financial data included in this model are those furnished by M/s.KPMG in November 2004.

( This space is intentionally left blank )

# COST OF SERVICE MODEL FOR APCPDCL

The cost of service calculations are based on the cost of service model COSTMOD, which handles an integrated utility, including generation, transmission and distribution.  The model is described in general terms in Appendix A.

In the present analysis we are dealing with the unbundled Andhra Pradesh electricity sector so that generation, transmission and four distribution companies are to be treated as separate entities. Because the separation of the assets of the former APSEB have already been segregated and indicated by M/s.KPMG in their financial models, the functionalization of the assets into generation, transmission and distribution is not required.

The model, as currently used, calculates the cost of serving all customers categories by voltage levels for each H.T consumer of APCPDCL.

All financial input into the model is taken from the M/s. KPMG ARR financial model, including revenue, expenditure and asset data.  To ensure complete compatibility between the data used for the cost of service work carried out by us and the annual revenue requirements, an interface file is used to transport data between the two models.  Alternatively the two models may be linked electronically.

The model may be rapidly changed to accept different assumptions regarding how the functionalization, classification and allocation are handled by the model. The following section summarises the main assumptions made in the various steps of the model.  In the following tables, the various options available are listed, with the selection adopted highlighted in bold lettering.

Module 1 Financial Reports

This is the first of four input files.  It contains the income and expenses data for the APCPDCL.  The data are entered via the interface file, which links COSTMOD with the M/s. KPMG financial model. The data is included for two financial years, 2004/05 & 2005/06.  Any of these years may be selected as the test year.  In the current work the test year selected is 2005/06.

Module 2: Revenue Accounts

In this module the data from module 1 may be modified to take into account any adjustments required to previously used data.  In the present work this feature is not required and has not been used.  It is retained for future use.

Module 3: Power System Statistics

This module includes the system data required for the cost of service calculation.  The energy and losses in the system are included along with the data regarding the customers responsible for the corresponding sales and losses.  The percentage loss quoted is the loss in the distribution system and hence accounts for the energy that is unavailable for sale to the retail customers.

APCPDCL customers are segregated into LT and HT customers which includes EHT (220 kV and 132 kV), Sub-transmission (33kV) and distribution (11kV and LV).  The EHT customers are included as APCPDCL customers, even though they may be connected at 220 kV or 132 kV.  In this study, technical losses experienced in EHT system are covered by EHT, 33kV, 11kV and L.T loads. Hence they need to be apportioned to all loads in the system.

The HT customer categories in the APCPDCL  system are;

- HT Customers - Industrial segregated – category I (at11KV,33KV & 220/132KV) - Industrial non-segregated – category II (at 11KV,33KV & 220/132KV) - Irrigation and Agriculture – category IV (11KV) - Railway traction – Category V (132KV) - Electricity Co-op Societies (at 11KV, 33KV) - Supply to Pondichery - Inter-state supplies - Temporary - Colony Consumption

The LT (400 Volts) customer categories in the APCPDCL system are;

- LT Customers - Domestic – category I - Non-domestic – category II - Industrial – category III - Cottage industries – category IV - Irrigation and Agriculture – category V - Public lighting – category VI - General purpose – category VII - Temporary – category VIII

The module includes the number of customers in each category and the consumption for each category.

Module 4: Operating Data

This is the final input module.  It includes the coincident and non-coincident demand data for the various customers.  The coincident demand is the estimated contribution of each category to the system peak demand and the non-coincident demand has been estimated from system load shapes derived and represents the peak demand of each customer category, irrespective of the time of day. Values used in this analysis are shown in Table 2-1.

Table 2-1

| Coincident Factors and Load Factors used | Coincidence Factor | Class Load Factor |
| --- | --- | --- |
| L.T Customers | Evening Peak |  |
| Domestic Category – Category – I | 98% | 47% |
| Non-Domestic – Category – II | 96% | 46% |
| Industrial – Category – III | 87% | 63% |
| Cottage Industries – Category IV | 87% | 63% |
| Irrigation and Agriculture – Category V | 15% | 47% |
| Public Lighting – Category VI | 90% | 42% |
| General Purpose – Category VII | 90% | 42% |
| Temporary – Category VIII | 90% | 42% |
| H.T Consumers |  |  |
| Industrial Segregated – Category I 11KV 33KV 220/132 KV | 85% | 64% |
|  | 85% | 64% |
|  | 90% | 67% |
| Industrial Non-Segregated – Category II 11KV 33KV 220/132 KV | 90% | 60% |
|  | 90% | 60% |
|  | 80% | 61% |
| Irrigation and Agriculture – Category IV 11KV 33KV | 15% | 47% |
|  | 15% | 47% |
| Railway Traction – Category – V (132KV) | 100% | 48% |
| RESCOs Cat – VI 11KV 33KV | 0% 90% | 0% 60% |
| Supply to Pondichery | 0% | 0% |
| Inter-State Supplies | 0% | 0% |
| Temporary | 90% | 60% |
| Colony Consumption | 98% | 47% |

The DISCOM peak demands, both coincident and non-coincident are estimated using basic load shape synthesis model. Load shapes of different categories of consumers are constructed based on the Load Shapes data collected from the DISCOMs. The following tabulation provides a derivation of the coincident and non-coincident peak demand, along with the assumptions for APCPDCL used in that derivation:

| CENTRAL | Energy (GWh) | Load Factors | Demand (MW) |
| --- | --- | --- | --- |
| Sales | 17,177 |  | 2297 |
| Loss as % of input | 17.91% |  | 20.63% |
| Losses in GWh | 3,747 |  | 597 |
| Sub Total | 20924 | 82.1% | 2,894 |

The load factor and coincidence factor included in the Model for each category are assumed based on a review of the characteristics of the loads and load mix in APCPDCL. One of the assumptions that requires discussion is the assessment of the timing of the system peak in the test year as this has a significant bearing on the coincidence factor for each of the customers. Recent data indicate that:

- The system peak demand of APCPDCL has occurred during Evening hours because of the predominant Domestic and Industrial loads in the DISCOM.

- So far power supply to agricultural consumers has been restricted to seven hours/day. APTRANSCO has divided the number of agricultural consumers into two groups and at any point of time, only one group of agricultural consumers would be supplied with electricity. Consequently, the load on the system is relatively flat with load factors hovering around 80% to 83%. Such a load management technique is desirable as it helps to reduce the generation requirement considerably. This supply policy to agriculture (i.e., 7 hours/day) is expected to continue in the foreseeable future. Accordingly, the system would experience peak load during evening hours because nearly 15% of total agricultural load will be on the system in addition to other consumers. On the other hand, the peak load in the morning hours is expected to be lower than the evening peak because only non-agricultural load would be supplied during the evening hours (i.e., 3-phase supply to agricultural consumers would be cut-off during evening peak hours)

- The load shape would change as and when the above restrictions are changed or lifted completely from the present practice.

- The load factors data of all Railway Consumers are collected from H.T billing data and the representative load shape data is collected from Ongole Traction sub-station for the financial year 2003-04 as this sub-station is constantly loaded and would closely represent the consumption pattern of the Railway Traction category for the A.P.

- Based on above considerations, it is assumed that the system peak demand in the fiscal year 2005/06 will occur in the evening. In addition, it is assumed that APCPDCL will also have morning peaks. This affects the selection of coincidence factor for each category.

Additional research is required to firm up these values.

Module 5: Rate Base Functionalization

No features of this module are required at this time since the functionalization of the asset base has been carried out in the transfer of assets work decided in the Transfer Scheme.

Module 6: Classification of Assets

In this module the assets may be classified into the three components demand, energy and customer.  The asset split at present is as follows;

- Land and Rights - Buildings - Hydraulic & other Civil Works - Plant and Machinery (substations) - Lines/Cables/Network - Vehicles - Furniture and Fixtures - Office Equipment - Spares etc

The options with respect to classification are as follows;

- Demand - Energy - Load factor - 50/50% demand/energy - customer - user defined - 50/50% demand/customer

In the model a number of different classifications have been used depending on the type of asset.  The following table indicates the selection made for each asset type.

| - Land and Rights - Buildings - Civil Works - Plant and Machinery (substations) - Lines/Cables/Network - Vehicles - Furniture and Fixtures - Office Equipment - Spares etc | - Demand - 50/50% demand/customer - Demand - Demand - Demand - 50/50% demand/customer - 50/50% demand/customer - 50/50% demand/customer - Demand |
| --- | --- |

Module 7: Asset Allocation

In this module the assets from Module 6 are allocated to the various customers.

A feature is provided to allocate the demand related assets to the various customers by the following.

- Coincident demand - Non-coincident demand - User defined

In the present model the coincident demand is used.

Module 8: Expenditure Functionalization and Classification

This module is used to functionalize the expenditures into the generation, transmission and distribution components. Since the functionalization of the asset base has been carried out in the transfer of assets work carried out in the Transfer Scheme, this function is not required.  The components of the expenditures listed below are retained for use in the later modules.  All expenditures have been assigned to distribution.

- Purchase of power - Generation of power - Repairs and maintenance - Employee costs - Administration and general expenses - Depreciation - Interest and financial charges - Other expenses

Module 9: Expenditure Classification

For the expenditures in module 8, this module classifies the expenditure into demand, energy and customer related items.  The options with respect to classification are;

- Demand - Energy - Load factor - 50/50% demand/energy - Customer - Rate base - Manual entry

The main expenditure of the DISCOM is for the purchase of electricity and this expenditure has been handled in two ways:

- The Energy cost is a “Flow-through” expenditure of APCPDCL’s share of the cost paid by APTRANSCO to its suppliers (APGENCO and IPPs) for the energy component of the power purchased. The demand cost is APCPDCL’s share of the sum of the demand costs incurred by APTRANSCO for the purchase of power plus the cost of operation of APTRANSCO.

- APCPDCL’s share of APTRANSCO’s cost of service is treated as power purchase cost.

All other expenditures have been classified as demand related.

Module 10: Expenditure Allocation

This module is used to allocate the expenditures to the individual customer categories.  The demand allocation may be made on a coincident or non-coincident demand basis. As discussed earlier the selection made in the model is to allocate the demand costs based on the coincident demand.

Energy costs are allocated based on the category energy consumption and the customer costs are allocated in proportion to the number of customers in the category.

Module 11: Non-Rate Revenue Functionalization, Classification and Allocation

As in other modules, the functionalization is not required.  The classification is to demand, energy and customer related components.  The different non-rate revenues that can be included in the module are interest from various sources such as staff loans and advances, on advances to suppliers and contractors, from banks; income from various sources such as from investments, trading, and other activities.  No such breakdown is provided at this time, so the non-rate revenues are clubbed together, as in the M/s. KPMG financial model.

In the allocation of these revenues, the allocation may be carried out using coincident or non-coincident peak demand for the demand component. In the model the allocation is currently based on coincident demand.

Module 12: Functionalization, Classification and Allocation of the Profit/Loss and Subsidies

The module treats Profit/Loss and Subsidies separately.

The profit/loss figure is taken as an input from the financial model of M/s.KPMG.  There is also the option of determining the profit allowed based on a rate of return on assets employed.  The profit used in analysis is the allowable return based on a return on assets.  This figure is determined in M/s.KPMG financial model.

The subsidy figure has been determined independently.  It is currently set at zero.

In common with the other modules the profit/loss and subsidies are organised by function (distribution), classified into the demand, energy and customer components and then allocated to the customers.

For the profit/loss figure, this has been classified where 100% is assigned to demand.

Module 13: Summary of Net Operating Costs

A summary of the results of the model are given in module 13 and these are discussed in the next section.

Module 14: Tariff Structures Adequacy and Analysis

A comparison of revenues and costs by customers is given in Module 14.

# results

Only one Scenarios was analysed:

Scenario: Each DISCOM is charged by APTRANSCO the full cost of serving that DISCOM using a two-part (energy and demand) tariff.

## Results

The following tabulation summarizes the results of the process:

- APCPDCL needs to handle 20923 MU which consist of sale of 17,176 MU to its customers and losses of 3,748 MU.

- Peak demand required by APCPDCL is 2,894 MW which consist of 2,297 MW to serve the customers, and 597 MW of losses in the system.

- The average unit cost of supplying the customers of APCPDCL is estimated at 2.86 Rs/kWh.

- The expected unit revenue from APCPDCL customers at current tariff is 2.42.

Table-3.1 compares the cost of service and revenue expected from current tariffs for the major categories. Table 3.2 provides detailed results for each category:

- Revenue and unit revenue at current tariffs.

- Allocated cost and unit allocated cost

- Revenue to cost ratio

- Weightage of each category.

Subsidies have not been considered.

APPENDIX A

# Description of THE cost of service model

## Introduction

The cost of service model (COSTMOD) is developed to carry out comparative cost/revenue analysis and develop rate alternatives according to economic/public acceptance criteria or assumptions. The objectives of this model are thus solely guided by the need to analyze rate performance on a comparative basis and to establish transfer price between the functional areas. The model provides an alternative to the exercise of responding to non-cost based corporate and/or political requirements.

The model is developed using a  spreadsheet program (Microsoft Excel) and is structured in such a way that it uses macro programming facilities, so that a person with basic computer skills can run the model with little effort.

The program is made user friendly as far as possible.  Options are provided in the menu interfaces in the model in the format of questions to answer, data to enter, and appropriate selections to be considered.

The model is designed to perform the following:

- act as a platform for data entry, edit and manipulation;

- maintain the data-base (for up to 4 financial periods);

- data analysis and processing;

- storage of  results for future analysis, and;

- Printing various reports.

The model calculates the cost of service in three steps;

- Functionalization, which is separating the assets and expenditures into the functional areas, generation, transmission and distribution.

- Classification, which for each functional area breaks out the assets and expenditures into three classes, demand, energy and customer related items.

- Allocation, which for each class breaks out the assets, expenditures,  revenues and subsidies into the customer categories to be served.

## Model Structure

COSTMOD is functionally structured into three distinctive components,

- Data Input/Edit;

- Process;

- Output.

The model is programmed in such a way that it requires minimum user interaction. User interactions are mainly limited to:

- answering questions and selecting menu-options and

- filling or editing data sheet that are presented to users.

## Using the model

### Data Input and Edit

This function provides interaction with the user for data entry for new periods where data are not yet provided in the data-base and to edit data for years which are already in the data base.

The four basic Modules for the  data input/edit are:

- MODULE  1  - Financial Reports / Income statement (Revenue Account)

- MODULE 2  - Financial Report / Revenue details (Element-wise Analysis of  Revenue).

- MODULE  3  - Power System Statistics

- MODULE  4  - Operating data

### Process

This portion of the model performs all the necessary calculations. Some data input is also required in the beginning of some of these modules.

There are nine Modules provided to perform technical calculations:

- MODULE 5   - Rate Base functionalistion

- MODULE 6   - Rate Base classification

- MODULE 7   - Rate Base allocation

- MODULE 8   - Expenditure functionalisation

- MODULE 9   - Expenditure classification

- MODULE 10 - Allocation of expenditure

- MODULE 11 - Non Rate Revenue functionalisation / classification /allocation

- MODULE 12 - Profit base functionalisation / classification /allocation

- MODULE 13 - Net operating costs

In these modules, the user has another level of interaction:

- The user may modify the functionalisation basis,

- the classification and

- the allocation factors.

The table below illustrates the choices built into the model; others can be added as desired.

Options Available for Functionalisation, Classification and Allocation

| Options | Functionalisation basis | Classification factor | Allocation factor |
| --- | --- | --- | --- |
| 1 | As per book data (default) | 100% Energy | Coincident Peak |
| 2 | Rate Base | 100% Demand | Non-Coincident Peak |
| 3 | Generation | Load Factor (Energy/Demand) | Energy Sales (including losses) |
| 4 | Transmission | 50% Demand/ 50% Energy | Number of consumers |
| 5 | Distribution | Distribution: 50% Demand / 50% Consumer | (to define) |
| 6 | Not Used | Rate Base | Manually entered data |

The model already has a set of default assumptions which may be changed or retained.  The model balances itself and the checks should always results in zero differences.

### Output

There is one module in this section that facilitates the analysis of the results generated by the model.

## Module Descriptions

A brief description of each Module is given below.

### Data Input Modules

Module 1 - Financial Reports

This Module accumulates all the data pertaining to the revenue account (incomes statement), on a fiscal year basis, and which are used in the costing procedure.   The source of this Module is the Annual Statement of Account, Statement 1 – Revenue Account.

Module 2 - Financial / Revenue Reports

The detailed sales revenue by class and type basis are provided in this Module. This Module uses the data from Annual Statement of Accounts, Schedules 1 and 2.

Module 3 - Power System Statistics

Here the information on the system load factor, revenue from sales of power, and number of customers by rate class is provided, which is used to determine additional allocation factors for the energy and customer-related. This Module has three sub-sections.

Sub-section 3.1: This section gives System Load Factor.  The discrimination between the load factors at the Generation, Transmission and Distribution levels is not considered.

Sub-section 3.2: This section provides the energy production and sales by customer category including voltage level breakup with in each  category, and for each function (Generation, Transmission, Distribution).  This provides the factors for the allocation of the energy related costs.

Sub-section 3.3: This section provides the number  of consumers in each customer classes and for each function (Generation, Transmission, Distribution).  This provides the factors for the allocation of the customer-related costs.

Module 4 - Operating Data

This portion of the model develops the demand allocation factors, inclusive of the demand losses, for each function (Generation, Transmission, Distribution) associated with the delivery system.  The data on the coincident demand and non-coincident demand of different customer classes are treated in this Module. This Module has two sub-sections.

Sub-section 4.1: This section provides the allocation factors based on the coincident demand.

Sub-section 4.2:   This section provides the allocation factors based on the non-coincident demand.

### The Process Modules

Module 5 - Rate Base Functionalisation

Functionalisation of Rate Base into the Generation, Transmission and Distribution categories is done in this module. The basic data for this module are derived from the Schedules 19 and 20 of the Statement 3 of  the Annual Accounts Statement.  A provision for the borrowing for working capita is included.

Two options are also provided in this module; one for the analysis including or excluding revenue subsidies, and the other is the adjustment or not of the Rate Base from grants and subsidies contribution towards the cost of capital assets.  A cross check with module 1 is provided in this module.

Module 6 - Classification of Rate Base

This module is divided into two separate tables starting with a summary of the functionalised assets split into demand, energy, and customer components. The second table gives the  breakdown of the assets into different component levels.  Criteria for classification alternatives  are provided  which can be selected  depending on the methodology being used.

Module 7 - Allocation of Rate Base

In this module the classified Rate Base is allocated to the various customer classes. Tables are provided at both the summary and detailed level.

Module 8 - Functionalization of expenses

This module starts with the expense portion of the cost of service study. Here the expenses are functionalised and the required cross references and functionalisation alternatives provided.

Module 9 - Classification of expenditure

This module is divided into two separate tables starting with a summary of the functionalised costs split into demand, energy, and customer components. The second table gives the breakdown of the costs into different component levels.  Criteria for classification alternatives are provided which can be selected depending on the methodology being used.

Module 10 - Allocation of expenditure

In this module the classified expenses from module 9 are allocated to the appropriate rate classes (customer) based on their responsibilities.

Module11 - Non-rate Revenue Analysis

The complete process of functionalisation, classification and allocation of  non-rate revenue is included in this module. Since any costs associated with the procurement of these revenues has already been allocated in the total expenses to the various rate classes, the process handled here will net out these items and leave only the costs to be recovered from tariffs.

Module 12 - Profits/losses Analysis

The complete process of functionalisation, classification and allocation of  profits including the revenue from subsidies, is done in this module.  The module is split into two parts.  The first part of the module determines the profits to be assigned to each rate class or alternatively.  No adjustment for losses is required as they are already reflected in the costs.

The second part of this module assesses which consumer categories should be allocated a part of the subsidy, then allocates the subsidy to those categories.

Module 13 - Summary of Net Operating Costs

This module summarises the net operating costs.  The total costs allocated are adjusted by the various non-rate revenue items, and by adding the profits earned or to be earned to arrive at net costs attributable to rate recovery.

The module also provides a summary of the unit costs by type (demand, energy and customer for each transmission customer.

### The Output Module

Module 14 - Tariff Adequacy and Analysis

This module provides the tariff structure and adequacy analysis.  Here, the class revenues are compared with the cost summary from module 13 to determine the level of recovery (performance) of each rate class. The revenue from each class is then measured against the assigned total net cost to determine the class performance; this is expressed as a percentage recovery and is commonly referred to as the Revenue/Cost Ratio.

APPENDIX B

Cost of Service - Scenario

TABLE OF CONTENTS

1	Eintroduction	1

2	COST OF SERVICE MODEL FOR APCPDCL	2

3	results	11

3.1	Results	11

A	Description of THE cost of service model	13

A.1	Introduction	13

A.2	Model Structure	14

A.3	Using the model	14

A.3.1	Data Input and Edit	14

A.3.2	Process	14

A.3.3	Output	16

A.4	Module Descriptions	16

A.4.1	Data Input Modules	16

A.4.2	The Process Modules	17

A.4.3	The Output Module	19

APPENDIX B

COST OF SERVICE MODEL – SCENARIO……………………………21