# Chapter 7 - Plans & Performance

## PLANS FOR UNDERTAKING LOAD RESEARCH

Requirement in the Guidelines :

Plans for undertaking load research to determine the load profile of consumers supplied under each tariff

(Guidelines – 10a)

APCPDCL’s Response.

APCPDCL  has furnished  profile of the consumers of different  classes with improved facilities of billing and load monitoring. APCPDCL  has established  a computerized  load monitoring  center as well as billing center  at Corporate Office and the  Load Research activity has been stepped up to meet the full requirements of guidelines of the Hon’ble  Commission in this regard.

Presently  the load  profiling  and monitoring  is being done at the 33/11 KV sub-stations  mainly for agricultural  load because  of the importance  of this category of consumers in the total energy consumption and also  its importance  in arriving at the energy losses.

APCPDCL  has provided metering arrangements on LV side 7964 Nos distribution transformers , feeding  predominantly  agricultural loads, including 6262 Nos distribution transformers  feeding  exclusively  agricultural  loads. The monthly readings of these meters are regularly  taken data is analysed to assess  the realistic level  of agricultural  consumption as per the directive  of the Commission.

APCPDCL is already  having sophisticated metering arrangements for the HT services. All these meters have the facility  of data retrieval  through MRI. APCPDCL  is under taking the load research  of all the HT services from the data retrieved through MRIs. In addition to the above, APCPDCL  have also embarked  on a  special project to study the Energy Account of Eighty Two  11 KV feeders from Fourteen 33/11 KV  sub-stations in Rangareddy  district. The project shall  assist  the company in assessing  the commercial  loss level and locating the pockets of theft  for further  physical  inspection and to detect the pilferage. APCPDCL  is also  studying  the feasibility  of introducing  remote meter reading, if considered economically viable. The  company has already started remotely monitoring the energy  flow data  input  these 14 sub-stations in order to assess the levels  of commercial  losses.

Regarding the load profile  of different  classes of metered consumers , it is to be stated  that the metering is already available for these consumers  and meter readings are being taken regularly. The details of monthly energy consumption and the connected load of various categories  is being  regularly in the  sales data base in the format prescribed by the Hon’ble  Commission. Thus  APCPDCL is able to provide a reasonably  realistic profile of the various  categories of consumers considering the class  load factor of the various classes of consumers.

The profile  of the various classes of consumers under different  sub-categories  are furnished, in the  prescribed formats , along with the tariff application.

For understanding  load research to determine the load profile  of the consumers under each tariff, it is  proposed to get the daily curves for 10 Nos . 11 KV  feeder feeding  predominantly 1) Domestic 2) Commercial ) Agriculture 4) LT Industries  5) HT Industries 6) HT Commercial.

## PLANS FOR IMPROVING THE SYSTEM POWER FACTOR

Requirement in the Guidelines :

Plans to improve the system power factor by providing an incentive for consumers to install correction capacitors. (Guidelines – 10b).

APCPDCL’s Response

All the consumers using induction motors have to install capacitors of appropriate rating to improve the power factor of their loads.

All the HT consumers & LT III B consumers are levied capacitor surcharge if the monthly power factor recorded is less than 0.9.

For consumers other than the above, if the capacitors of appropriate rating are not provided, 25% capacitor surcharge is being levied on the monthly-billed amount.

In respect of agricultural consumers, counseling is being done through local offices for erection of capacitors along with other DSM measures like providing of ISI Motor, erection of friction less foot valve, use of PVC Pipes etc to improve the efficiency of pumping.

2 MVAR capacitors at 33/11 kV substations and 600 kVAR capacitors on 11 kV feeders are being extensively used to correct the system power factor.

## PLANS FOR DETERMINING THE RELATIONSHIP BETWEEN Kwh CONSUMED AND CONNECTED LOADS

Requirement in Guidelines

Plans for determining the relationship between Kwh consumed and connected loads on which unmetered charges are levied. (Guidelines 10 c)

APCPDCL’s Response:

The assessment of consumption for the un metered LT agricultural consumers is being done based on the readings of the meters provided on the sample agricultural DTRs. These consumers are given three phase supply for regulated hours per day as per the approval of the Hon’ble Commission. Hence all the agricultural pumpsets are assumed to use power during the three phase supply timings.

The information of the connected load (in HP) and the recorded consumption as per the LV side meter readings of agricultural DTRs is used to arrive at the KWH / HP of the agricultural load, from DTR level to mandal and hence district level for assessment of agricultural consumption.

## PROGRAMME FOR CONVERTING UNMETERED CONNECTIONS TO METERED SUPPLY

Requirement in the Guidelines:

Programme for converting unmetered connections to metered supply, together with investment needed. (Guidelines 10d).

APCPDCL’s Response

APCPDCL submits that due to the GoAP policy of free power to agricultural category, there is very little scope of improvement in metering to individual agricultural consumers under optional requests and also under tatkal, as the scheme stands discontinued.

The comprehensive metering plan of agricultural services will be taken up as per the policy decided by APTransco and GoAP, under the guidance of the                        Hon’ble Commission.

## METHOD OF ASSESSING CONSUMPTION WHEN METERS ARE NOT INSTALLED / DEFECTIVE

1.	Requirement in the Guidelines

Method of assessing consumption when meter is not installed or is defective, together  with basis for such assessment (Guidelines 10 e).

2.	CPDCL’s Response

The methodology provided under clause 22.3.3 of the existing Terms and Conditions of Supply is adopted for assessment of consumption of a category of consumer, necessitated due to metering not being available or defective.

The CPDCL humbly requests the Hon’ble Commission for finalisation and approval of the draft revised terms and conditions of supply.

## PLANS FOR RATIONALISING EXISTING MANPOWER

Requirement in the Guidelines

Plans, both short-term and long-term for rationalization of existing manpower (Guidelines 10f)

APCPDCL’s  Response:

In order to retain the existing trained manpower the Assistant Engineers who were appointed on Contract basis are regularized based on the performance report of past two years. Similarly the issue of regularizing the Contract Sub-Engineers and LDCs is also under examination. To cater to the ever-increasing consumers and to provide better service, a study has been conducted to optimize/rationalize the workload, for improving the performance of the Operation Sections, where in it was recommended to bifurcate 76 overloaded Operation Sections and form 50 additional Operation Sections. On the same line, it was felt necessary that an additional 81 Sub-Engineer posts are to be sanctioned for providing necessary support. Certain Sub-Divisions in Construction, Quality Control and M&P/Protection/Operation Sub-Divisions in Ranga Reddy(North) were formed by making local adjustment and  sanction of additional 14 ADE(Elec.) posts for the above places shall be obtained. In view of the presence of more No of Operation Circles in CPDCL compared to other DISCOMs, on the administrative front it was assessed that an additional 7 Personnel Officer and 4 Junior Personnel Officer posts may be added to the existing strength for smooth functioning.

It was decided to strengthen the Audit wing and in the process Charted Accountants were appointed for each of the 10 Operation Circles for continuous auditing of the activities and for keeping a vigil on the procedures in vogue and for curtailing the waste expenditure. As a part to bring out competency, improve internal efficiency and practice latest business standards personnel having higher qualifications shall be appointed from open market.

Workload Norms, Job descriptions for each post and minimum staff requirements for each office are under examination and pending finalization of the Norms; to lessen the financial commitment, reduction of staff is contemplated while formation of new offices after exhausting the option of internal adjustment of posts. As a cost cutting measure the Private Accounting agencies are continued and non-routine works are awarded on contract basis duly following the procedures in vogue. As a stop-gap-arrangement, till the vacancies are filled certain works such as Driving , Typing and Office Assistance are being entrusted to private agencies to carry out the works on yearly contract basis, to avoid dislocation of works. Steps are taken that the contractor employs personnel with adequate qualification and checks are done about the compliance of the statutory obligations.

The performance all the employees is being continuously monitored based on certain objectives, key performance indicators, pre-defined targets with an Object to increase efficiency and improve performance standards of the employees, bring an overall improvement in the performance of the organization. This system is also used to assess and record the performance level of all the employees based on which corrective actions are being taken. Consistent good performers are identified and it is planned to give award/rewards and it is planned to link the performance to the Productivity Linked Incentive Scheme which is yet to be launched. Also based on the performance evaluation right personnel is moved to the right place.

With a vision that all employees shall be systematically trained , CPDCL has Developed need based training calendar covering 15000 Mandays in our training center by maintaining 100% attendance. On the name of Training on Wheels conducted training programmes at Divisions. Soft skills have been developed to cater to the changed scenario and many programmes are conducted to improve the work culture. It is proposed to decentralize the training activity and in this line programmes are scheduled in Rural Sections/Divisions.. One rural Circle (Mahabubnagar) and one City Circle (Central) were made as model circles by implementing “Total Quality Management” and in the due course shall be extended to all the  Circles.

## Expansion Plan for a Capital Investments

Requirement in the Guidelines

Expansion plan for all capital investment (Guide Lines 10g)

APCPDCL’s Response

As on 31st March 2004 the existing substations, lines and other relevant statistics of APCPDCL

## Table 10.g.1 Existing Lines and Substations:

| Sl. No. | Particulars | Unit | Quantity |
| --- | --- | --- | --- |
| 1. | 33 KV Substations | Nos. | 912 |
| 2. | 33 KV Lines | KM | 11652 |
| 3. | 11 KV Lines | KM | 73756 |
| 4. | Distribution Transformers | Nos. | 118573 |
| 5. | Towns Electrified | Nos. | 74 |
| 6. | Villages Electrified | Nos. | 6496 |
| 7. | Hamlets Electrified | Nos. | 6516 |
| 8. | Dalitwadas Electrified | Nos. | 11146 |
| 9. | Weaker Section Colonies Electrified | Nos. | 5478 |

During 2004-05, the programme of addition of substations is as follows:

Table 10.g.2 Planned Substations

| Voltage (KV) | Nos. of Substations |
| --- | --- |
| 33 KV / 11 KV | 68 |

## Table 10.g.4 Planned Release of Services to Weaker Sections

| Category | Number of Connections |
| --- | --- |
| Hamlets | 656 |
| Dalitwadas | 959 |
| Weaker Section Colonies | 328 |
| Tribal Hamlets | 487 |
| Agricultural pumpsets | 16500 |
| Laying of conventional lines in villages with SPV panels | - |

This investment is broadly meant for the following purposes:

Rs. in crores

| Investment | 2004-05 | 2005-06 |
| --- | --- | --- |
| System Improvement | 94.82 | 121.00 |
| Loss Reduction |  |  |
| a) HVDS | 90.74 | 92.00 |
| b) Metering | 11.67 | 60.00 |
| Rural Electrification | 112.77 | 62.00 |
| T & D Improvements | 45.00 | 50.00 |
| Release of new services | 45.00 | 40.00 |
| Total | 400.00 | 425.00 |

## NUMBER AND DURATION OF INTERRUPTIONS ON THE NETWORK

1.	Requirement in the Guidelines:

Number and duration of supply interruptions on the distribution networks, separately for 33KV, 11KV and LT systems. Details of steps proposed to improve performance and monitoring. (Guidelines – 11a).

2.	APCPDCL’s Response:

| The number and duration of interruptions and breakdowns, recorded at substations for all 33KV and 11KV feeders are given in table 11a1 and table 11a2 respectively. Regarding interruptions on the LT system, the complaints received at FOC (Fuse of Calls) centers are categorized under the following heads: Service wire defects LT fuse blown LT line fault Horn gap fuse blown DTR failure Each circle of APCPDCL has a control room for load monitoring and in turn to report to the LMC located at Corporate office for monitoring the interruptions of 33KV, 11KV and LT distribution networks as well as the rectification measures that were carried out. The LMC monitors power supply position round the clock in APCPDCL. APCPDCL has fixed the time limits within which the restoration of normalcy of lines/ equipment are to be achieved. It is the responsibility of the field officers to adhere to these standards. The field officers also conduct pre-monsoon inspections and rectify and defects noticed by them during this process. The projects wing monitor overloaded lines and equipment and take necessary steps to strengthen or expand the net work whereever required. Electrical clearances between trees and line/equipment are checked regularly to minimize the interruptions due to fouling of branches with live conductor etc. The APCPDCL also provided additional vehicles to operation sections to minimize the time for the breakdowns and for regular maintenance of lines and distribution transformers. The APCPDCL is also taking steps to minimize the interruptions to consumers by providing HVDS system, which is having lower capacities of distribution transformers and decreasing the ratio of transformer to the consumer. Providing of logic controllers for ensuring regulated hours of supply. In order to further stabilize the supply conditions in the primary distribution system 68 Nos 33/11KV substations, 380KM of 33KV line and 315KM of 11KV line and distribution transformers are also proposed in the areas of over load and low voltage pockets. During this financial year 10 Nos 33/11KV substations are commissioned in the constituency of Dharmavaram, Nallamada and Anantapur of Anantapur circle, Nandyal, Aloor and Eemmiganur of Kurnool circle, Ramayampet and Ramayampet of Medak circle Chevella of RangaReddy South circle P&T Colony, Uppal of RanagaReddy North circle. In addition to this to provide 24hours supply to villages, single phase transformers are erected for the village loads and this has resulted in good improvement in voltage levels. 1944 Nos. feeders (i.e., 288 Nos. in Anantapur circle, 266Nos. in Kurnool circle, 393 Nos. in Mahabub nagar circle, 357 Nos. in Medak circle 527 Nos. in Nalgonda circle 113 Nos. in Ranga Reddy South circle) are identified and so far 1879 Nos. are completed. (i.e., 269 Nos. in Anantapur circle, 266Nos. in Kurnool circle, 393 Nos. in Mahabub nagar circle, 329 Nos. in Medak circle, 509 Nos. in Nalgonda circle and113 Nos. in Ranaga Reddy South circle. Balance feeders will be completed before 31st March 2005. By this the number of breakdowns and interruptions have substantially reduced as the length of LT line is reduced. In twin cities to reduce the interruptions on account of over head lines, UG cable system is proposed and in this financial year the routes are identified and works are in progress. The works will be completed before 31st March 04. The other areas will be taken up in a phased manner. The UG cable system will drastically cut down the breakdowns. The information relating to breakdowns and interruptions is monitored at Corporate office on daily basis by getting the information from all the control rooms of circles to the main control room in APCPDCL. Table 11 a. 1. Number and Duration of Supply Interruptions on 33 KV and 11 KV Networks. ( Consolidated) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year |  | Month | No. of Interruptions |  |  |  | Duration in hours |  |  |  |  | No. of Breakdowns |  |  |  | Duration in hours |  |  |  |
|  |  |  | 33 KV |  | 11 KV |  | 33 KV Lines |  | 11 KV lines |  |  | 33 KV |  | 11 KV |  | 33 KV Lines |  | 11 KV lines |  |
| 2003 |  | Apr-03 | 3912 |  | 17528 |  | 3688 |  | 40908 |  |  | 87 |  | 739 |  | 478 |  | 1898 |  |
| 2003 |  | May-03 | 1751 |  | 18178 |  | 1793 |  | 23839 |  |  | 47 |  | 510 |  | 197 |  | 1173 |  |
| 2003 |  | Jun-03 | 922 |  | 9433 |  | 985 |  | 18481 |  |  | 68 |  | 703 |  | 223 |  | 1739 |  |
| 2003 |  | Jul-03 | 911 |  | 8694 |  | 854 |  | 11757 |  |  | 45 |  | 631 |  | 171 |  | 1828 |  |
| 2003 |  | Aug-03 | 1088 |  | 14583 |  | 1129 |  | 37018 |  |  | 36 |  | 802 |  | 179 |  | 2550 |  |
| 2003 |  | Sep-03 | 1532 |  | 16206 |  | 1322 |  | 38064 |  |  | 43 |  | 453 |  | 188 |  | 1165 |  |
| 2003 |  | Oct-03 | 1369 |  | 16646 |  | 1521 |  | 38428 |  |  | 44 |  | 961 |  | 213 |  | 2977 |  |
| 2003 |  | Nov-03 | 1206 |  | 12577 |  | 841 |  | 28055 |  |  | 12 |  | 326 |  | 32 |  | 902 |  |
| 2003 |  | Dec-03 | 1027 |  | 11382 |  | 703 |  | 28218 |  |  | 22 |  | 311 |  | 62 |  | 791 |  |
| 2004 |  | Jan-04 | 908 |  | 11102 |  | 455 |  | 144000 |  |  | 44 |  | 583 |  | 173 |  | 1513 |  |
| 2004 |  | Feb-04 | 1523 |  | 13685 |  | 918 |  | 25520 |  |  | 21 |  | 396 |  | 47 |  | 974 |  |
| 2004 |  | Mar-04 | 2469 |  | 13165 |  | 2058 |  | 22375 |  |  | 43 |  | 467 |  | 147 |  | 1224 |  |
| 2004 |  | Apr-04 | 1906 |  | 14322 |  | 2595 |  | 27782 |  |  | 94 |  | 903 |  | 501 |  | 3499 |  |
| 2004 |  | May-04 | 1758 |  | 16229 |  | 1068 |  | 23972 |  |  | 90 |  | 971 |  | 299 |  | 3163 |  |
| 2004 |  | Jun-04 | 1292 |  | 16936 |  | 1413 |  | 25162 |  |  | 68 |  | 995 |  | 463 |  | 3204 |  |
| 2004 |  | Jul-04 | 2032 |  | 13280 |  | 1285 |  | 19880 |  |  | 70 |  | 842 |  | 217 |  | 3107 |  |
| 2004 |  | Aug-04 | 1713 |  | 11924 |  | 1184 |  | 12555 |  |  | 60 |  | 537 |  | 121 |  | 1899 |  |
| 2004 |  | Sep-04 | 5713 |  | 14047 |  | 189 |  | 2688 |  |  | 24 |  | 283 |  | 240 |  | 1809 |  |
| Table 11.a.2 - 33 kV & 11 kV Interruptions during 2003-2004 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| S.No. | Name of the Circle |  |  | 33 kV SYSTEM |  |  |  |  |  |  | 11 kV SYSTEM |  |  |  |  |  |  |  |  |
|  |  |  |  | No. of feeders |  | Length of lines Km |  | No. of interruptions |  | Duration Hrs | No. of feeders |  | Length of lines Km |  | No. of interruptions |  | Duration Hrs |  |  |
| 1 | Anantapur |  |  | 62 |  | 2042 |  | 3904 |  | 1059 | 442 |  | 9966 |  | 3315 |  | 53579 |  |  |
| 2 | Kurnool |  |  | 56 |  | 1865 |  | 814 |  | 52 | 361 |  | 8532 |  | 6345 |  | 372 |  |  |
| 3 | Mahabubnagar |  |  | 62 |  | 1990 |  | 5571 |  | 1276 | 479 |  | 17256 |  | 2680 |  | 3067 |  |  |
| 4 | Nalgonda |  |  | 71 |  | 2105 |  | 40 |  | 209 | 592 |  | 12878 |  | 159 |  | 188 |  |  |
| 5 | Medak |  |  | 84 |  | 1481 |  | 8482 |  | 11994 | 551 |  | 8300 |  | 9548 |  | 25512 |  |  |
| 6 | RR North |  |  | 33 |  | 343 |  | 404 |  | 7 | 158 |  | 1588 |  | 2515 |  | 283 |  |  |
| 7 | RR South |  |  | 39 |  | 989 |  | 176 |  | 15 | 230 |  | 4042 |  | 7361 |  | 2495 |  |  |
| 8 | Hyderabad |  |  | 76 |  | 469 |  | 1039 |  | 364 | 429 |  | 2030 |  | 7851 |  | 10068 |  |  |
|  | TOTAL |  |  | 483 |  | 11285 |  | 20430 |  | 14976 | 3242 |  | 64592 |  | 39774 |  | 47342 |  |  |

| Table 11.a.3 - 33 kV & 11 kV breakdowns during 2003-2004 (From 1-4-04 to 30-9-04) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S.No. | Name of the Circle | 33 kV SYSTEM |  |  |  | 11 kV SYSTEM |  |  |  |
|  |  | No. of feeders | Length of lines Km | No. of breakdowns | Duration Hrs | No. of feeders | Length of lines Km | No. of breakdown | Duration Hrs |
| 1 | Anantapur | 63 | 2071 | 73 | 410 | 474 | 10165 | 197 | 1272 |
| 2 | Kurnool | 56 | 1910 | 89 | 451 | 408 | 8891 | 98 | 377 |
| 3 | Mahabubnagar | 67 | 2077 | 223 | 1236 | 528 | 17256 | 3074 | 4353 |
| 4 | Nalgonda | 76 | 2150 | 36 | 138 | 678 | 14124 | 76 | 190 |
| 5 | Medak | 84 | 1481 | 57 | 89 | 551 | 8400 | 2377 | 4467 |
| 6 | RR North | 33 | 327 | 15 | 1 | 179 | 1653 | 212 | 49 |
| 7 | RR South | 39 | 1056 | 13 | 43 | 248 | 4408 | 181 | 261 |
| 8 | Hyderabad | 88 | 481 | 66 | 58 | 447 | 1852 | 3054 | 4540 |
|  | TOTAL | 506 | 11554 | 572 | 2426 | 3513 | 66749 | 9269 | 15509 |

## PERIODS WHEN VOLTAGE/FREQUENCY WAS BEYOND PRESCRIBED LIMITS

Requirement in the Guidelines:

Periods when voltage and/or frequency  was beyond the prescribed  limits on primary distribution ( 33KV, 11KV) networks. Details of steps proposed to improve performance and monitoring.  ( as given in Guidelines – 11b)

APCPDCL’s Response :

### Voltage

The 400KV Substations commissioned at Mamidipally and Kurnool have substantially contributed in improving power supply system as a whole.

In order to further stabilize the supply conditions in the primary distribution system during 2004-05, 68 Nos 33/11KV substations proposed.  During this financial year 10 Nos 33/11KV substations are commissioned in the constituency of Dharmavaram, Nallamada and Anantapur of Anantapur circle, Nandyal, Aloor and  Eemmiganur of Kurnool circle, Ramayampet and  Ramayampet of Medak circle Chevella of RangaReddy South circle.  P&T Colony, Uppal of RanagaReddy North circle.

Frequency:

Normally the grid frequency is being maintained always above 49HZ only. In case of any disturbances load shedding is resorted to regulate system frequency and at times.  During system emergencies 132KV feeders are tripped to bring necessary load relief. The load despatch center, at Vidyut Soudha controls the frequency emergencies giving instructions to 220KV and 132KV substation operators for giving load relief on different feeders in all the circles. The load despatch center will be in constant touch with generating stations to take necessary measures to maintain frequency.

## PENDING APPLICATIONS:

Requirement in the Guidelines:

Number of supply or connection applications pending for more than six months, categorized by the reasons why (Guidelines- 11 C).

CPDCL’s Response:

The CPDCL humbly submits that various initiatives are in place for proper monitoring of applications for new connections duly reducing the redundancies to suit the standards set by the Hon’ble Commission.

The release of services under agricultural category is based on the guidelines and targets set by the Government of Andhra Pradesh.

The pendency of applications due to various reasons is enclosed below:

| Table 11.c.1 - Number of connection applications pending more than six months as on 31-03-2004 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ATP | KNL | MBN | NLG | MDK | RRN | RRS | HDN | HDS | HDC | CPDCL |
| LT |  |  |  |  |  |  |  |  |  |  |  |
| CAT1 | 401 | 5 | 39 | 264 | 96 | 226 | 77 | 49 | 0 | 0 | 1157 |
| CAT II | 57 | 9 | 5 | 25 | 20 | 4 | 9 | 34 | 0 | 0 | 163 |
| CAT III | 5 | 15 | 2 | 54 | 6 | 10 | 10 | 0 | 0 | 0 | 102 |
| CAT IV | 21 | 53 | 0 | 24 | 0 | 0 | 4 | 0 | 0 | 0 | 102 |
| CAT V | 6342 | 4176 | 45640 | 10223 | 6820 | 81 | 5346 | 0 | 0 | 0 | 78628 |
| CAT VI | 16 | 47 | 0 | 115 | 0 | 0 | 72 | 0 | 0 | 0 | 250 |
| CAT VII | 1 | 0 | 0 | 13 | 0 | 0 | 6 | 0 | 0 | 0 | 20 |
| CAT VIII | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| HT |  |  |  |  |  |  |  |  |  |  |  |
| CAT I | 2 | 0 | 4 | 10 | 2 | 27 | 11 | 2 | 1 | 0 | 59 |
| CAT II | 0 | 4 | 0 | 4 | 0 | 0 | 4 | 3 | 8 | 3 | 26 |
| CAT IV | 0 | 1 | 8 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 11 |
| CAT V | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| CAT VI | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| OTHERS | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Total | 6626 | 6628 | 38436 | 30571 | 8048 | 5562 | 1840 | 1655 | 3146 | 9 | 80518 |

| Table 11.c.2 - Number of connection applications pending more than six months as on 30-09-2004 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ATP | KNL | MBN | NLG | MDK | RRN | RRS | HDN | HDS | HDC | CPDCL |
| LT |  |  |  |  |  |  |  |  |  |  |  |
| CAT1 | 183 | 8 | 0 | 435 | 26 | 247 | 5 | 15 | 0 | 0 | 919 |
| CAT II | 71 | 9 | 0 | 76 | 30 | 2 | 0 | 9 | 0 | 0 | 197 |
| CAT III | 13 | 16 | 2 | 59 | 0 | 7 | 0 | 0 | 0 | 0 | 97 |
| CAT IV | 19 | 84 | 0 | 1 | 2 | 0 | 0 | 0 | 0 | 0 | 106 |
| CAT V | 3614 | 2326 | 46504 | 10465 | 7058 | 102 | 5479 | 0 | 0 | 0 | 75548 |
| CAT VI | 37 | 171 | 0 | 84 | 1 | 0 | 0 | 0 | 0 | 0 | 293 |
| CAT VII | 25 | 0 | 0 | 25 | 0 | 0 | 0 | 0 | 0 | 0 | 50 |
| CAT VIII | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| HT |  |  |  |  |  |  |  |  |  |  |  |
| CAT I | 5 | 1 | 4 | 13 | 1 | 28 | 9 | 6 | 9 | 0 | 76 |
| CAT II | 0 | 3 | 0 | 5 | 0 | 0 | 4 | 6 | 2 | 11 | 31 |
| CAT IV | 0 | 2 | 6 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 9 |
| CAT V | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| CAT VI | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 2 |
| OTHERS | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| Total | 6291 | 4902 | 38055 | 29548 | 8131 | 4278 | 1788 | 2017 | 2151 | 7 | 77329 |

Table 11.c.2 - Number of connection applications pending more than six months as on 31-03-2004

| Category |  | Formalities not completed by consumer |  | Being processed in office |  | For Want of material | Erection work in progress | Any other reason * |  | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LT Supply |  |  |  |  |  |  |  |  |  |  |
| Domestic |  | 388 |  | 6 |  | 293 | 462 | 8 |  | 1157 |
| Non-domestic & Commercial |  | 57 |  | 0 |  | 10 | 91 | 5 |  | 163 |
| Industrial |  | 44 |  | 2 |  | 3 | 53 | 0 |  | 102 |
| Cottage industries |  | 61 |  | 0 |  | 0 | 37 | 4 |  | 102 |
| Agriculture |  | 1157 |  | 822 |  | 1189 | 1333 | 74127 |  | 78628 |
| Street lights & Water supply in local bodies |  | 126 |  | 10 |  | 16 | 79 | 19 |  | 250 |
| General purpose |  | 11 |  | 0 |  | 0 | 6 | 3 |  | 20 |
| Temporary supply |  | 0 |  | 0 |  | 0 | 0 | 0 |  | 0 |
| HT Supply |  |  |  |  |  |  |  |  |  |  |
| Category 1 |  | 53 |  | 0 |  | 0 | 6 | 0 |  | 59 |
| Category 2 |  | 19 |  | 1 |  | 0 | 4 | 2 |  | 26 |
| Category 4 |  | 9 |  | 0 |  | 0 | 1 | 1 |  | 11 |
| Category 5 |  | 0 |  | 0 |  | 0 | 0 | 0 |  | 0 |
| Category 6 |  | 0 |  | 0 |  | 0 | 0 | 0 |  | 0 |
| Others |  | 0 |  | 0 |  | 0 | 0 | 0 |  | 0 |
| Total |  | 1925 |  | 841 |  | 1511 | 2072 | 74169 |  | 80518 |
| Table 11.c.4 - Number of connection applications pending for more than six months as on 30-09-2004 |  |  |  |  |  |  |  |  |  |  |
| Category | Formalities not completed by consumer |  | Being processed in office |  | For Want of material |  | Erection work in progress | Any other reason * | Total |  |
| Domestic | 286 |  | 0 |  | 385 |  | 213 | 35 | 919 |  |
| Non-domestic & Commercial | 49 |  | 5 |  | 10 |  | 123 | 10 | 197 |  |
| Industrial | 47 |  | 0 |  | 8 |  | 42 | 0 | 97 |  |
| Cottage industries | 90 |  | 0 |  | 0 |  | 16 | 0 | 106 |  |
| Agriculture | 69 |  | 0 |  | 276 |  | 0 | 75203 | 75548 |  |
| Street lights & Water supply in local bodies | 176 |  | 5 |  | 5 |  | 92 | 15 | 293 |  |
| General purpose | 0 |  | 0 |  | 0 |  | 40 | 10 | 50 |  |
| Temporary supply | 0 |  | 0 |  | 0 |  | 0 | 0 | 0 |  |
| Category 1 | 68 |  | 0 |  | 3 |  | 5 | 0 | 76 |  |
| Category 2 | 23 |  | 1 |  | 1 |  | 5 | 1 | 31 |  |
| Category 4 | 7 |  | 0 |  | 0 |  | 2 | 0 | 9 |  |
| Category 5 | 0 |  | 0 |  | 0 |  | 0 | 0 | 0 |  |
| Category 6 | 2 |  | 0 |  | 0 |  | 0 | 0 | 2 |  |
| Others | 0 |  | 1 |  | 0 |  | 0 | 0 | 1 |  |
| Total | 875 |  | 12 |  | 688 |  | 538 | 75274 | 77329 |  |

## ACCIDENTS

Requirement in Guidelines :

Number of reported fatal and non-fatal accidents for the year 2002 –2003 differentiating between accidents involving humans and those involving animals. An analysis of steps to reduce such accidents must also be provided (Guidelines – 11d)

- Requirement in Guidelines :

Number of reported fatal and non-fatal accidents for the year 2003 –2004 differentiating between accidents involving humans and those involving animals. As anlysis of steps to reduce such accidents must also be provided (Guidelines – 11d)

- APCPDCL’ s Response

Data on the number of fatal and non-fatal accidents differentiated between accidents involving human and those involving animals are presented in Table 11d1. This data includes accidents in 33KV, 11KV and LT distribution network only. The total number of fatal and non fatal accidents recorded in APCPDCL in 2003 – 2004 is shown in the table below. The accident tally includes departmental staff, contract labour and members of genera public. The accidents involving general public shown in table 11d1.

Table 11.d1 – Fatal and Non-Fatal accidents :

| Category | Total No. of Accidents in 2003 – 04 |  |  | Total |
| --- | --- | --- | --- | --- |
|  | Departmental | Contract Labour | Public |  |
| Fatal |  |  |  |  |
| Human Beings | 6 | 8 | 244 | 258 |
| Animals | 0 | 0 | 78 | 78 |
| # Total | 6 | 8 | 322 | 336 |
| Non-Fatal |  |  |  |  |
| Human Beings | 28 | 16 | 51 | 95 |
| ## Animals | 0 | 0 | 1 | 1 |
| Total | 28 | 16 | 52 | 96 |

The details of accidents categorized under departmental and non-departmental functions. Table 11d2 summarize the number of fatal and non fatal accidents recorded various circles for the year 2003 – 2004.

# REASONS FOR NON-DEPARATMENTAL ACCIDENTS :

- Non availability of Guy Insulators to the stays.

- Snapping conductors of AAC/ACSR/Copper conductor of LT lines and 11KV lines on account of gales, cyclones and aging.

- Removal of stays by the public resulting in leaning of the dead-end poles and consequent sagging of the live lines / wires with reduced ground clearances.

- Dashing of lorries and other loaded vehicles against electric poles/stays resulting in snapping of the conductors or drooping of the live conductor etc.

- Falling of advertising boards on the live lines.

- Members of the public coming into contact or within arcing distance of live lines while carrying out work such as digging of bore wells. Assembling TV antennas or cutting the leaves of palm trees/coconut trees in the agricultural fields etc.,

- In the agriculture belt consumers are extending the supply by highly non standard service wires on wooden sticks and in the most substandard way. Public in most cases farmers only are getting electrocuted by touching the same wires whenever they are falling on the ground.

- Tampering with the electrical wiring by consumers trying to rectify faults in their own installations.

- Constructions of houses or multistoried buildings without maintaining proper clearances and set-back from existing live electrical lines or location of distribution transformer close to a building without maintaining mandatory electrical clearances.

Animals and cattle rearers are the worst hit among others in the non-departmental accidents, as they do not notice the loose or fallen electrical lines subsequent to a gale or storm. There is a need for these persons to be alerted and properly educated in the local villages.

All the line patrolling staff carry out immediate rectification of the snapped lines or conductors and damaged or learning poles particularly in village areas. A proper record or log book is maintained by them during their patrolling duties which is reviewed and any defects noticed by them Viz. Bottom rusted poles, conductor with out proper supports rectified to avoid accidents after such calamities like gales etc.

PLANS FOR IMPLEMENTING PREVENTIVE MEASURES :

- Standard construction practice to be ensured whenever new distribution lines are laid and energised.

- Raising of plinths and DTR boxes to prevent touching of live lines by human beings as well as animals.

- Providing fencing to major DTR structures to prevent for touching by human beings and animals.

- An alerting mechanism to be arranged in liasion with villages well in advance of charging any new distribution line. Distribution pamphlets and making loud speaker announcements making Tom Tom in villages may also be restored to as an added precautionary measure.

- Clearances for O/H lines as specified in IE rules 77,78,79 and 80 to be strictly maintained and verified at the time of statutory inspections of lines & substations before charging.

- Even RCO / PSCC poles used in the rural areas must be earthed with pig tailed    GI wire.

- Guy insulators for stay wire be provided to prevent inadequate energisation.

- Anti-climbing devices may be provided to prevent people from scaling towers or poles.

- Foot cross arms may be provided at least for angle points and cut points for isolation.

- All LT tapping poles are to be provided with sectionalise fuses along with arial boards for services.

- Double feed LT lines may be cautioned with boards, as well as by creating a dead span.

- Neutral for each DTR along with street light phase may be insisted for each DTR, so that street light phase is limited to distribution transformer wise.

- Safety camps conducted weekly for O&M staff and contract labourers in APCPDCL.

- Electrical safety week was observed in CPDCL from 01.05.2003 to 07.05.2003. During this period medical checkups were arranged to O&M staff and contract labour and their families in all divisions of CPDDCL and message was given to use safety appliances while working on electric lines to avoid electrical accidents.

- Safety equipment and material (T&P) are procured and provided to all Circles in APCPDCL.

- Rubber gloves and gant belts.

- Safety belts.

- Helmet

- Discharge earth rods.

- Insulated tools like cutting pliers, screw drivers, bamboo ladders etc.

- Special ladder with fiber glass rails are procured and provided in twin cities to attend breakdowns.

Table 11.d 2 statement showing Fatal / Non-Fatal accidents to general public / animals during the period from 01.04.2003 -31.03.2004.

| Sl.No | Name of the Circle | Human |  | Animals |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Fatal | Non-Fatal | Fatal | Non-Fatal |
| 1 | Anantapur | 36 | 15 | 4 | 0 |
| 2 | Kurnool | 48 | 16 | 13 | 0 |
| 3 | Mahabubnagar | 38 | 11 | 4 | 0 |
| 4 | Nalgonda | 47 | 13 | 22 | 0 |
| 5 | Medak | 26 | 5 | 16 | 0 |
| 6 | Ranga Reddy - South | 28 | 9 | 16 | 0 |
| 7 | Ranga Reddy - North | 15 | 7 | 2 | 0 |
| 8 | Hyd – North | 8 | 12 | 0 | 0 |
| 9 | Hyd - Central | 10 | 5 | 1 | 0 |
| 10 | Hyd – South | 2 | 2 | 0 | 1 |
|  | A.P.C.P.D.C.L | 258 | 95 | 78 | 1 |

REASONS FOR DEPARTMENTAL ACCIDENTS :

- Non observance of safety rules.

- Non utilisation of safety appliances by the workmen though they are supplied and trained to use them.

- Non-operation of one of the blades of Air Break (AB) switches due to bad maintenance, leading to arcing and burn injuries.

- Slipping from poles and O/H lines while on work due to non-utilisation of safety belt.

- Attending of fuse off calls complaints on the poles without using proper safety gloves etc.

- Due to improper co-ordination and communication of line clearance or improper earthing of the line or equipment at the work spot.

- Due to negligence or by over confidence to visually ensure that the AB switch is kept open while undertaking renewal or replacement of a Blown Horn Gap (HG) fuse.

- While working carelessly on the bottom portion of rusted rail poles. R.S. Joists or damaged PSCC/RCC poles in the ground, which cannot be easily detected.

- Working at double feed points without working knowledge of the feeding arrangements and without ensuring that there is no back – feeding or charged condition of line.

- Allowing un-authorized persons to work on lines.

REMEDIAL ACTION TO REDUCE ACCIDENTS:

- Almost all fatal or non fatal accident cases have been due to electrocution of humans or animals by coming into contact with live conductors. In some cases the live conductors were found to have improper ground clearances. Some of the accidents were also cause due to conductors snapping but continuing to be live. There have also been cases where the stay wire or poles have inadvertently become energise due to improper earthing absence of guy insulator etc. In the accidents occurred to the farmers, it is generally observed that the cause of accidents are due to using of un-standard service wires or not taking proper care while switching off and switching on the starters of the motors. In some cases it is observed that lifting of the pipes without noticing the overhead lines. In Urban areas the plumbers and centring workers while working at the construction of buildings are becoming victims of electrocution due to touching the lines passing over the buildings or by the side of the buildings while carrying the rods and pipes etc. In some cases, it is also observed that the lorry drivers are parking their vehicles underneath the lines and climbing the lorry cabin and by over sight coming in touch with live lines and becoming the victims of electric accidents.

- It is necessary to point out that all the above situations are only due to improper construction and operation and maintenance of lines and distribution transformers. In all such cases the responsibility for such electrocution should be fixed on the concerned staff who have contributed to the electrocution by virtue of their lapse / negligence.

- For any lapse, suitable prompt action is taken against the concerned public or staff to discourage their negligence or casual approach.

- Deterrent action taken against all the erring field staff who fail to take safety precautions while working on lines and also ensure that adequate quantity of safety appliances are made available to all the concerned staff members. Safety training courses are regularly conducted.

- Classes are being conducted for the workmen on payday or any suitable working day to enlighten them on safety measures and to avoid electrical accidents by proper utilisation of safety devices and precautions as per the safety rules. The category of employees generally involved in the departmental accidents are of the following category.

- Helpers.

- A.L.M’s

- Linemen.

- The category of victims, besides animals involved in the non-departmental category of accident victims generally are :

- Farm workers.

- Cattle workers.

- Linemen / Mazdoors (P&T)

- Village Electricity Workers / Electricians of Panchayat Municipality.

- Casual labourers of contractors.

### INADEQUATE AND DEFECTIVE METERS:

1.	Requirement in Guidelines:

Number of inadequate and defective meters, programme and phasing of investment for replacement. (Guidelines-11 e).

CPDCL’s Response:

Under the directions of the Hon’ble Commission, CPDCL is implementing the metering plan of replacing the existing old meters with high quality meters in all towns and mandal head quarters.

Adequate meters are being maintained in stock in the circles for immediate replacement of defective meters reported.

The status of the defective meters during the year 2003-04 is indicated below:

Table 11.e.1 - Stuck up Meters -  2003-04

| Circle | Stuck up on 1.4.2003 |  |  | Replaced during 2003-04 |  |  | Stuck up during 2003-04 |  |  | Stuck up as on 1.4.2004 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 Ph | 3 Ph | Total | 1 Ph | 3 Ph | Total | 1 Ph | 3 Ph | Total | 1 Ph | 3 Ph | Total |
| Anantapur | 2937 | 38 | 2975 | 31421 | 684 | 32105 | 29599 | 1076 | 30675 | 1115 | 430 | 1545 |
| Kurnool | 176 | 15 | 191 | 81176 | 1546 | 82722 | 81404 | 1537 | 82941 | 404 | 6 | 410 |
| Mahaboobnagar | 10355 | 420 | 10775 | 24119 | 627 | 24746 | 22216 | 570 | 22786 | 8452 | 363 | 8815 |
| Nalgonda | 46992 | 133 | 47125 | 37328 | 73 | 37401 | 18240 | 255 | 18495 | 27904 | 315 | 28219 |
| Medak | 5225 | 485 | 5710 | 13483 | 464 | 13947 | 16804 | 277 | 17081 | 8546 | 298 | 8844 |
| Ranga Reddy-N | 152 | 5 | 157 | 8628 | 1163 | 9791 | 9014 | 1166 | 10180 | 538 | 8 | 546 |
| Ranga Reddy - S | 9274 | 7 | 9281 | 19097 | 165 | 19262 | 15083 | 166 | 15249 | 5260 | 8 | 5268 |
| Hyd-North | 109 | 40 | 149 | 8069 | 3358 | 11427 | 8041 | 3358 | 11399 | 81 | 40 | 121 |
| Hyd-Central | 389 | 161 | 550 | 8372 | 1886 | 10258 | 8393 | 1827 | 10220 | 410 | 102 | 512 |
| Hyd-South | 1104 | 2 | 1106 | 4461 | 154 | 4615 | 3539 | 169 | 3708 | 182 | 17 | 199 |
| TOTAL | 76713 | 1306 | 78019 | 236154 | 10120 | 246274 | 212333 | 10401 | 222734 | 52892 | 1587 | 54479 |

Table 11.e.2 - Burnt Meters -  2003-04

| Circle | Burnt Meters as on 1.4.2003 |  |  | Replaced during 2003-04 |  |  | Burnt Meters during 2003-04 |  |  | Burnt Meters as on 1.4.2004 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 Ph | 3 Ph | Total | 1 Ph | 3 Ph | Total | 1 Ph | 3 Ph | Total | 1 Ph | 3 Ph | Total |
| Anantapur | 503 | 21 | 524 | 1715 | 182 | 1897 | 1575 | 190 | 1765 | 363 | 29 | 392 |
| Kurnool | 29 | 5 | 34 | 7294 | 264 | 7558 | 7454 | 262 | 7716 | 189 | 3 | 192 |
| Mahaboobnagar | 517 | 62 | 579 | 3111 | 130 | 3241 | 3879 | 142 | 4021 | 1285 | 74 | 1359 |
| Nalgonda | 601 | 0 | 601 | 1025 | 3 | 1028 | 1470 | 13 | 1483 | 1046 | 10 | 1056 |
| Medak | 3051 | 148 | 3199 | 3318 | 241 | 3559 | 1647 | 148 | 1795 | 1380 | 55 | 1435 |
| Ranga Reddy - N | 7 | 3 | 10 | 1657 | 246 | 1903 | 1655 | 243 | 1898 | 5 | 0 | 5 |
| Ranga Reddy - S | 1512 | 4 | 1516 | 3462 | 86 | 3548 | 2825 | 87 | 2912 | 875 | 5 | 880 |
| Hyd-North | 27 | 2 | 29 | 2374 | 724 | 3098 | 2384 | 743 | 3127 | 37 | 21 | 58 |
| Hyd- Central | 136 | 17 | 153 | 1501 | 377 | 1878 | 1424 | 368 | 1792 | 59 | 8 | 67 |
| Hyd- South | 213 | 4 | 217 | 1299 | 59 | 1358 | 1104 | 57 | 1161 | 18 | 2 | 20 |
| TOTAL | 6596 | 266 | 6862 | 26756 | 2312 | 29068 | 25417 | 2253 | 27670 | 5257 | 207 | 5464 |

## UNAUTHORISED CONNECTIONS:

Requirement in Guidelines:

Number of unauthorized connections (as a result of sample survey) to be reported by tariff category.  Measures adopted to curb unauthorized consumption.  Results and programme for the future.  (Guidelines – 11 f).

APCPDCL’s Response

In CPDCL 37,629 nos unauthorized agricultural connections registered in June 2000, have been regularized.

58,620 nos unauthorized agricultural connections have been disconnected so far.

APCPDCL is taking all steps to disconnect the unauthorized connections in line with the GoAP policy, before the rabi season commences.

Table 11 f1 : Number of services inspected and booked during each of the months from April 2003 to Sept 2004.

| Month | Inspected | Booked |
| --- | --- | --- |
| Apr | 46099 | 3835 |
| May | 28062 | 3224 |
| Jun | 21118 | 3368 |
| Jul | 30440 | 4266 |
| Aug | 38028 | 5347 |
| Sep | 32192 | 5347 |
| Oct | 32963 | 3403 |
| Nov | 45494 | 4729 |
| Dec | 34037 | 2593 |
| Jan | 27877 | 1490 |
| Feb | 31614 | 1938 |
| Mar | 33168 | 1921 |
| Apr | 20442 | 995 |
| May | 33597 | 1797 |
| Jun | 34191 | 1775 |
| Jul | 42442 | 2122 |
| Aug | 41639 | 3079 |
| Sep | 40069 | 3146 |

## REVENUE ARREARS:

1.	Requirement in Guidelines:

Amount of revenue arrears at beginning and end of year analysed age wise on a six monthly basis.  Reasons for non-collection and details of litigation involving collection of revenue of Rs.50,000 or more should also be provided (Guide Lines 11 – g).

APCPDCL’s Response:

The following tables illustrate the arrears position of APCPDCL as on 31-3-2004.

| Schedule 11 g1 |  |
| --- | --- |
| Break up of arrears on various accounts as on March 31, 2004 |  |
|  | Rs. in Lakhs |
| Live Services | 26582.51 |
| State Govt. Departments | 403.33 |
| State Government Undertakings | 725.44 |
| Central Govt. Departments/Undertakings | 15452.54 |
| Local Bodies | 142.59 |
| Court Cases | 8387.56 |
| Disconnected services | 36894.32 |
| Debts decreed to be withdrawn | 49.20 |
| Others(RR Act, RESCO & Others) | 5396.52 |
| Net Arrears | 94034.01 |

| Schedule 11g2 |  |
| --- | --- |
| Category wise arrears as on 31st Mar'2004 |  |
| Category |  |
| LT | (Rs. In Lakhs) |
| L.T. Cat.-I Domestic | 42314.10 |
| L.T. Cat.-II Non-Domestic | 6163.63 |
| L..T.Cat-III Normal Industrial | 4583.46 |
| L.T. Cat-IV Cottage Ind. | 38.09 |
| L.T. Cat-V Agricultural | 0.00 |
| L.T. Cat-VI Street Lights | 2091.80 |
| L.T. Cat-VII General Purpose | 607.85 |
| L.T. Cat-VIII Temporary Supply | 117.50 |
| LT-Tot | 55916.43 |
| Category | (Rs. In Lakhs) |
| HT |  |
| H.T. Cat.I Industrial | 30818.21 |
| H.T. Cat.II Non-Industrial | 1216.96 |
| H.T. Cat.IV Agricultural (IV A) | 260.80 |
| H.T. Cat.V Railway Traction | 456.48 |
| H.T. Cat.VI Residential | -355.84 |
| R.E.S.C.O. | 5723.06 |
| H.T.Temp.Supply | -2.09 |
| HT-Tot | 38117.58 |
| LT+HT Tot | 94034.01 |

Note: Unbilled revenue and provision for bad debts not taken into account

Schedule  11 g3

| Ageing Analysis of receivables |  |
| --- | --- |
| Total For the LT | Rs.lacs. |
| Duration | As on 31-3-2004 |
| 3-6 months old | 13924.74 |
| 6-12 months old | 9312.12 |
| More than 12 months old | 32679.57 |
| Total | 55916.43 |
| Total For the HT | Rs.lacs. |
| Duration | As on 31-3-2004 |
| 3-6 months old | 14866.49 |
| 6-12 months old | 10032.48 |
| More than 12 months old | 13218.61 |
| Total | 38117.58 |
| Total For the LT+HT | Rs.lacs. |
| Duration | As on 31-3-2004 |
| 3-6 months old | 28791.23 |
| 6-12 months old | 19344.60 |
| More than 12 months old | 45898.18 |
| Total | 94034.01 |

Detailed list of consumers having Rs. 50,000 or more arrears as on September 30, 2004 have been submitted.

### INITIATIVES TAKEN TO REALISE THE ARREARS:

- To have a realistic analysis of collectable and un-collectable arrears outstanding as on 31-03-2004, audit of receivable has been assigned for age-wise analysis of LT and HT services and to assess recovery potential of c.c.charges arrears due.

- In the fortnightly videoconferences with the Superintending Engineers / Opns, the disconnection of services whose arrears are more than Rs 5000 are being reviewed. At the end of every month, the disconnections of HT services who have not paid their monthly CC charges is also reviewed.

- The CAT analysis having list of F-class services is being sent to the subdivisions every month for review and disconnections.

- Sanction of installment facility LT II, III and HT I ,II has been withdrawn from the field offices so that there will not be further addition to the arrears.

- Pursuing the Government heads of the departments for realization of C.C charges dues. The c.c.charges arrears in respect of Govt Department Departments, Municipalities and Panchayats are being received through centralized payments made by the Govt. of A.P.

- Organizing the special drives for collection of arrears and physical verification of services under disconnection (UDC) duly ensuring the defaulting consumers are not availing supply through link services or other means.

- 100% implementation of issue of ‘D’ list and operation in respect finalised cases of theft & malpractices and back billing cases. This is also being pursued while holding video conferences regularly for effective monitoring.

- Issue of one month notices to Under Disconnection Services (UDC) as per B.P.Ms.No.151 & terminate the agreement duly adjusting the Consumption Deposit against the C.C charges dues.  Organised disconnection of link services of the already disconnected services for realization of arrears outstanding.

- Implementation of RR Act to recover the long pending dues by deploying the Dy. Tahasildars from Revenue Department. A substantial amount has been recovered through the efforts.

10.	 Arrears under Court cases have been pursued and got disposed in favour of the company and realization process is under progress. Monthly meetings are being held with the concerned accounts and legal officers to monitor the court cases. In the cases where the amount is high, senior advocates are being engaged.

## METER READING AND COLLECTION

1.	Requirement in Guidelines:

Data should be provided for each consumer category indicating (i) the normal meter reading cycle and the number of meter reading not carried out in accordance with the cycle and (ii) the number of consumer bills not served within 15 days of the meter reading.  Please also provide details of plans to improve the efficiency and punctuality of meter reading, the dispatch of consumer bills and cash collection. (Guidelines 11 g-contd).

2.	APCPDCL’s Response

APCPDCL is arranging the Meter Reading of all the Metered Services as follows:

Table 11.g.4 – MBC Schedule

| I. Domestic II Non-Domestic VII General Purpose | Meter reading, Billing & Collection (MBC) is done bi-monthly for majority of the consumers and monthly billing is done in all major Towns and Hyderabad City through Hand held Computer billing machines. High value monthly billing is done all over the DISCOM |
| --- | --- |
| III Industrial IV Cottage Industries VI Street Lights & PWS | Meter reading, Billing & Collection (MBC) is done monthly. |
| V Agriculture | Billing is done as per the slab system commensurate with the connected load as per the Tariff Order. Pass Books are issued to the consumers duly indicating the amount payable in each month. Since, Govt. of A.P. is paying the c.c.charges for providing free power supply to this category, the DISCOM has decided to issue half yearly bills to the consumers for collection of customer charges. |
| All the HT Categories | The Billing is done monthly for all categories. Meter reading is done monthly for all Categories. Collection is done for all categories on a monthly basis. |

APCPDCL has achieved substantial progress in the matter of collection of the revenues over the past few years and the overall collection efficiency has improved.  In respect of the meter reading default  cases, the following data is furnished for information of the Hon’ble Commission.

Table 11.g.5 – Meter Reading Defaults

| Month | Total Services | RNF Cases | Percentage (%) |
| --- | --- | --- | --- |
| April-04 | 4047915 | 9475 | 0.23 |
| May-04 | 4121273 | 8853 | 0.21 |
| June-04 | 4102046 | 9244 | 0.23 |
| July-04 | 4154892 | 9670 | 0.23 |
| September-04 | 4090199 | 8601 | 0.21 |

In the matter of Bill Distribution, APCPDCL made pioneering efforts to eliminate the defaults and consequent complaints in this regard and commenced the Spot Billing System in the Twin Cities and Municipalities of Rangareddy District from November, 2001onwards on a regular basis.  APCPDCL has also started the Spot Billing System in the other district from January, 2002 onwards. All the   Mandals and Villages are covered by Spot Billing System and bills are being issued  on bi-monthly basis. Monthly Spot Billing System with the handheld Computer devices is covered in all the Municipalities and Towns of  the DISCOM.  The Spot Billing System has almost eliminated the complaints of non-receipt of the bills and reading not furnished cases.