# Plans and Performance

## Plans for undertaking load research

### Requirement in the Guidelines

Plans for undertaking load research to determine the load profile of consumers supplied under each tariff. (Guidelines – 10a).

### APNPDCL’s Response

In its endeavor to record the electrical energy consumption precisely and safeguard the revenues for the company, NPDCL has continued its efforts to augment the ongoing extensive metering plan. The plan envisages replacement of old, burnt, stuck up, defective and sub standard electro mechanical meters with high accuracy and high quality electromechanical and electronic meters. This would provide improved data for consumer load profiling.

The metering plan is being implemented under different heads to cover all the aspects of energy audit.

Metering plan 1 involves replacement of single phase and three phase meters for domestic consumers.

| Plan | Description |
| --- | --- |
| Metering Plan – 1 | Single Phase High Quality Meters: REC has sanctioned bulk schemeS during 2002-03 and 2003-04 under SI: Meters for meters replacement of existing old single-phase meters. Under these schemes a total of 3,60,260 meters were replaced with high quality meters upto 11/05 and the balance work is under progress.. |
| Metering Plan – 2 | Three Phase High Quality Meters: During 2002-03, REC has sanctioned SI meters under bulk schemes for replacement of old 3- electro-mechanical meters with high quality meters. This scheme covers replacement of 58,000 3- meters. As on 11/2005 about 33,331 nos. of old meters were replaced with new high quality meters and the balance work is under progress. |
| Metering Plan – 3 | Metering 11 KV feeders: During 2003-04 REC has sanctioned a scheme under SI meters which covers provision of HT Tri-vector electronic meters on 585 Nos. 11 KV feeders. All 11 KV feeders are provided with Electronic meters. |
| Metering Plan – 4 | LT Electronic Tri-Vector Meters – During 2003-04 REC has sanctioned a scheme under SI meters for replacement of 2000 Nos. electro-mechanical meters existing for industrial services with LT electronic Tri-vector meters. The procurement is being initiated. |
| Metering Plan-5 | APDRP(Accelerated Power Development and Reforms Programme) for Warangal District was sanctioned with funding from Government of India (50%) & REC (50%) during 2003-04. Under this project 86,000Nos. 1-phase, 10,300 Nos. 3- and 855 Nos. LT electronic Tri vector meters 274 HT electronic Tri vector meters for 11 KV feeders, 1200 Nos. LT electronic meters on LV side of DTRs were proposed. The progress upto 11/2005 is as follows. Replaced 86,000 Single phase meters, 10,300 3- meters, 855 LT electronic tri-vector meters, 274 HT electronic trivector meters and 648 LT electronic meters on LV side of DTRs upto 11/2005. The balance work is under progress and programmed to complete during 2005-06. |
| Metering Plan-6 | APDRP (Accelerated Power Development and Reforms Programme) for 18 towns of NPDCL is sanctioned with funding from Government of India (50%) & REC (50%) during 2002-03. and Under this project it is proposed to replace 2,77,950 single phase meters 5,970 3- meters, 11 Nos. 33kv feeders, 35 Nos. 11kv feeders, 2284 HT tri-vector meters, 187 LT tri-vector meters. The progress up to 11/05 is 2,77,950 single-phase meters, 5970 3-Ph meters 11 Nos. 33kv feeders, 35 Nos. 11kv feeders, 450 Nos. HT tri-vector meters, 187 LT electronic tri-vector meters were replaced. The balance work is under progress and programmed to complete during 2005-06. |

With the implementation of this programme it is expected that APNPDCL will be in a better position to commence compiling the load survey and load profile data of various areas and consumer categories in a phased manner.  However in the HT Industrial category where substantial information already exists from the existing metering and billing data, APNPDCL is profiling the customers, their consumption pattern, the industry characteristics, load factors, response to price signals, etc much more closely to develop a structured load research program and base load forecast and tariff changes based on this information.

## Plans for improving the system power factor

### Requirement in the Guidelines

Plans to improve the system power factor by providing an incentive for consumers to install correct capacitors. (Guidelines – 10b)

### APNPDCL’s Response

The existing tariff levies a penalty by way of a surcharge on HT supplies for low power factor, as indicated below:

The power factor for the month is the ratio of kilowatt-hours to the kilovolt-ampere-hours supplied to the consumer during the month.  The power factor is calculated up to two decimal places.  The power factor of the consumer’s installation should not be less than 0.90.  If the power factor falls below 0.90 during any month, the consumer has to pay a surcharge as per the existing Tariff Order

Should the power factor drop below 0.75 and remain so for a period of 2 consecutive months, it must be brought up to 0.90 within a period of 6 months by methods approved by APNPDCL.  In case of failure of the consumer to rectify the power factor within limits, and without prejudice to the right of APNPDCL to collect surcharge and without prejudice to such other rights as having accrued to APNPDCL or any other right of APNPDCL, the supply to the consumer may be disconnected.

All LT industrial consumers using induction motors/welding transformers shall install LT shunt capacitors of specified ratings as indicated in the Tariff order.

In other cases the following action is contemplated.

- If any consumer fails to install the capacitors at all, or fails to install the capacitors of required rating or if the capacitors already installed, are found to be damaged, defective or non-functional during  inspection, the consumer is served with a notice to get the same installed/replaced/rectified or corrected within 30 days of the inspection and intimate the concerned Section Officer (operation), who inspects the capacitors again to verify their satisfactory performance.

- In the event of failure on the part of the industrial consumer to comply with the above notice, APNPDCL shall  levy a surcharge of 25% per month on the bill amount from the date of release of power to the service/the date from which the capacitor was defective or ceased to function, till such a time the capacitor is installed/replaced/rectified or corrected provided that such retrospective levy of surcharge is limited to one year from the date of inspection.  The consumer aggrieved by the retrospective levy may appeal to the concerned Superintending Engineer (Assessment), whose decision is final.

- In case the rated capacity of the induction motor/welding transformer falls in between the steps of stipulated ratings, the consumer has to install capacitors suitable for the next higher step.

- Failure on the part of the consumer to comply with the above notice, is treated as a violation of terms and conditions of the supply and APNPDCL reserves the right to terminate the contract and collect the sum equivalent to the minimum charges for the balance initial period of agreement.

- APNPDCL has  provided capacitive compensation  to the tune of 548 MVAR on LV side of 33/11kv  substations and 2844MVAR on 11kv line.

## The Investment Plan for the ensuing year

A detailed investment plan for the ensuing year is provided in section 2 of this document. Section 2 also illustrates the licensee’s investment plan for the remaining years of the control period.

## Plans for determining the relationship between kwh consumed and connected loads

### Requirement in Guidelines

Plan for determining the relationship between kWh consumed and connected loads on which un-metered charges are based.  (Guidelines – 10c).

### APNPDCL’s Response

APNPDCL is in the process of making accurate assessment of the agricultural consumption in the DISCOM. The assessment of consumption for the un-metered LT agricultural consumers is being done based on the readings of the meters provided on the sample agricultural DTRs. These consumers are given three-phase supply for regulated hours per day as per the approval of the Hon’ble Commission. Hence, these  agricultural pump sets are assumed to use power during the three phase supply timings.

The information of the connected load (in HP) and the recorded consumption as per the LV side meter readings of agricultural DTRs is used to estimate the    KWH / HP of the agricultural load, from DTR level to Mandal and hence district level for assessment of agricultural consumption.

All efforts are being made by APNPDCL to improve the coverage ratio in respect of sampling LV side DTR meter readings for arriving the accurate estimate of agricultural estimate consumption to the extent possible.

## Program for converting un-metered connections to metered supply

### Requirement in the Guidelines

Programme for converting un-metered connections to metered supply, together with investment needed.  (Guidelines – 10d).

### APNPDCL’s Response

Un-metered supply exists primarily in the agricultural category.  All other services of APNPDCL are ordinarily metered.  There are no un-metered services existing among the other categories of consumers. All the new services are released after fixing meters. Steps are being taken to detect unauthorised services and disconnect them.

APNPDCL submits that pursuant to the GoAP policy of free power to agricultural category, the comprehensive metering plan of agricultural services will be taken up as per the policy decided by the GoAP, under the guidance of the   Hon’ble Commission.

## Method of Assessing Consumption When Meters Are Not Installed / Defective

### Requirement in the Guidelines

Method of assessing consumption when meter is not installed or is defective, together with basis for such assessment.  (Guidelines – 10e)

### APNPDCL’s Response

Except agricultural connections, all other supplies are metered and billed.  The provision under clause 22.3.2 of Terms and Conditions of erstwhile APSEB Board are being followed for billing of all categories of loads during the period when the meter remained defective or uninstalled till finalization of draft General Terms and conditions of Supply by th Hon’ble Commission. If the premises were found locked at the time of meter reading, the procedure under clause 22.3.3 of Terms and Conditions is followed and charges levied.

## Plans for rationlizing existing manpower

### Requirement in the Guidelines

Plans, both short-term and long-term, for rationalization of existing manpower. (Guide lines – 10f)

APNPDCL’s Response

As per section 24 of the Andhra Pradesh Electricity Reform Act., 1998 (APERA) Act. No. 30 of 1998 relating to personnel, it is envisaged that the schemes of transfer of personnel to the new entitles, (formed consequent to the restructuring i.e., APTransco/ APGenco) shall be consistent with the tripartite agreement entered into between the Government of Andhra Pradesh (Go A.P.), the erstwhile APSEB, and its employees.

This tripartite agreement entered into between the Go A.P., the erstwhile APSEB, and its employees specified that “In the event of reorganization of APSEB, into two or more corporate entitles, the government and the APSEB hereby guarantee that there will be no retrenchment of employees for any reason and their status/ services will not change”. Further, in section 24 of the APERA, relating to personnel, it is envisaged that the scheme of transfer of personnel to the new entitles shall be consistent with the tripartite agreement. Accordingly, the transfer of personnel from APTransco to the successor entitles has been implemented by seeking options from employees with effect from 01-09-2002.

In the context prescribed above, APNPDCL has not contemplated any reduction in Manpower. For improving organizational effectiveness, organization development has been taken up as one of the priority areas. Appreciating the need to improve the levels of productivity of the employees, no additional manpower/ manpower for replacement e.t.c., are taken except to the minimum. Against the total sanctioned strength of 10,586, a total No. of 8,455 employees are on the rolls and thus there has been a gap of 2,131 employees. These 8,455 employees are classified into 7212 no. of regular employees, 642 contract employees and 601 apprentices. The same number of employees is considered for projecting for the ensuing year. It is being endeavored to optimally utilize the existing manpower and step up productivity and service levels.

Change of perception being important to improve the performance levels of the employees, brining about business and customer orientation has been addressed across the company in general and practicing business manager concept at section level and above. For this purpose, initiatives like, continuous training, performance management, communication with employees at regular intervals, introduction of systems relating to employee involvement e.t.c. have been undertaken.

As regards training, around 10258 Mandays were spent during 2004-05 and in in-house training programmes. These programmes were organized in respect of all levels, particularly, in the operational area, right from JLM to ADE level. The programme contents include emerging business scenario, changing profile of the customer, new Electricity Atc., business concepts, managerial skills and technical inputs in the areas of Substation maintenance, network maintenance, DTR up keep, pilferage/ theft of energy, safety e.t.c. These contents were developed in order to provide general exposure whereby employee perception about business scenario and customer improves and at the same time technical skill up-gradation is also served. In addition, induction training programs were conducted in respect of employees recruited in Engineering Service, Accounts Service i.e., of duration of 10 days.

Training module is in the pipeline for training of O&M personnel engaged through contractors in Sub-Station maintenance, e.t.c., in order that they are equipped with necessary technical and behavioral skills. In order to promote performance- oriented culture and establish accountability through the process of ownership, an integrated performance management system has been in implementation. The system focuses on performance improvement, employee involvement, employee development and feedback. For performance clarity, key performance indicators have been identified at each level and employees’ performance is being reviewed against these indicators. For continuous improvement through motivation Non-monetary incentive system is in place, which recognizes group performance. A detailed study is in progress as to the incentive system to be made applicable to recognize individual performance. To encourage employees to think and act creatively and to bring out their talents a comprehensive suggestions scheme has been put in place. However, to inculcate the habit of thinking and offering suggestions for improvements in their areas of operation, regular interactive meetings are held with all employees at divisional level as an open forum. In these forums, employees are being taken through Company’s operational and financial performance, issues relating to performance impediments and improvements discussed, suggestions from them are taken. In addition, an initiative is underway to form quality circles/ teams at distribution or section level to provide for employee involvement and promote ownership of the processes/ activities in their areas of operation.

Further, measures are being taken for field level restructuring and work process rationalization for better utilization of engineering talent and improving managerial ability particularly at section and subdivision levels. A comprehensive exercise is proposed to be undertaken for redefining roles different levels and, also review of the work norms in the light of competitive scenario that is likely to emerge in the wake of the new Electricity Act. 2003, for improving the internal efficiency through utilization of the existing resources to the best possible extent.

Strategy focus – continuous improvement:

Having spent considerable effort in training and development in the HRM strategy is being oriented to become more focused a continuous improvement which is more demonstrable and measurable. For the purpose, specific focus elements have been identified i.e., team work, participation in improvement, use of quality tools, cost of poor quality and knowledge sharing. It is proposed to institutionalize a culture of excellence in the work areas and thereby greater business focus among all in general and section officers and above in particular and increased engagement levels in the employees.

Leadership development:

This has been recognized as one of the key elements of the HRM strategy particularly in the wake of Electricity Act 2003 for enhancing value to customers. A multilayered process in the offing for leadership development.

Structural changes:

To improve the competitiveness, some of the initiatives being rolled out and formation of organizational Excellence; functional structure and providing unified directions from Corporate Office with respect to support functions with professionalization of these areas and instituting quality across the Company integrating various quality methodologies such as ISO 9000, Five S and Six Sigma, e.t.c,

Employee interactive meetings:

Extensive emphasis is being laid on employee interaction and wide spread communication through focus groups.

It s realized that for Human performance improvement, problems in workplaces which stem from the environment should be tackled so as to make to APNPDCL employees “as a place to work” maximizing employee satisfaction and productivity. Thrust is also getting placed on values and self actualization.

Competency development:

Considerable thought is now being given to competency development as a driving force to achieve strategic organizational objectives.

- Competency as required in the context of the present and future environment that accounts for sustained success within the frame work of organizational values:

- Open competition for building skill and capabilities through teams competing at regular intervals through oral and written presentations on topical themes.

- Formation of professional circles where ADEs and AEs meet every fortnight at Sub-division level to share their knowledge and experiences and discuss topical issues.

## Expansion plan for all capital investments

### Requirement in the Guidelines

Expansion plan for all capital investment (Guidelines 10g).

### APNPDCL’s Response

As on March, 31, 2005 the existing sub stations and lines and other relevant statistics of APNPDCL are given below:

| Sl. No. | Particulars | As on 31.3.2005 |
| --- | --- | --- |
| 1. | 33 KV sub stations | 556 Nos. |
| 2. | 33 KV lines | 7361 Km. |
| 3. | 11 KV lines | 44981 Km. |
| 4. | LT lines | 119499 Km. |
| 5. | Distribution Transformers |  |
|  | Single-phase | 30750 Nos. |
|  | Three-phase | 56092 Nos. |
|  | TOTAL | 86842 Nos. |
| 6. | Towns Electrified | 21 Nos. |
| 7. | Villages Electrified | 5558 Nos. |
| 8. | Hamlets Electrified | 5004 Nos. |
| 9. | Dalit Wadas Electrified | 8777 Nos. |
| 10 | Weaker Section Colonies Electrified | 4160 Nos. |
| 11 | Tribal Habitations | 1666 Nos. |
| 12. | Laying of conventional lines with SPV panels (Progress included in village electrification) | ---- |

During 2005-2006, 50 Nos. 33/11 KV Sub Stations are programmed. Out of which 32no’s Sub-stations are commissioned up to 11/2005.

In addition to the above programme for Rural Electrification (RE) is as follows:

| Consumer category | 2004-05 | 2005-06 | 2006-07 | 2007-08 | 2008-09 |
| --- | --- | --- | --- | --- | --- |
| Rural Electrification |  |  |  |  |  |
| Hamlets |  |  |  |  |  |
| Dalitwadas | 1430 | 970 | 842 | 606 | 445 |
| W.S Colonies |  |  |  |  |  |
| Tribal Habitations |  |  |  |  |  |

APNPDCL proposes to strengthen and expand its distribution network with the funds from various sources like APDRP, JBIC and REC, etc.  Details of investments proposed for the control period is summarized in section 2 of this document.

## Number and duration of interruptions on the network

### Requirement in the Guidelines

Number and duration of supply interruptions on the distribution networks.  Separately for 33 KV, 11 KV and LT systems.  Details of steps proposed to improve performance and monitoring. (Guidelines – 11a).

### APNPDCL’s Response

The following Measures are taken to reduce the number and duration of supply interruptions on the distribution networks for 33 KV, 11 KV and LT systems .

In an effort to improve the quality of supply to the consumers besides strengthening the network APNPDCL has taken many steps to minimise the interruptions in supply and improve the standards by upgradation/ improvement in the network, Reduction in rate of DTR failures, establishing the Load Dispatch Centres, Consumer service centres, pre-monsoon inspections. identification of nodal officer for monitoring the interruptions on industrial express feeders.

Upgradation/ Improvement in the Network:

In an effort to reduce the interruptions, the overloaded 33/11 KV substations are being identified. 38 Nos new 33/11 KV substations were erected during the year to avoid overloading of the 11 KV feeders. Further, in places characterized by overloaded DTRs and lengthy LT feeders, additional DTRs were erected to reduce interruptions. The no of such improvement DTRs erected during the year 2004-05 were 20,168 Nos.

Reduction of rate of DTR failures :

APNPDCL has undertaken many initiatives to reduce the rate of DTR failure. This has resulted in a reduction in the number of interruptions to the consumers. The DTR failure percentage has gradually comedown from 15.8% in 2003-04 to 11.2% in 2004-05.

Load Dispatch Centre:

A load dispatch centre was established at corporate office of  APNPDCL 0n 5.3.2004 which monitors the load position within the DISCOM and load reliefs if any will be  effected based on  State Load Dispatch centre’s instructions .Load Dispatch Centre also monitors  the power output  from generating  stations, load dispatch   to all circles and coordinates with the LMC’s of circles.  It will also monitor 33 KV line breakdowns and see that it is rectified at the earliest.

Load Monitoring cell:

NPDCL is monitoring the interruption & breakdowns through a well-organized   set-up established for the purpose of improving quality and reliability of supply for 24:00 hours. Each circle of APNPDCL has a Load Monitoring Cell (LMC) and all the LMC’s in turn report to a centralised LMC located at Corporate Office.  This cell monitors the interruptions in the 33 KV, 11 KV and LT distribution networks and also persue its rectification. APNPDCL has fixed the time limits within which the restorations normalcy of lines/equipment are to be achieved and it is the responsibility of its field officers to adhere the standards.

.Premonsoon inspections:

The field officers also conduct pre-monsoon inspection and rectify all the defects noticed by them during this process.  The operation wing of APNPDCL monitor overloaded lines and equipment and take necessary steps to balance the load.  Electrical clearances between trees and line/equipment are checked regularly to minimize interruptions due to falling of tree branches with live conductor, etc.

Special maintenance gangs are organized and officers are identified for attending the breakdowns in their jurisdiction in the shortest possible time.

Consumer service centres:

58 Nos. consumer service centres covering 65 subdivisions were established throughout APNPDCL. These consumer centres receive complaints from consumers and monitor till rectification.

The number and duration of interruptions and breakdowns, recorded at sub-stations for all 33 KV feeders and 11kv feeders are given in Table 11.a.1 and Table 11.a.2 respectively.  Regarding interruptions on the LT system, the complaints received at FOC (fuse off calls) centres are categorized under the following heads

- Individual fuse off calls

- Service wire defects

- LT fuse blown

- Lt line fault

- Horn Gap fuse blown

- DTR failure

The interruptions recorded in NPDCL urban area during 2004-2005 are shown in table 11.a.3. The average time taken for rectification of different categories of complaints is also shown against them. The monthly details of interruptions are also being furnished on-line to APERC in RIS Information Formats.

Number and duration of supply interruptions on 33 KV and 11 KV networks (consolidated) for 2003-04 and 2004-05 are tabled below for comparison.

| Interruptions & Breakdowns during 2003-04 & 2004-05 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | No. of Interruptions |  | Duration in hours |  | No. of Breakdowns |  | Duration in hours |  |
|  | 33 KV | 11 KV | 33 KV Lines | 11 KV Lines | 33 KV | 11 KV | 33 KV Lines | 11 KV Lines |
| 2003-04 | 20905 | 168224 | 20505 | 97883 | 658 | 7834 | 3471 | 28657 |
| 2004-05 | 13977 | 93738 | 12507 | 49975 | 595 | 7031 | 2683 | 25431 |

| Circle | 33 KV System |  |  |  |  |  | 11 KV System |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | No. of Feeders | Length of line Km | No. of Interr- uptions | Duration Hrs. | Per 100 Km |  | No. of Feeders | Length of line Km | No. of Interr- uptions | Duration Hrs. | Per 100 Km |  |
|  |  |  |  |  | No. | Duration Hrs |  |  |  |  | No. | Duration Hrs |
| Warangal | 57 | 1881.26 | 1901 | 1997.07 | 101.0 | 106.2 | 510 | 11931.64 | 17716 | 6899 | 148.5 | 57.8 |
| Karimnagar | 78 | 1896.89 | 6093 | 6173.1 | 321.2 | 325.4 | 518 | 10794.29 | 37259 | 33755 | 345.2 | 312.7 |
| Khammam | 40 | 1225.44 | 1446 | 153.4 | 118.0 | 12.5 | 248 | 6556.59 | 14909 | 1359 | 227.4 | 20.7 |
| Nizamabad | 73 | 1287.90 | 2393 | 3680.3 | 185.8 | 285.8 | 484 | 7479.95 | 5999 | 5333 | 80.2 | 71.3 |
| Adilabad | 29 | 1069.83 | 2144 | 503.34 | 200.4 | 47.0 | 229 | 8218.41 | 17855 | 2629 | 217.3 | 32.0 |
| TOTAL | 277 | 7361.32 | 13977 | 12507.2 | 189.9 | 169.9 | 1989 | 44980.88 | 93738 | 49975 | 208.4 | 111.1 |

Table 11.a.1 – 33 KV & 11 KV Interruptions during 2004-05

Table 11.a.2 – 33 KV & 11 KV Breakdowns during 2004-05

| Circle | 33 KV System |  |  |  |  |  | 11 KV System |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | No. of Feeders | Length of line Km | No. of breakdowns | Duration Hrs. | Per 100 Km |  | No. of Feeders | Length of line Km | No. of Breakdowns | Duration Hrs. | Per 100 Km |  |
|  |  |  |  |  | No. | Duration Hrs |  |  |  |  | No. | Duration Hrs |
| Warangal | 57 | 1881.26 | 93 | 349.77 | 4.9 | 18.6 | 510 | 11931.64 | 2316 | 5939.46 | 19.4 | 49.8 |
| Karimnagar | 78 | 1896.89 | 96 | 398.22 | 5.1 | 21.0 | 518 | 10794.29 | 1057 | 6418.30 | 9.8 | 59.5 |
| Khammam | 40 | 1225.44 | 98 | 369.10 | 8.0 | 30.1 | 248 | 6556.59 | 386 | 840.05 | 5.9 | 12.8 |
| Nizamabad | 73 | 1287.90 | 190 | 867.59 | 14.8 | 67.4 | 484 | 7479.95 | 2366 | 9571.17 | 31.6 | 128.0 |
| Adilabad | 29 | 1069.83 | 118 | 698.62 | 11.0 | 65.3 | 229 | 8218.41 | 906 | 2662.23 | 11.0 | 32.4 |
| Total | 277 | 7361.32 | 595 | 2683.30 | 8.1 | 36.5 | 1989 | 44980.88 | 7031 | 25431.21 | 15.6 | 56.5 |

On comparison of interruptions & breakdowns of 33 KV & 11 KV lines for the period 2003-04 & 2004-05 the performance of the lines is improved very much.  During 2003-04 the numbers of 33 KV & 11 KV line interruptions are 20905 and 168224 respectively and the corresponding interruption duration was 20505 hours and 97887 hours respectively.  Whereas during 2004-05 the no of interruptions came down to 13977 Nos. and 93738 Nos. and the corresponding durations were 12507 hours and 49974 hours respectively which indicates improvement in the performance compared to the previous year

It can be seen from the above the duration of interruptions per 100 Km of 33 kv line has decreased compared to previous year. This is due to better load management and avoiding of overload of 33 KV feeders causing tripping of feeders. Further, during interruptions of 33 kv feeders alternate supply being arranged to the 33/11 kv sub stations resulted in reduction of duration of interruptions.  Separate breakdown gangs kept alert and are equipped with proper tools and plants, safety tools, breakdown vans, communication instruction, etc., to attend immediately and rectify the cause of interruption .The aspect of maintaining quality of the plant and equipment during the time of purchase are also being reviewed and necessary corrective action being taken.

The reasons for breakdowns and interruptions in the APNPDCL in  33 KV, 11 KV and LT distribution network are largely due to the vulnerability of the vast network of overhead distribution lines to inclement weather, fouling of tree branches, snapping of conductors and overloaded network elements.  APNPDCL is taking the following steps to improve the performance of the distribution system.

- Replacement of existing aged conductors in a phased manner, since conductor snapping and jumper failures are found to be the major causes for interruptions. Refurbishment programme for lines has been taken up  and the work is in progress.

- Reinforcement of the conductor of overloaded lines, periodical tree cutting work to prevent falling of branches with conductors, conducting quarterly inspection of lines and timely rectification of defects noticed.

- Breakdown staffs for attending to supply interruptions are arranged round the clock by placing them in shift duties.

- Whenever any breakdowns or failure of supply arises the consumer can inform the concerned circle Load Monitoring Cell over the phone.

- Emergency vehicles are kept ready for breakdown staff round the clock to facilitate quick and timely deployment of staff and minimise the duration of interruptions.

- Safety appliances such as gloves, safety shoes, helmets and earthing rods are also provided to the breakdown staff.

- Lines are sectionalised by providing Air Break switches for isolating faulty sections and restoring supply to healthy sections to reduce interruption to the healthy section for prolonged periods till the rectification of the faulty section.

Interruptions in LT Distribution network of Municipalities for the year 2004-2005

The number and duration of interruptions of LT distribution network recorded in Municipal areas are as shown below.  Specific remarks of interruptions will be recorded at the local FOC, it will work for 24:00 hours and attend to the complaints.

Table 11.a.3

| Review of Complaints received at FOC Centres and rectification during 2004-05 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| District | Town/ Muncipality | Type of Complaints | No. of Complaints | No. of Consumers affected | Time for rectification (hrs) |  |  |  |  |  |
|  |  |  |  |  | Max. |  | Min. |  | Avg. |  |
| Warangal | Warangal | Individual FOCs | 13629 | 13629 | 04:00 |  | 00:30 |  | 02:30 |  |
|  |  | Service Wire defect | 249 | 249 | 04:00 |  | 03:00 |  | 02:00 |  |
|  |  | LT Fuses blown | 2145 | 29946 | 00:40 |  | 00:20 |  | 00:20 |  |
|  |  | LT line fault | 421 | 5988 | 02:00 |  | 01:00 |  | 01:30 |  |
|  |  | HG Fuse blown | 653 | 12407 | 00:40 |  | 00:20 |  | 00:20 |  |
|  |  | DTR Failure | 22 | 4513 | 08:00 |  | 04:00 |  | 06:00 |  |
|  |  | TOTAL | 17119 | 66732 |  |  |  |  |  |  |
|  | Jangaon | Individual FOCs | 3519 | 3519 | 4:00 |  | 1:00 |  | 2:30 |  |
|  |  | Service Wire defect | 118 | 118 | 4:00 |  | 1:00 |  | 2:00 |  |
|  |  | LT Fuses blown | 211 | 10550 | 0:30 |  | 0:05 |  | 0:20 |  |
|  |  | LT line fault | 167 | 1046 | 6:00 |  | 0.30 |  | 4:00 |  |
|  |  | HG Fuse blown | 16 | 2400 | 1:00 |  | 0:15 |  | 0:25 |  |
|  |  | DTR Failure | 8 | 1650 | 10 |  | 0.25 |  | 0.5 |  |
|  |  | TOTAL | 3872 | 18237 |  |  |  |  |  |  |
| - Karimnagar | Karimnagar | Individual FOCs | 1620 | 1620 | 04.:00 |  | 0.35 |  | 2.35 |  |
|  |  | Service Wire defect | 240 | 240 | 3.20 |  | 2.15 |  | 2.40 |  |
|  |  | LT Fuses blown | 150 | 4520 | 1.30 |  | 0.20 |  | 1.10 |  |
|  |  | LT line fault | 23 | 6280 | 1.25 |  | 0.45 |  | 0.50 |  |
|  |  | HG Fuse blown | 20 | 1920 | 1.40 |  | 0.20 |  | 0.55 |  |
|  |  | DTR Failure | 30 | 4520 | 7.00 |  | 4.00 |  | 6.00 |  |
|  |  | TOTAL | 1083 | 18100 |  |  |  |  |  |  |
|  | Jagtial | Individual FOCs | 951 | 951 | 04:00 |  | 0.25 |  | 2.35 |  |
|  |  | Service Wire defect | 102 | 102 | 3.9 |  | 1.4 |  | 2.24 |  |
|  |  | LT Fuses blown | 42 | 4632 | 3.3 |  | 0.2 |  | 1.25 |  |
|  |  | LT line fault | 45 | 956 | 1.5 |  | 0.35 |  | 1.1 |  |
|  |  | HG Fuse blown | 6 | 751 | 1.5 |  | 0.1 |  | 0.35 |  |
|  |  | DTR Failure | 19 | 4913 | 7 |  | 5.40 |  | 6.00 |  |
|  |  | Total | 1165 | 12305 |  |  |  |  |  |  |
|  | Korutla | Individual FOCs | 242 | 242 | 04:00 |  | 0.3 |  | 2.4 |  |
|  |  | Service Wire defect | 64 | 64 | 3.9 |  | 1.35 |  | 2.15 |  |
|  |  | LT Fuses blown | 32 | 1020 | 3.2 |  | 0.2 |  | 1.36 |  |
|  |  | LT line fault | 18 | 2543 | 1.25 |  | 0.3 |  | 1 |  |
|  |  | HG Fuse blown | 6 | 673 | 1.55 |  | 0.15 |  | 0.46 |  |
|  |  | DTR Failure | 33 | 4846 | 8.00 |  | 6.00 |  | 6.00 |  |
|  |  | TOTAL | 395 | 9388 |  |  |  |  |  |  |
| Karimnagar | Godavari Khani | Individual FOCs | 345 | 345 | 04:00 |  | 0.25 |  | 2.3 |  |
|  |  | Service Wire defect | 143 | 143 | 3.2 |  | 1.45 |  | 2.22 |  |
|  |  | LT Fuses blown | 25 | 1653 | 1.3 |  | 0.2 |  | 1.25 |  |
|  |  | LT line fault | 24 | 5482 | 1.2 |  | 0.54 |  | 1.2 |  |
|  |  | HG Fuse blown | 8 | 487 | 1.9 |  | 0.2 |  | 0.43 |  |
|  |  | DTR Failure | 19 | 3562 | 6 |  | 6.00 |  | 6.00 |  |
|  |  | TOTAL | 564 | 11672 |  |  |  |  |  |  |
| Khammam | Khammam | Individual FOCs | 998 | 2450 | 4.00 |  | 0.30 |  | 2.15 |  |
|  |  | Service Wire defect | 177 | 1820 | 4.00 |  | 1.00 |  | 3.5 |  |
|  |  | LT Fuses blown | 211 | 420 | 7.00 |  | 2.00 |  | 4.5 |  |
|  |  | LT line fault | 124 | 550 | 8.00 |  | 1.00 |  | 4.5 |  |
|  |  | HG Fuse blown | 0 |  | 4.00 |  | 1.00 |  | 2.00 |  |
|  |  | DTR Failure | 22 | 840 | 19:00 |  | 9.00 |  | 14 |  |
|  |  | TOTAL | 1532 | 6080 | 24.00 |  | 1.00 |  | 5.00 |  |
|  | Kothagudem | Individual FOCs | 617 | 1520 | 04.00 |  | 1.30 |  | 3.65 |  |
|  |  | Service Wire defect | 114 | 1408 | 4.00 |  | 1.30 |  | 3.65 |  |
|  |  | LT Fuses blown | 155 | 360 | 4.00 |  | 2.00 |  | 4.5 |  |
|  |  | LT line fault | 82 | 462 | 6.00 |  | 1.30 |  | 3.65 |  |
|  |  | HG Fuse blown | 27 | 211 | 5.00 |  | 1.00 |  | 3 |  |
|  |  | DTR Failure | 34 | 634 | 20.00 |  | 11.00 |  | 15.5 |  |
|  |  | TOTAL | 1029 | 4595 | 24.00 |  | 1.00 |  | 4.00 |  |
|  | Paloncha | Individual FOCs | 548 | 610 | 04.00 |  | 2.00 |  | 3.5 |  |
|  |  | Service Wire defect | 96 | 332 | 04.00 |  | 2.00 |  | 3.5 |  |
|  |  | LT Fuses blown | 152 | 246 | 03.00 |  | 2.00 |  | 2.5 |  |
|  |  | LT line fault | 25 | 122 | 6.30 |  | 1.30 |  | 3.8 |  |
|  |  | HG Fuse blown | 0 |  | 4.00 |  | 2.00 |  | 3.00 |  |
|  |  | DTR Failure | 14 | 392 | 20.00 |  | 10.00 |  | 15 |  |
|  |  | TOTAL | 835 | 1702 |  |  |  |  |  |  |
|  | Yellandu | Individual FOCs | 718 | 760 | 04.00 |  | 1.00 |  | 3.5 |  |
|  |  | Service Wire defect | 133 | 490 | 4.00 |  | 1.30 |  | 3.65 |  |
|  |  | LT Fuses blown | 176 | 239 | 4.00 |  | 1.30 |  | 3.65 |  |
|  |  | LT line fault | 98 | 384 | 6.00 |  | 1.00 |  | 3.5 |  |
|  |  | HG Fuse blown | 18 | 135 | 8.00 |  | 2.00 |  | 5 |  |
|  |  | DTR Failure | 10 | 425 | 18.00 |  | 10.00 |  | 14 |  |
|  |  | TOTAL | 1153 | 2433 |  |  |  |  |  |  |
| Nizamabad | Nizamabad | Individual FOCs | 7420 | 7420 | 4:00 |  | 1:00 |  | 2:00 |  |
|  |  | Service Wire defect | 490 | 490 | 3:00 |  | 4:00 |  | 3:00 |  |
|  |  | LT Fuses blown | 3446 | 3446 | 0:30 |  | 1:00 |  | 0:45 |  |
|  |  | LT line fault |  |  |  |  |  |  |  |  |
|  |  | HG Fuse blown | 1724 | 1724 | 0:30 |  | 1:00 |  | 0:45 |  |
|  |  | DTR Failure |  |  |  |  |  |  |  |  |
|  |  | TOTAL | 13080 | 13080 |  |  |  |  |  |  |
|  | Bodhan | Individual FOCs | 295 | 295 | 4:00 |  | 1:00 |  | 2:00 |  |
|  |  | Service Wire defect | 25 | 25 | 3:00 |  | 4:00 |  | 3:00 |  |
|  |  | LT Fuses blown | 336 | 336 | 0:30 |  | 1:00 |  | 0:45 |  |
|  |  | LT line fault |  |  |  |  |  |  |  |  |
|  |  | HG Fuse blown | 72 | 72 | 0:30 |  | 1:00 |  | 0:45 |  |
|  |  | DTR Failure |  |  |  |  |  |  |  |  |
|  |  | TOTAL | 728 | 728 |  |  |  |  |  |  |
| Nizamabad | Kamareddy | Individual FOCs | 136 | 136 | 4:00 |  | 1:00 |  | 2:00 |  |
|  |  | Service Wire defect | 31 | 31 | 4:00 |  | 4:00 |  | 3:00 |  |
|  |  | LT Fuses blown | 196 | 196 | 0:30 |  | 1:00 |  | 0:45 |  |
|  |  | LT line fault |  |  |  |  |  |  |  |  |
|  |  | HG Fuse blown | 55 | 55 | 0:30 |  | 1:00 |  | 0:45 |  |
|  |  | DTR Failure |  |  |  |  |  |  |  |  |
|  |  | TOTAL | 418 | 418 |  |  |  |  |  |  |
| Adilabad | Adilabad | Individual FOCs | 3345 | 3345 | 04.00 |  | 0.3. |  | 2.1 |  |
|  |  | Service Wire defect | 1012 | 1027 | 4.00 |  | 0.30 |  | 1.35 |  |
|  |  | LT Fuses blown | 845 | 865 | 1.20 |  | 1.12 |  | 0.40 |  |
|  |  | LT line fault | 134 | 250 | 1.35 |  | 0.50 |  | 1 |  |
|  |  | HG Fuse blown | 790 | 825 | 0.35 |  | 0.10 |  | 0.1 |  |
|  |  | DTR Failure | 45 | 1530 | 14.00 |  | 4.00 |  | 5.3 |  |
|  |  | TOTAL | 6171 | 7842 |  |  |  |  |  |  |
|  | Nirmal | Individual FOCs | 340 | 340 | 3.10 |  | 0.5 |  | 1 |  |
|  |  | Service Wire defect | 150 | 150 | 4.00 |  | 0.35 |  | 1.3 |  |
|  |  | LT Fuses blown | 230 | 230 | 3.15 |  | 0.55 |  | 0.45 |  |
|  |  | LT line fault | 80 | 1780 | 1.30 |  | 0.2 |  | 0.3 |  |
|  |  | HG Fuse blown | 190 | 340 | 2.55 |  | 1.15 |  | 1.15 |  |
|  |  | DTR Failure | 105 | 650 | 12.00 |  | 5.00 |  | 5.30 |  |
|  |  | TOTAL | 1095 | 3490 |  |  |  |  |  |  |
|  | Bhainsa | Individual FOCs | 160 | 160 | 4.00 |  | 0.50 |  | 1.15 |  |
|  |  | Service Wire defect | 30 | 30 | 4.00 |  | 0.50 |  | 2.00 |  |
|  |  | LT Fuses blown | 220 | 790 | 2.45 |  | 1.00 |  | 1.35 |  |
|  |  | LT line fault | 75 | 1610 | 3.00 |  | 0.40 |  | 0.44 |  |
|  |  | HG Fuse blown | 270 | 1350 | 3.20 |  | 1.25 |  | 1.00 |  |
|  |  | DTR Failure | 45 | 250 | 13.00 |  | 4.00 |  | 4.30 |  |
|  |  | TOTAL | 800 | 4190 |  |  |  |  |  |  |
|  | Mancherial | Individual FOCs | 1340 | 1340 | 3.55 |  | 0.48 |  | 1.17 |  |
|  |  | Service Wire defect | 190 | 190 | 3.15 |  | 0.45 |  | 1.15 |  |
|  |  | LT Fuses blown | 95 | 1030 | 3.15 |  | 0.40 |  | 0.55 |  |
|  |  | LT line fault | 93 | 2150 | 2.15 |  | 0.30 |  | 0.50 |  |
|  |  | HG Fuse blown | 79 | 1578 | 3.50 |  | 1.00 |  | 1.12 |  |
|  |  | DTR Failure | 14 | 480 | 9.00 |  | 4.00 |  | 4.15 |  |
|  |  | TOTAL | 1811 | 6768 |  |  |  |  |  |  |
|  | S.Kagaznagar | Individual FOCs | 132 | 132 | 04.15 | 2.23 |  | 1.15 |  |  |
|  |  | Service Wire defect | 84 | 84 | 2.55 | 0.50 |  | 0.52 |  |  |
|  |  | LT Fuses blown | 48 | 185 | 1.15 | 0.48 |  | 0.23 |  |  |
|  |  | LT line fault | 24 | 1250 | 3.00 | 2.26 |  | 1.25 |  |  |
|  |  | HG Fuse blown | 24 | 1630 | 5.25 | 2.00 |  | 0.45 |  |  |
|  |  | DTR Failure | 16 | 1466 | 8.30 | 4.15 |  | 5.25 |  |  |
|  |  | TOTAL | 328 | 4747 |  |  |  |  |  |  |
|  | Bellampally | Individual FOCs | 144 | 144 | 04.00 | 3.25 |  | 2.21 |  |  |
|  |  | Service Wire defect | 96 | 96 | 3.28 | 2.20 |  | 0.14 |  |  |
|  |  | LT Fuses blown | 72 | 763 | 3.12 | 0.50 |  | 1.25 |  |  |
|  |  | LT line fault | 26 | 1260 | 4.00 | 0.58 |  | 1.12 |  |  |
|  |  | HG Fuse blown | 22 | 962 | 1.15 | 0.55 |  | 0.45 |  |  |
|  |  | DTR Failure | 12 | 1321 | 9.30 | 4.00 |  | 3.00 |  |  |
|  |  | TOTAL | 372 | 4546 |  |  |  |  |  |  |

## Periods when Voltage/Frequency was beyond prescribed limits

### Requirement in Guidelines

Periods when voltage and/or frequency was beyond the prescribed limits on primary distribution (33 KV, 11 KV) networks.  Details of steps proposed to improve performance and monitoring. (Guidelines – 11b).

### APNPDCL’s Response

In order to improve the supply conditions in the primary distribution system 50 Nos. 33/11 KV Sub Stations and 15 MVAR LT capacitors placed on LV side of DTRs are proposed to be added during the current year out of which 32 sub stations are commissioned up to 30.11.2005.  The hourly voltage profile for 11 KV bus voltages on 30.09.2005, for the following sub stations are furnished hereunder:

| Sl.No. | Time | 11 KV Voltages at 33/11 KV SS at R.N.Gutta | 11 KV Voltages at 33/11 KV SS at Bodhan | 11 KV Voltages at 33/11 KV SS at Poduru |
| --- | --- | --- | --- | --- |
| 1 | 1:00 | 11.5 | 11.8 | 11.8 |
| 2 | 2:00 | 11.5 | 11.8 | 11.8 |
| 3 | 3:00 | 11.4 | 11.6 | 11.8 |
| 4 | 4:00 | 11.4 | 11.6 | 11.6 |
| 5 | 5:00 | 11.2 | 11.6 | 11.6 |
| 6 | 6:00 | 11.2 | 11.6 | 11.6 |
| 7 | 7:00 | 11.2 | 11.8 | 11.6 |
| 8 | 8:00 | 11.1 | 11.6 | 11.4 |
| 9 | 9:00 | 11.1 | 11.4 | 11.2 |
| 10 | 10:00 | 11.2 | 11.4 | 11.2 |
| 11 | 11:00 | 11.2 | 11.4 | 11.4 |
| 12 | 12:00 | 11.3 | 11.4 | 11.5 |
| 13 | 13:00 | 11.2 | 11.4 | 11.5 |
| 14 | 14:00 | 11.2 | 11.4 | 11.5 |
| 15 | 15:00 | 11.3 | 11.4 | 11.5 |
| 16 | 16:00 | 11.3 | 11.4 | 11.5 |
| 17 | 17:00 | 11.2 | 11.6 | 11.5 |
| 18 | 18:00 | 11.2 | 11.2 | 11.6 |
| 19 | 19:00 | 11.1 | 11.2 | 11.6 |
| 20 | 20:00 | 11.1 | 11.4 | 11.6 |
| 21 | 21:00 | 11.2 | 11.4 | 11.6 |
| 22 | 22:00 | 11.2 | 11.4 | 11.6 |
| 23 | 23:00 | 11.2 | 11:0 | 11.6 |
| 24 | 24:00 | 11.1 | 11.8 | 11.8 |

1. Frequency recorded in Financial Year 2004-2005

Normally the grid frequency is being maintained always above 49 hz only. In case of any disturbances the load shedding is resorted to regulate the system frequency . The load dispatch centre monitors the system frequency and is in constant touch with the 220/132/33 KV substations and the generating stations.

2. Rostering of agricultural feeders

Not less than 7 hours continuous supply in one spell or in two spells at the request of consumers is being ensured for agricultuaral consumers. In case of necessity due to the crop pattern additional hours of supply is ensured to save standing crops. Lighting supply ensured during the rest of the period by extending single phase supply.

3. Phase Supply to Agricultural feeders:

The following table provides the schedule of 3-phase supply to agricultural feeders in rural areas:

Time Periods

A 	04:30 	to 	11:30

B	11:30		to	18:30

## APPLICATIONS PENDING FOR CONNECTION

### Requirement in the Guidelines

Number of applications for supply or connection pending for more than six months, categorized by the reason why.  (Guidelines – 11c)

### APNPDCL’s Response

APNPDCL has taken all steps for regular monitoring of the status of applications for new connections and initiated for processing of applications and releases through the cuctomer service centres. Data on the number of connection applications pending for more than six months as on 31.03.2005 and categorization by the reason for pendency are presented in the Schedules 11.c.1 & 11.c.2 and Tables 11.c.1.1 through 11.c.1.5 respectively.  The customer category-wise pending applications have also been presented.

Schedule 11.c.1

Number of service connection applications other than agricultural pending more than six months as on 31.03.2005

*(Target of releasing agricultural connections per year depend on Go. AP policy)

| Category | Total |
| --- | --- |
| LT Supply |  |
| Domestic | 0 |
| Non-domestic & Commercial | 0 |
| Industrial | 35 |
| Cottage industries | 0 |
| Street lights & Water supply in local bodies | 175 |
| General purpose | 0 |
| Temporary supply | 0 |
| Total LT | 210 |
| HT Supply |  |
| Category 1 | 5 |
| Category 2 | 2 |
| Category 4 | 44 |
| Category 5 | 0 |
| Category 6 | 2 |
| - Others | 3 |
| TOTAL HT | 56 |
| Total | 266 |

| Category | Formalities not completed by Consumer | Being processed in Office | For want of material | Erection Work in Progress | Any Other Reason | Total |
| --- | --- | --- | --- | --- | --- | --- |
| LT Supply |  |  |  |  |  |  |
| Domestic | 0 | 0 | 0 | 0 | 0 | 0 |
| Non-domestic & Commercial | 0 | 0 | 0 | 0 | 0 | 0 |
| Industrial | 24 | 0 | 0 | 11 | 0 | 35 |
| Cottage industries | 0 | 0 | 0 | 0 | 0 | 0 |
| Street lights & Water supply in local bodies | 170 | 0 | 0 | 5 | 0 | 175 |
| General purpose | 0 | 0 | 0 | 0 | 0 | 0 |
| Temporary supply | 0 | 0 | 0 | 0 | 0 | 0 |
| Total LT | 194 | 0 | 0 | 16 | 0 | 210 |
| HT Supply |  |  |  |  |  |  |
| Category 1 | 0 | 0 | 0 | 5 | 0 | 5 |
| Category 2 | 2 | 0 | 0 | 0 | 0 | 2 |
| Category 4 | 5 | 0 | 0 | 32 | 7 | 44 |
| Category 5 | 0 | 0 | 0 | 0 | 0 | 0 |
| Category 6 | 2 | 0 | 0 | 0 | 0 | 2 |
| Others | 1 | 0 | 0 | 2 | 0 | 3 |
| TOTAL HT | 10 | 0 | 0 | 39 | 7 | 56 |
| Total | 204 | 0 | 0 | 55 | 7 | 266 |

| Circle wise break up of pending applications due to non-Payment of Development charges/Service Line charges and Consumpion Deposit not completed as on 31.03.2005 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CIRCLE | LT SUPPLY |  |  |  |  |  |  | HT Supply |  |  |  |  |  |  |
|  | CAT1 | CAT II | CAT III | CAT IV | CAT VI | CAT VII | CAT VIII | CAT I | CAT II | CAT IV | CAT V | CAT VI | OTHERS | Total |
| WARANGAL | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 6 |
| KARIMNAGAR | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 0 | 0 | 1 | 7 |
| KHAMMAM | 0 | 0 | 4 | 0 | 80 | 0 | 0 | 0 | 0 | 2 | 0 | 2 | 0 | 88 |
| NIZAMABAD | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| ADILABAD | 0 | 0 | 13 | 0 | 90 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 103 |
| TOTAL | 0 | 0 | 24 | 0 | 170 | 0 | 0 | 0 | 2 | 5 | 0 | 2 | 1 | 204 |

| Circle-wise break up of pending applications being processed as on 31.03.2005 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CIRCLE | LT SUPPLY |  |  |  |  |  |  | HT Supply |  |  |  |  |  |  |
|  | CAT1 | CAT II | CAT III | CAT IV | CAT VI | CAT VII | CAT VIII | CAT I | CAT II | CAT IV | CAT V | CAT VI | OTHERS | Total |
| WARANGAL | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| KARIMNAGAR | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| KHAMMAM | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| NIZAMABAD | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| ADILABAD | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| TOTAL | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Circle wise break up of pending applications due to want of material as on 31.03.2005 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CIRCLE | LT SUPPLY |  |  |  |  |  |  | HT Supply |  |  |  |  |  |  |  |
|  | CAT1 | CAT II | CAT III | CAT IV | CAT VI | CAT VII | CAT VIII | CAT I | CAT II | CAT IV | CAT V | CAT VI | OTHERS | Total |  |
| WARANGAL | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| KARIMNAGAR | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| KHAMMAM | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| NIZAMABAD | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| ADILABAD | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| TOTAL | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |

Circle-wise break up of pending applications due to erection work in progress as on 31.03.2005

|  | LT SUPPLY |  |  |  |  |  |  | HT Supply |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CAT1 | CAT II | CAT III | CAT IV | CAT VI | CAT VII | CAT VIII | CAT I | CAT II | CAT IV | CAT V | CAT VI | OTHERS | Total |
| WARANGAL | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 5 | 0 | 0 | 0 | 5 |
| KARIMNAGAR | 0 | 0 | 5 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 0 | 0 | 2 | 11 |
| KHAMMAM | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 0 | 1 | 0 | 0 | 0 | 3 |
| NIZAMABAD | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 2 |
| ADILABAD | 0 | 0 | 6 | 0 | 5 | 0 | 0 | 3 | 0 | 20 | 0 | 0 | 0 | 34 |
| TOTAL | 0 | 0 | 11 | 0 | 5 | 0 | 0 | 5 | 0 | 32 | 0 | 0 | 2 | 55 |

| Circle-wise break up of pending applications due to other reasons as on 31.03.2005 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CIRCLE | LT SUPPLY |  |  |  |  |  |  | HT Supply |  |  |  |  |  |  |
|  | CAT1 | CAT II | CAT III | CAT IV | CAT VI | CAT VII | CAT VIII | CAT I | CAT II | CAT IV | CAT V | CAT VI | OTHERS | Total |
| WARANGAL | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| KARIMNAGAR | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| KHAMMAM | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| NIZAMABAD | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 7 | 0 | 0 | 0 | 7 |
| ADILABAD | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| TOTAL | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 7 | 0 | 0 | 0 | 7 |

This page has intentionally been left blank

## Accidents

### Requirement in Guidelines

Number of reported fatal and non-fatal accidents for the year 2004-2005  differentiating between accidents involving human and those involving animals.  As analysis of steps to reduce such accidents must also be provided. (Guidelines -  11d)

### APNPDCL’s Response

Data on the number of fatal and non-fatal accidents differentiated between accidents involving human and those involving animals are presented in Table 11.d.1. The total number of fatal and non-fatal accidents recorded in APNPDCL in  2004-05 is shown in the Table below. The accidents tally includes departmental staff, contract labour and members of general public. The accidents involving non-departmental person are shown in Table 11.d.2.

Table 11.d.1 – Fatal and Non-fatal accidents

| Category | Total No. of Accidents in 2004-05 Department, Contract Labour & Public |
| --- | --- |
| Fatal |  |
| Human Beings | 202 |
| Animals | 191 |
| Total | 393 |
| Non – Fatal |  |
| Human Beings | 38 |
| Animals | 0 |
| Total | 38 |

Table 11.d.2 summarises the number of fatal and non-fatal accidents recorded in various circles for the year 2004-05.

Table 11.d.2 – statement showing non-departmental fatal/non-fatal accidents to general public/animals during the period from 01.04.2004 to 31.03.2005

| Sl. No. | Circle | HUMAN |  | ANIMAL |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Fatal | Non-Fatal | Fatal | Non-Fatal |
| 1. | WARANGAL | 73 | 6 | 54 | 0 |
| 2. | KARIMNAGAR | 33 | 2 | 26 | 0 |
| 3. | KHAMMAM | 30 | 17 | 62 | 0 |
| 4. | NIZAMABAD | 41 | 6 | 3 | 0 |
| 5. | ADILABAD | 25 | 7 | 46 | 0 |
|  | TOTAL | 202 | 38 | 191 | 0 |

Necessary action is being taken to review the issues like operation / construction / maintenance practices in place, training of manpower, providing proper tools, tackles and safety devices, etc., and taking corrective actions for improvement in this area.

### Remedial action to reduce accidents

- Almost all fatal or non-fatal cases have been due to electrocution of human or animals by coming into contact with live conductors.  Some of the accidents were also caused due to conductor snapping but continuing to be live. There have also been cases where the stay-wire or pole has inadvertently become energized.  Necessary steps are being taken to immediately rectify these defects, wherever noticed, to avoid accidents.

- For any lapse, suitable prompt action is taken against the concerned public or staff to discourage their negligence or casual approach.

- The concerned superior staff take deterrent action against all the erring field staff who fail to take safety precautions while working on lines and also ensure that adequate quantity of safety appliances are made available to all the concerned staff members.  Safety training courses are regularly conducted.

- Classes are being conducted for the workmen on payday or any suitable working day to enlighten them on safety measures and to avoid electrical accidents by proper utilization of safety devices and precautions as per the safety rules.  Normally, the employees in workmen category are involved in the departmental accidents

- Accidents occur for various reasons.  Some of them are listed below :

### Reasons for Departmental Accidents

    - Non-observance of safety rules.

    - Non-utilization of safety appliances by the workmen though they are supplied and trained to use them.  The person entrusted with work/supervision must be made accountable for not insisting on the usage of safety appliances and proper safety earthing practices by workmen under their control.

    - Non-operation of one of the blades of Air Break (AB) Switches, due to bad maintenance, leading to arcing and burn injuries.

    - Slipping from poles and O/H lines while on work, due to non-utilization of safety belt.

    - Attending of fuse off-call  complaints on the poles without using proper safety gloves, etc,.

    - Due to improper co-ordination and communication of line clearance or improper earthing of the line or equipment at the work spot.

    - Due to negligence or overconfidence to visually ensure that the AB switch is kept open while undertaking renewal or replacement of a blown Horn Gap (HG) fuse.

    - While working carelessly on the bottom portion of rusted rail poles, R.S joists or damaged PSCC/RCC poles in the ground, which cannot be easily detected.

    - Working at double feed points without working knowledge of the feeding arrangement and without ensuring that there is no back-feeding or charged condition of line.

    - Allowing un-authorized persons to work on line.

    - Inadvertent energizing of consumers’ installations without proper safety earthing and without checking whether they are ready or not.

    - In manned sub-stations, where the work of routine maintenance/breakdowns is taken up under direct supervision of responsible staff, accidents take place due to inadequate precautionary measures taken before the work is commenced.

    - On load closing or opening of AB Switches on 11 KV and 33 KV side should be avoided and the connected Circuit Breaker should be tripped before carrying out such operations.

### Reasons for Non-Departmental Accidents

- Snapped conductors of AAC/ACSR/Copper conductor of LT lines and 11 KV lines on account of gales and cyclones. (Aged copper conductors need to be replaced where necessary)

- Failure or breakage of stays resulting in leaning of the dead-end poles and consequent sagging of the live lines/wires with reduced ground clearances.

- Dashing of lorries and other loaded vehicles against electric poles/stays resulting in snapping of the conductors or dropping of the live conductors, etc,

- Members of the public coming into contact or within arcing distance of live lines while carrying out work such as digging of bore wells, assembling TV Antennas or Cutting the leaves of palm trees/coconut trees in the agricultural fields, etc.

- Non-replacement of worn-out service wires of the consumers in time leading to accidental energisation of bearer-wires., and leading to electrical accidents.

- Tampering with the electrical wiring by consumers trying to rectify faults in their own installations.

- Construction of houses or multi-storied buildings without maintaining proper clearances and set-back from existing live electrical lines or location of distribution transformer close to a building without maintaining mandatory electrical clearances.

Animals and cattle rearers are the worst hit among others in the non-departmental accidents, as they do not notice the loose or fallen electrical lines subsequent to a gale or storm. There is a need for these persons to be alerted and properly educated in the local villages.

All the line patrolling staff carry out immediate rectification of the snapped lines or conductors and damaged or leaning poles particularly in village areas.

### Plans for implementing preventive measures

- Standard construction practice to be ensured whenever new distribution lines are laid and energized.

- An alerting mechanism to be arranged in liaison with villages well in advance of charging any new distribution line.  Distributing pamphlets and making loudspeaker announcements may also be resorted to as an added precautionary measure.

- Consumers’ installations to be periodically inspected and tested and the defects rectified by the consumers.

- Clearances for O/H lines as specified in IE Rules 77, 78, 79, 80 and 81 to be strictly maintained and verified at the time of statutory inspections of lines and sub-stations, before charging.

- Guarding of the O/H lines by cradles be provided as required under rule 66 and 91 regarding safety and protective devices for rendering the lines electrically harmless in case it breaks.

- All metal supports of O/H lines and attached metallic fittings to be permanently earthed as per I.E Rule No.90.

- Danger Boards affixed wherever necessary.

- Even the RCC/PSCC poles used in the rural areas must be earthed with pig tailed GI wire.

- Guy insulators for stay wire be provided to prevent inadvertent energisation.

- Anti-climbing devices to be provided to prevent people from scaling towers or poles.

## INADEQUATE AND DEFECTIVE METERS

### Requirement in Guidelines

Number of inadequate and defective meters, programme and phasing of investment for replacement. (Guidelines – 11e)

### APNPDCL’s Response  :

The number of defective meters pertaining to the LT categories are presented in the following tables.

| Defective Meters for the year 2004-2005 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Circle | Defective as on 01.04.2004 | Replaced during 2004-2005` | Defective during 2004-2005 | Defective as on 01.04.2005 |
| Warangal | 20000 | 47818 | 41156 | 13338 |
| Karimnagar | 13375 | 27484 | 23155 | 9046 |
| Khammam | 6533 | 27095 | 23924 | 3362 |
| Nizamabad | 6911 | 31643 | 29230 | 4498 |
| Adilabad | 7852 | 24298 | 22598 | 6152 |
| TOTAL | 54671 | 158338 | 140063 | 36396 |

Burnt Meters for the year 2004-05

|  | Defective as on 01.04.2004 |  |  | Replaced during 2004-2005` |  |  | Defective during 2004-2005 |  |  | Defective as on 01.04.2005 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Circle | 1 Ph | 3 Ph | Total | 1 Ph | 3 Ph | Total | 1 Ph | 3 Ph | Total | 1 Ph | 3 Ph | Total |
| Warangal | 21 | 0 | 21 | 2286 | 25 | 2311 | 2762 | 25 | 2787 | 497 | 0 | 497 |
| Karimnagar | 504 | 0 | 504 | 2016 | 0 | 2016 | 1790 | 0 | 1790 | 278 | 0 | 278 |
| Khammam | 762 | 0 | 762 | 1478 | 43 | 1521 | 1114 | 43 | 1157 | 398 | 0 | 398 |
| Nizamabad | 483 | 9 | 492 | 969 | 52 | 1021 | 964 | 597 | 1561 | 478 | 554 | 1032 |
| Adilabad | 1093 | 3 | 1096 | 2650 | 16 | 2666 | 2263 | 24 | 2287 | 706 | 11 | 717 |
| TOTAL | 2863 | 12 | 2875 | 9399 | 136 | 9535 | 8893 | 689 | 9582 | 2357 | 565 | 2922 |

Stuck-up Meters for the year 2004-2005

| Circle | Stuck up as on 01.04.2004 |  |  | Replaced during 2004-2005 |  |  | Defective during 2004-2005 |  |  | Defective as on 01.04.2005 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 Ph | 3 Ph | Total | 1 Ph | 3 Ph | Total | 1 Ph | 3 Ph | Total | 1 Ph | 3 Ph | Total |
| Warangal | 19979 | 0 | 19979 | 45493 | 14 | 45507 | 38355 | 14 | 38369 | 12841 | 0 | 12841 |
| Karimnagar | 12871 | 0 | 12871 | 25468 | 0 | 25468 | 21365 | 0 | 21365 | 8768 | 0 | 8768 |
| Khammam | 5771 | 0 | 5771 | 25541 | 33 | 25574 | 22734 | 33 | 22767 | 2964 | 0 | 2964 |
| Nizamabad | 6277 | 142 | 6419 | 29720 | 902 | 30622 | 26909 | 760 | 27669 | 3466 | 0 | 3466 |
| Adilabad | 6749 | 7 | 6756 | 21602 | 30 | 21632 | 20288 | 23 | 20311 | 5435 | 0 | 5435 |
| TOTAL | 51647 | 149 | 51796 | 147824 | 979 | 148803 | 129651 | 830 | 130481 | 33474 | 0 | 33474 |

The reduction of defective meters in APNPDCL is as shown below:

Replacement of stuck up and burnt meters is a continuous process since meters become defective due to various factors.  The available staff and equipment are being utilized to the optimum level by organizing additional shifts. As a result of this the number of defective meters has come down from 54671 to  36,396  towards the end of 2004-05.

## UNAUTHORIZED CONNECTIONS

### Requirement in Guidelines :

Number of unauthorized connections (as a result of sample survey) to be reported by tariff category. Measures adopted to curb unauthorized consumption. Results and programme for the future.   (Guidelines – 11 f).

### APNPDCL’s Response  :

Detection of Pilferage of Energy Wing (DPE) comprising of engineers and field officers are vigilant and detect any un-authorized connections during their routine duties. As and when un-authorized services are detected immediate action is taken.

Anti Power Theft Squads (APTS) manned by Police Officers also assist in vigilance work, register the cases on complaints from  Engineers / DPE wing and undertake investigation, arrest or prosecution of defaulters.

Plans to reduce un-authorized consumption in future

      - LT Low Consumption Services:

- Sealing of terminal covers taken up by operation wing for avoiding tapping at meters

- The stuck up/defective meters are replaced by operation wing. Action is being taken for replacement of defective meters.  Defective meters of high value services are replaced on priority.

- In order to ensure better sealing, meter boxes are provided and sealed for new as well as existing services.

- Assistant Engineer (Operation) is instructed to take check readings of 1% of the total number of services under his jurisdiction.

- Majority of the meters are shifted to the front of the premises in all major towns to facilitate easy access and discourage meddling with the meter.

- Numbered seal bits are being used and records maintained so that tampered or re-placed seals could be detected and the culprits can be booked for counterfeiting the seals and theft of energy.

      - LT High Consumption Services:

- For all CT metered services standard pilfer proof boxes to accommodate CTs and meter are installed to ensure security from un-authorized tampering.

- All the Electro-mechanical type meters are being replaced with high accuracy meters.

- Assistant Engineer (Operation) has to take the meter readings monthly.

      - HT Services:

- Replacement of existing electro-mechanical meters by Electronic meters for accurate and correct recording. These meters have a facility of recording previous history of resetting of CMD, which would prevent meddling with CMD recording.

- Ensuring sealing of AB Switches to avoid possible tampering of metering equipment by switching off the supply.

- Fixing of accountability on ADE’s and DE’s for taking monthly reading for services up to 1000 KVA and above respectively and putting seals on MD setting mechanism and meter boxes.

- Distribution transformers are shifted outside the premises of consumer to avoid direct tapping from bare overhead lines in consumer premises.

Operation and DPE wings of APNPDCL undertake special drives individually and jointly to detect theft and un-authorized consumption cases. The following table indicates the number of cases booked during these drives.

| Month | No. Of cases booked |
| --- | --- |
| April, 2004 | 346 |
| May, 2004 | 1043 |
| June, 2004 | 1726 |
| July, 2004 | 2390 |
| August, 2004 | 3910 |
| September, 2004 | 4462 |
| October, 2004 | 1937 |
| November, 2004 | 1995 |
| December, 2004 | 2224 |
| January, 2005 | 2698 |
| February, 2005 | 2468 |
| March, 2005 | 1839 |
| TOTAL | 27038 |
| April, 2005 | 2129 |
| May, 2005 | 1415 |
| June, 2005 | 1394 |
| July, 2005 | 1465 |
| August, 2005 | 979 |
| September, 2005 | 723 |
| TOTAL | 8105 |

The provisions of Electricity Act- 03 on prevention of theft of energy are being publicised for increased public awareness on all plat forms of meeting held with the consumers, and during special revenue collection programmes.

      - Other Steps to Reduce un-authorized consumption include:

- Review of meter exceptionals and replacement of defective meters has been taken up on priority basis and this activity is being reviewed every month as one of the items on the performance indices of circles and by fixing accountability at every level. Sealing of meter terminal covers is nearing completion and sealed boxes are being provided for new services.

- In addition, special drives are being conducted by regular operation staff.  Intensive inspection of services are being undertaken to find out theft, malpractices, defects in meters, and billing and metering system.

- Already all the HT services have been provided with electronic meters to deter tampering by the consumer, which was prevalent in the case of electro-mechanical meters.

- Detection of pilferage of energy wing with technical staff and Anti Power Theft Squads (APTS) manned by police officers on deputation, are working under an officer of rank of Inspector General of Police who is under deputation from Government of Andhra Pradesh to curb the menace of theft of energy. The cases are reported on receipt of a complaint to the Engineers of APTS under Section 39 (Theft of Energy) and 44 (Pilferage of Energy by meddling with meters) of Indian Electricity Supply Act. DPE wing is reinforced by redeployment of staff and vigilance on suspected services is intensified.

- Accountability and responsibility for proper billing is fixed on all concerned staff.

- Officers review critical customers from the Meter Reading Books / Bill Book Abstracts and take check readings at various levels for monitoring Metered sales.

- The MBC (Metering, Billing and Collections) activity is being streamlined with inbuilt checks, regular review, supervision and prompt follow up action.

## REVENUE ARREARS.

### Requirement in Guidelines:

Amount of revenue arrears at beginning and end of year analysed age wise on a six monthly basis.  Reasons for non-collection and details of litigation involving collection of revenue of Rs.50,000 or more should also be provided.  (Guidelines 11 – g).

### APNPDCL’s Response:

#### Ageing Analysis of receivables LT

| Ageing Analysis of receivables LT Rs. In Crores |  |
| --- | --- |
| Duration | As on 31.3.2005 |
| 3-6 months old | 41.92 |
| 6-12 months old | 13.07 |
| More than 12 months old | 233.23 |
| Total | 288.22 |

#### Ageing Analysis of receivables HT

| Ageing Analysis of receivables HT |  |
| --- | --- |
| Duration | As on 31.3.2005 |
| 3-6 months old | 6.37 |
| 6-12 months old | 7.44 |
| More than 12 months old | 33.75 |
| Total | 47.56 |
| LT + HT | 335.77 |

Reasons for non-collection from debtors

As the cases filed in the court are pending for years together, the arrears could not be realized. By the time the cases are disposed off by the court the consumer is alienating the proposed bills leaving no asset to realize the dues.

For recovery of arrears through Revenue Recovery Act, one Deputy Thasildar for each circle is appointed on deputation from Revenue Department to speed up the process of recovery.

With the surcharge waival scheme announced in 12/2005, the licensee is expecting to collect a reasonable portion of arrears from Domestic consumers.

Details of Arrears above Rs. 50000 are provided in ANNEXURE-2 to this filing.

Details of Litigation:

Total amount pending in litigation has been identified as Rs.8.93 Crores as on April 1, 2005.

Break up of arrears on various accounts as on March 2005 is provided below for APNPDCL along with the figures of 2003-04.

| Arrears Analysis |  |  |
| --- | --- | --- |
| Particulars | 31-3-2005 | 31-3-2004 |
| Live services | 156.82 | 374.69 |
| State Govt. departments | 2.17 | -1.63 |
| State Govt. Undertakings | -3.63 | -3.66 |
| Central Govt. Deps./Undertakings | 41.37 | 44.24 |
| Local bodies | 4.28 | -2.69 |
| Court cases | 9.75 | 8.96 |
| Disconnected services | 122.54 | 130.6 |
| Debts decreed to be withdrawn | 0.27 | 0.25 |
| Debts under RR Act | 2.19 | 1.34 |
| Net Arrears | 335.77 | 552.10 |

A comparison of category-wise arrears for the consolidated distribution business is provided below:

| Category | Particulars | As on 31-03-2005 | As on 31.03.2004 |
| --- | --- | --- | --- |
| LT |  |  |  |
| I | Domestic | 233.10 | 197.74 |
| II | Non-Domestic | 24.97 | 19.84 |
| III | Industrial | 17.14 | 14.33 |
| IV | Cottage Industries | 0.33 | 0.13 |
| V | Agriculture | 9.51 | 272.31 |
| VI | Public Lighting | 3.73 | -2.4 |
| VII | General Purpose | -0.68 | -1.47 |
| VIII | Temporary | 0.11 | 0.11 |
| LT TOTAL |  | 288.22 | 500.59 |
| HT |  |  |  |
| I | Industrial Segregated | 48.86 | 51.69 |
| II | Industrial Non-Seg. | 0.96 | 0.85 |
| IV | Agriculture | 1.64 | 1.94 |
| V | Railway Traction | -2.61 | -1.59 |
| VI | Colony consumption | 0.06 | 0.1 |
| VII | Rescon | -1.34 | -1.48 |
| VIII | Temporary | 0.00 | 0 |
| HT TOTAL |  | 47.56 | 51.51 |
|  | LT + HT TOTAL | 335.77 | 552.1 |

NPDCL has pursued effective disconnection of default services and increased revenue collection initiatives. The year to year change is depicted below.

## METER READING AND COLLECTION:

### Requirement in Guidelines:

Data should be provided for each consumer category indicating (i) the normal meter reading cycle and the number of meter reading not carried out in accordance with the cycle and (ii) the number of consumer bills not served within 15 days of the meter reading.  Please also provide details of plans to improve the efficiency and punctuality of meter reading, the dispatch of consumer bills and cash collection. (Guide lines 11g)

### APNPDCL’s Response

HT TARIFFS

The tariffs are applicable for supply of electricity to H.T. Consumers having loads with a CMD OF 70 KVA and above and / or having a connected load exceeding 75 HP.  The following are the different categories notified under H.T. Tariffs.

H.T. Category – I			Industrial

H.T. Category – I (B)		Ferro Alloys

H.T. Category – II			Commercial (Non-Industrial)

H.T. Category-IV (A)		Irrigation and agricultural (Govt.)

H.T. Category-IV (B)		Irrigation and agricultural (Private)

H.T. Category-V			Railway Traction

H.T. Category – VI		Townships & Residential colonies

- Mode of Billing

Monthly

Meter Reading Schedule :

i.  HT Services with CMD 1 MVA and above:

Meter Readings are taken between 21 to 23rd of every month.

ii) HT Services with CMD below 1 MVA:

Meter Readings are taken between 24th to 27th of every month.

Computerization of HT Billing

A common software for billing of H.T. Services is developed In-House.  The database is in ‘ORACLE’.

Preparation of Bills

The Meter readings are brought to Headquarters and data entry is done.  Some units are bringing the readings on a floppy disk.  The bills are generated and handed over to the respective units.  The bills are posted to the consumers from Warangal only.

Bill date and time allowed for payment:

For the services with CMD 1 MVA and above, the bill date is 26th and for the services with CMD of below 1 MVA the bill date is the end of every month.  15 days time is allowed for payment of the bill.  After the due date, if the payment is not received, a 15 days notice is served on the consumer.  If any consumer fails to pay the bill amount even within the 15 days period, the service is ordered for disconnection without any further notice.

Collection Centres:

Collection Counters are available at every circle office for receiving the CC charges bills from the consumers.   Consumers can send cheques / DDs by post.

Additional charges for belated payments:

All consumers shall pay additional charges @ 0.07 paise per rupee per day of delay on the amount of the bill for the period of delay if he does not pay the bill within the due date of payment.

- LT TARIFFS

The tariffs are applicable for supply of electricity to LT Consumers with a connected load of 56 KW/75 HP and below.

The following are the different categories notified under LT Tariff.

1.    L.T. Category -  I			Domestic

2.    L.T. Category – II			Non-Domestic & Commercial

3.    L.T. Category – III(A)		Industrial Normal Category

4.    L.T. Category – III(B)		Industrial Optional Category

5.    L.T. Category – IV 			Cottage Industries

6.    L.T. Category -  V (A)		Agriculture

7.    L.T. Category -  V (B)		Tathkal metered

8.    L.T. Category -  VI			Local Bodies

9.    L.T. Category – VII			General Purpose

10.  L.T. Category – VIII			Temporary Supply

### Mode of Billing

### Monthly

(i)  All services coming under LT Category III, IV, VI & VIII.

(ii)  All high value services under Category I, II, V and VII as notified by the Board.  The    consumers whose monthly consumption is above 500 units under Category I and 400 units under Category II and 575 units under Category VII are considered as high value services as per the tariffs notified in BP Ms No.32 (Opn. Comml.) dt.29-07-96.

In all municipal towns Cat-I services are being billed monthly.

### Bi – Monthly

All the services under Category I, II and VII other than the high value services existing in the rural area and LT-V consumers under paying category are billed under Bi-monthly system.

### Slab – System

This is applicable to LT Category V Agricultural consumers under paying category where meters are not presently existing.

### Monthly Billing Programme

LT High Value Services are billed monthly.  Readings for these services are taken between 15th to 20th of the month and bills are issued by 25th of every month with bill date on the 1st of the succeeding month.  15 days time is allowed from the date of bill.

### Bi-Monthly Billing Programme

- After introduction of spot billing, readings are taken throughout the month for the services of the respective month and the respective group, which is coming up for billing as per cycle.

  - 1st to 10th First cycle

  - 11th 20th Second cycle

  - 21st to 30th Third cycle

- Readings are taken in the presence of the consumer and served on the spot; as such bill date is the date of reading.

- Due date of payment is15 days from the date of the bill.

- Date of disconnection is after 15 days from the due date of payment.

- Spot Billing system has been introduced through the area of NPDCL  to arrest the consumer complaints and improve the revenue demand duly serving the C.C. bills promptly on the spot.

- For re-connection of power supply after disconnection, the consumer has to pay reconnection fee plus additional charges on the bill amount @ 0.07 paise per rupee per day of delay calculated from the sixteenth day from the due date mentioned in the bill, up to the date of payment.

- The collection work is attended by the revenue cashiers of APNPDCL generally, except in some areas where the collection work is also entrusted to certain private collection agencies like Lion’s Club, Rotary Club, Chamber of Commerce, Setwen, Colony Welfare Association, Retired army personnel etc., and E-seva centres.

- In rural areas the collection programme is drawn by AAO/ERO with the consultation of field officers covering all the distributions, and communicated to the Revenue Cashier (RC) and section officers.  The RC will go to the Distribution as per programme and collect the CC charges and remit the same to the bank / ERO.  In addition to the field collections, regular cash collection centres also function in the main ERO.

- Spot collection / special Drive collections are also arranged for improvement of Revenue collections after the completion of R.C. Collection Programme

EROs Computerization:

The accounting and billing work of ERO’s has been totally computerized.  About 20 lakh services are maintained by the private accounting agencies and 10 lakh services handled by the in house computers.

Computerization of Accounts:

An ambitious programmed of integrated accounts Computerization was taken in NPDCL. A comprehensive and integrated accounts package namely, Systems Applications and Products(SAP), is being developed in NPDCL. The package includes important modules viz., Financial and Management Accounting, Inventory Management, Pay-roll Management, LOC Management, fixed Assets Register, Loan monitoring and MIS. The programme is presently running on pilot-project basis and within a short period, the licensee is going to implement in a complete shape.

## Write-off of arrears/arrears collection:

The licensee is making audit of receivables periodically either through the outsourcing agencies or in-house audit so as to verify in detail the long pending arrears and after having  physical and panchamas etc., the writing off of the arrears is taken place.

Remittances (LT & HT):

Revenue remittences of the field units i.e., EROs and HT revenue units at circle offices is being operated through Banks.  CC charges collected from the consumers have to be remitted on the same day to the extent possible.  The amount collected after closing hours of the bank have to be remitted on the very next day in the Bank. The banks have to transfer the amounts remitted by the EROs and HT revenue units by TT to their Headquarters branch at WARANGAL. Dues collected through E-seva are deposited as per the agreement. As regards to collection of arrears the matter is being pursued by our Vigilence cell besides using RR Act Judiciary.

Collection of CC charges from Disconnected Services (LT & HT)

If a service is under disconnection continuously for 3 months, a notice is served on the consumer allowing one month’s time for payment of all the dues outstanding and getting the supply restored.  If the consumer fails to clear the dues and obtain the reconnection order within the time limit allowed, his agreement is terminated.  The metering equipment and service lines are also dismantled and devoluted to stores.  The existing consumption deposit is adjusted against the CC charges dues.  Persons will be made to realize the balance CC charges as per the provisions of the RR Act.

Consumption deposit (LT & HT)

Consumption deposit is maintained equivalent to 3 months estimated consumption charges in case of bi-monthly services and 2 months in case of monthly services.  The adequacy of consumption deposit is reviewed every year and shortfall if any demanded from the consumers and in case of excess the same will be refunded against the CC bills of the consumer according to the terms and conditions of Tariff.