# introduction

This report presents the estimated cost of service for various consumer categories of the Central Power Distribution Company Limited (APCPDCL), for the year starting on April 1, 2007 and ending on March 31, 2008.  The objective of this report is to classify the costs into demand; energy and customer related components and then apportion the same to various customer categories.

The steps involved in the analysis are:

- Forecasting the energy and peak demand requirements for the power system in the year under consideration;

- Forecasting the energy and peak demand requirements at the transmission-distribution interface in that year;

- Estimating the energy and peak demand requirements for each customer category for that year;

- Estimating the costs of providing the energy and peak demand required for each customer category; and

- Classifying and allocating the above costs to various consumer categories of APCPDCL at the retail level.

(This space is intentionally left blank)

# COST OF SERVICE MODEL FOR APCPDCL

The cost of service calculations are based on the cost of service model COSTMOD, which handles an integrated utility, including generation, transmission and distribution.

In the present analysis we are dealing with the unbundled Andhra Pradesh electricity sector so that generation, transmission and four distribution companies are to be treated as separate entities.

The model, as currently used, calculates the cost of serving all customers categories of APCPDCL.

All financial input into the model is as per the ARR for the year 2007-08, including revenue, expenditure and asset data.

The following section summarises the main assumptions made in the various steps of the model.

Financial Reports

This forms the base for the income and expenses data for the APCPDCL.  The data is included for two financial years, 2006/07 & 2007/08.  Any of these years may be selected as the test year.  In the current work the test year selected is 2007/08.

Revenue Accounts

In this the data from Financial Reports may be modified to take into account any adjustments required to previously used data.  In the present work this feature is not required and has not been used.  It is retained for future use.

Power System Statistics

This part includes the system data required for the cost of service calculation.  The energy and losses in the system are included along with the data regarding the customers responsible for the corresponding sales and losses.  The percentage loss quoted is the loss in the distribution system and hence accounts for the energy that is unavailable for sale to the retail customers.

APCPDCL customers are segregated into LT and HT customers which includes EHT (220 kV and 132 kV), Sub-transmission (33kV) and distribution (11kV and LV).  The EHT customers are included as APCPDCL customers, even though they may be connected at 220 kV or 132 kV.  In this study, technical losses experienced in EHT system are covered by EHT, 33kV, 11kV and L.T loads. Hence they need to be apportioned to all loads in the system.

The HT customer categories in the APCPDCL system are;

- HT Customers - Industrial - Cat- I (11KV, 33KV and 220/132 KV) - HT Others - Cat- II (11KV, 33KV and 220/132 KV) - Irrigation and Agriculture - Cat-IV (11KV) - Railway Traction - Cat V (132KV) - RESCOs Cat - VI (11KV & 33 KV) - Temporary - Colony Consumption

The LT (400 Volts) customer categories in the APCPDCL system are;

- LT Customers - Domestic – category I - Non-domestic – category II - Industrial – category III - Cottage industries – category IV - Irrigation and Agriculture – category V - Public lighting – category VI - General purpose – category VII - Temporary – category VIII

This part includes the number of customers in each category and the consumption for each category as per the filing made by the DISCOM.

Operating Data

This is the final input module.  It includes the coincident and non-coincident demand data for the various customers.  The coincident demand is the estimated contribution of each category to the system peak demand and the non-coincident demand has been estimated from system load shapes derived and represents the peak demand of each customer category, irrespective of the time of day. Values used in this analysis are shown in Table 2-1.

Table 2-1

| Coincident Factors and Load Factors used | Coincidence Factor | Class Load Factor |
| --- | --- | --- |
| L.T Customers | Evening Peak |  |
| Domestic Category – Category – I | 94% | 48% |
| Non-Domestic – Category – II | 93% | 48% |
| Industrial – Category – III | 93% | 68% |
| Cottage Industries – Category IV | 93% | 68% |
| Irrigation and Agriculture – Category V | 15% | 45% |
| Public Lighting – Category VI | 93% | 47% |
| General Purpose – Category VII | 93% | 47% |
| Temporary – Category VIII | 93% | 47% |
| H.T Consumers |  |  |
| Industrial– Category I 11KV 33KV 220/132 KV | 84% | 66% |
|  | 84% | 66% |
|  | 82% | 70% |
| HT Others – Category II 11KV 33KV 220/132 KV | 90% | 60% |
|  | 90% | 60% |
|  | 80% | 56% |
| Irrigation and Agriculture – Category IV 11KV 33KV | 25% | 25% |
|  | 25% | 25% |
| Railway Traction – Category – V (132KV) | 90.0% | 47% |
| RESCOs Cat – VI 11KV 33KV | 10% 90% | 60% 60% |
| Temporary | 90% | 60% |
| Colony Consumption | 94% | 48% |

The DISCOM peak demands, both coincident and non-coincident are estimated using basic load shape synthesis model. Load shapes of different categories of consumers are constructed based on the Load Shapes data collected from the field. The following tabulation provides a derivation of the coincident peak demand, along with the assumptions for APCPDCL used in that derivation:

| CENTRAL | Energy (GWh) | Coincident Demand (MW) |
| --- | --- | --- |
| Sales | 21,417 | 2752 |
| Loss as % of input | 15.63% | 17.18% |
| Losses | 3,967 | 571 |
| Sub Total | 25,384 | 3,323 |

The load factor and coincidence factor included in the Model for each category are assumed based on a review of the characteristics of the loads and load mix in APCPDCL. One of the key assumptions is on the assessment of the timing of the system peak in the test year and this has a significant bearing on the coincidence factor for each of the customers. Recent data indicate that:

- The system peak demand of APCPDCL is occurring during Morning hours due to Agricultural loads. But only evening peak is considered because, system peak is occurring in the evening without agriculture load.

- So far, power supply to agricultural consumers has been restricted to seven hours/day. APTRANSCO has divided the number of agricultural consumers into two groups and at any point of time, only one group of agricultural consumers would be supplied with electricity. Consequently, the load on the system is relatively flat with load factors hovering around 82% to 85%. Such a load management technique is desirable as it helps to reduce the generation requirement considerably. This supply policy to agriculture (i.e., 7 hours/day) is expected to continue in the foreseeable future. Accordingly, the system peak load is considered during evening hours because nearly 10% of total agricultural load will be on the system in addition to other consumers. On the other hand, the peak load in the morning hours is expected to be higher than the evening peak because of the load management methodology adopted with predominant agricultural load being supplied during the morning hours (3-phase supply to agricultural consumers would be cut-off during evening peak hours)

- The load shape would change as and when the above restrictions are changed or lifted completely from the present practice.

- Based on above considerations, it is assumed that the system peak demand in the fiscal year 2007/08 will occur in the evening. In addition, it is assumed that APCPDCL will also have morning peaks. This affects the selection of coincidence factor for each category.

Expenditure Functionalization and Classification

This module is used to functionalize the expenditures into the generation, transmission and distribution components.  The components of the expenditures listed below are retained for use in the later modules.  While the power purchase cost is assigned to generation, the transmission and SLDC Charges are assigned to the transmission and the rest expenditures have been assigned to distribution.

- Purchase of power - Transmission & SLDC Charges - Generation of power - Repairs and maintenance - Employee costs - Administration and general expenses - Depreciation - Interest and financial charges - Other expenses

Expenditure Classification

This section classifies the expenditure into demand, energy and customer related items.  The options with respect to classification are;

- Demand - Energy - Load factor - 50/50% demand/energy - Customer - Rate base - Manual entry

The main expenditure of the DISCOM is for the purchase of electricity and this expenditure has been handled in two ways:

- The Energy cost is a “Flow-through” expenditure of APCPDCL’s share of the cost paid to its suppliers (APGENCO, IPPs and Others) for the energy component of the power purchased. The demand cost is APCPDCL’s share of the sum of the demand costs incurred for the purchase of power plus the Transmission and SLDC charges.

- APCPDCL’s share of cost of power purchase is treated as power purchase cost and is allocated to the generation.

All other expenditures have been classified as demand related and is assigned to the Transmission.

Expenditure Allocation

The expenditures are apportioned to the individual customer categories.  The demand allocation may be made on a coincident or non-coincident demand basis. As discussed earlier the selection made in the model is to allocate the demand costs based on the coincident demand for Generation, Transmission and Distribution.

Energy costs are allocated based on the category energy consumption and the customer costs are allocated in proportion to the number of customers in the category.

The next step is to allocate the non-rate revenue across consumer categories. As in other modules, the functionalization is not required.  The classification is to demand, energy and customer related components

In the allocation of these revenues, the allocation may be carried out using coincident or non-coincident peak demand for the demand component. In the model the allocation is currently based on coincident demand. However, the expenditures relating to the Distribution business is handled separately and allocated to the consumers on Non-coincident demand basis. Before doing so these expenses are apportioned to different voltages in proportion to its assets value.

The next step is Functionalization, Classification and Allocation of the Interest/ Return on Capital Employed (ROCE). The interest/ ROCE figure is taken as an input from the financial model.  The figure used in analysis is the allowable return on regulated rate base or the ROCE limited to interest expenses for the year.

In common with the other allocations, the interest/ ROCE allocation are organised by function (distribution), classified into the demand, energy and customer components and then allocated to the customers.

For the interest/ ROCE figure, this has been classified where 100% is assigned to demand.

A summary of the results of the model are the outputs and these are discussed in the next section and a comparison of revenues and costs by customers is made in this part of the computation.

# results

The following tabulation summarizes the results of the process:

- APCPDCL needs to handle 25,384 MU, which consist of sale of 21,417 MU to its customers and losses of 3,967 MU.

- Peak demand required by APCPDCL is 3,323 MW, which consist of 2,752 MW to serve the customers, and 571 MW of losses in the system.

- The average unit cost of supplying the customers of APCPDCL is estimated at 2.44 Rs/kWh.

- The expected unit revenue from APCPDCL customers at current tariff is 2.35 Rs/kWh.

Table-3.1 compares the cost of service and revenue expected from current tariffs for the major categories and Table - 11 provides detailed results for each category:

- Revenue and unit revenue at current tariffs.

- Allocated cost and unit allocated cost

- Revenue to cost ratio

- Weightage of each category

Subsidies have not been considered.

| Revenue to Cost Comparison |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Revenue from Sale of Power (Lakhs) |  |  | Subsidies (Lakhs) |  | Operating Costs (Lakhs) (incl. Profits, if any) |  |  | (Revenue + Subsidy) /Cost Comparison |  |  |  | Cost/Revenue Comparison |
| CONSUMER CATEGORIES |  | Revenue from Sale of Power | Average realisation per unit (paise/kWh) |  |  |  | Operating Expenditure adjusted from Non Rate Revenue and Profits/Losses | Net Operating Cost per total unit sales (paise/kWh) |  |  |  | Weight of the Consumer Category in % of the total MU sold |  |  |
| Low Tension Supply |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Domestic - Category I | 95,495 | 258.51 |  | 0 |  | 116,620 | 315.69 |  | 82% |  | 17.25% |  | 82% |
|  | Non-domestic Supply - Category II | 67,349 | 579.82 |  | - |  | 38,691 | 333.10 |  | 174% |  | 5.42% |  | 174% |
|  | Industrial Supply - Category III | 46,675 | 419.34 |  | - |  | 31,316 | 281.36 |  | 149% |  | 5.20% |  | 149% |
|  | Cottage Industries - Category IV | 288 | 197.87 |  | - |  | 397 | 272.78 |  | 73% |  | 0.07% |  | 73% |
|  | Irrigation and Agriculture - Category V | 1,402 | 1.91 |  | 0 |  | 151,374 | 205.89 |  | 1% |  | 34.33% |  | 1% |
|  | Public Lighting - Category VI | 7,311 | 181.41 |  | - |  | 16,291 | 404.24 |  | 45% |  | 1.88% |  | 45% |
|  | General Purpose - Category VII | 1,965 | 403.08 |  | - |  | 1,939 | 397.71 |  | 101% |  | 0.23% |  | 101% |
|  | Temporary - Category VIII | 558 | 620.00 |  | - |  | 392 | 435.90 |  | 142% |  | 0.04% |  | 142% |
|  | Total Low Tension Supply | 221,043 | 160.22 |  | 0 |  | 357,020 |  |  |  |  | 64.42% |  | 62% |
| High Tension Supply |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Industrial Segregated - Cat- I (11KV) | 87,016 | 418.30 |  | - |  | 48,738 | 234.29 |  | 179% |  | 9.71% |  | 179% |
|  | Industrial Segregated - Cat- I (33KV) | 88,163 | 354.14 |  | - |  | 51,095 | 205.24 |  | 173% |  | 11.62% |  | 173% |
|  | Indusl. Segregated - Cat-I (220/132KV) | 43,780 | 262.21 |  | - |  | 30,685 | 183.78 |  | 143% |  | 7.80% |  | 143% |
|  | Indusl. Non-Segregated - Cat- II (11KV) | 38,243 | 535.51 |  | - |  | 18,601 | 260.47 |  | 206% |  | 3.33% |  | 206% |
|  | Indusl. Non-Segregated - Cat- II (33KV) | 11,472 | 523.84 |  | - |  | 4,862 | 222.01 |  | 236% |  | 1.02% |  | 236% |
|  | Indusl. Non-Segre - Cat-II (220/132KV) | 1,643 | 458.17 |  | - |  | 724 | 201.84 |  | 227% |  | 0.17% |  | 227% |
|  | Irrigation and Agriculture - Cat-IV (11KV) | 896 | 122.68 |  | - |  | 1,976 | 270.63 |  | 45% |  | 0.34% |  | 45% |
|  | Irrigation and Agriculture - Cat-IV (33KV) | 4,158 | 227.01 |  | - |  | 3,915 | 213.71 |  | 106% |  | 0.86% |  | 106% |
|  | Railway Traction - Cat V (132KV) | 3,757 | 420.00 |  | - |  | 2,101 | 234.87 |  | 179% |  | 0.42% |  | 179% |
|  | RESCOs Cat - VI (11KV) | - | 0.00 |  | - |  | (0) | 0.00 |  | 0% |  | 0.00% |  | 0% |
|  | RESCOs Cat - VI (33KV) | 0 | 0.00 |  | - |  | 0 | 0 |  | 0% |  | 0.00% |  | 0% |
|  | Power Intensive Industries | - | 0.00 |  | - |  | 0 | 0 |  | 0% |  | 0.00% |  | 0% |
|  | Supply to Pondichery | - | 0.00 |  | - |  | 0 | 0 |  | 0% |  | 0.00% |  | 0% |
|  | Inter-State Supplies | - | 0.00 |  | - |  | 0 | 0 |  | 0% |  | 0.00% |  | 0% |
|  | Temporary | - | 0.00 |  | - |  | (0) | 0 |  | 0% |  | 0.00% |  | 0% |
|  | Colony Consumption | 2,334 | 350.00 |  | - |  | 1,972 | 295.68 |  | 118% |  | 0.31% |  | 118% |
|  | Total High Tension Supply | 281,461 |  |  | - |  | 164,668 |  |  |  |  | 35.58% |  | 171% |
| TOTAL |  | 502,504 | 234.63 |  | 0 |  | 521,688 | 243.59 |  | 96% |  | 100.00% |  | 96% |