COST OF SERVICE

2011-12

CENTRAL POWER DISTRIBUTION COMPANY OF A.P LIMITED

# Introduction

This report presents the estimated cost of service for various consumer categories of the Central Power Distribution Company Limited (APCPDCL), for the year starting on April 1, 2011 and ending on March 31, 2012.  The objective of this report is to classify the costs into demand; energy and customer related components and then apportion the same to various customer categories.

The steps involved in the analysis are:

- Forecasting the energy and peak demand requirements for the power system in the year under consideration;

- Forecasting the energy and peak demand requirements at the transmission-distribution interface in that year;

- Estimating the energy and peak demand requirements for each customer category for that year;

- Estimating the costs of providing the energy and peak demand required for each customer category; and

- Classifying and allocating the above costs to various consumer categories of APCPDCL at the retail level.

# COST OF SERVICE MODEL FOR APCPDCL

The cost of service calculations are based on the cost of service model developed for CPDCL. The model, as currently used, calculates the cost of serving all customers categories of APCPDCL.

All financial input into the model is as per the ARR for the year 2011-12, including revenue, and expenditure data.

The following section gives a brief overview of the Cost of Service model developed for AP CPDCL

Financial Input

Cost Elements as per the ARR

Technical Input

Energy Sales ( MU)

Coincident Demand (MW)

Non- Coincident Demand (MW)/ Contracted Demand (MW)

Expenditure Classification

Classification of the cost elements –

Demand Related

Energy Related

Customer Related

Expenditure Allocation

Allocation of classified costs to Consumer Categories

Model Outputs

Category-wise PP Cost allocation

Category-wise Transmission Cost allocation

Category-wise Distribution Cost allocation

Category-wise Consolidated Cost Summary

Financial Input Sheet

This forms the base for the income and expenses data for the APCPDCL.  The values are as per the ARR for the year 2011-12.

Technical Input Sheet

This part includes the system data required for the cost of service calculation.  The energy and losses in the system are included along with the data regarding the customers responsible for the corresponding sales and losses.  The percentage loss quoted is the loss in the distribution system and hence accounts for the energy that is unavailable for sale to the retail customers.

APCPDCL customers are segregated into LT and HT customers, which includes EHT (220 kV and 132 kV), Sub-transmission (33kV) and distribution (11kV and LV).  The EHT customers are included as APCPDCL customers, even though they may be connected at 220 kV or 132 kV.  In this study, technical losses experienced in EHT system are covered by EHT, 33kV, 11kV and L.T loads. Hence they need to be apportioned to all loads in the system.

The HT customer categories in the APCPDCL system are;

HT Customers - Industrial – Cat- I (11KV, 33KV and 220/132 KV) - HT Others – Cat- II (11KV, 33KV and 220/132 KV) - Irrigation and Agriculture – Cat-IV (11KV, 33KV and 220/132KV) - Railway Traction – Cat V (132KV) - Colony Consumption ( 11KV, 33KV) - Temporary

The LT (400 Volts) customer categories in the APCPDCL system are;

LT Customers - Domestic – category I - Non-domestic – category II - Industrial – category III - Cottage industries – category IV - Irrigation and Agriculture – category V - Public lighting – category VI - General purpose – category VII - Temporary – category VIII

Energy Sales in MU, Non- coincident demand and coincident demand data is entered for the above customer categories

The coincident demand is the estimated contribution of each category to the system peak demand and the non-coincident demand has been estimated from system load shapes derived and represents the peak demand of each customer category, irrespective of the time of day. Values used in this analysis are shown in Table 2-1.

Table 2-1

| Coincident Factors and Load Factors used | Coincidence Factor | Class Load Factor |
| --- | --- | --- |
| L.T Customers |  |  |
| Domestic Category – Category – I | 100% | 78% |
| Non-Domestic – Category – II | 100% | 74% |
| Industrial – Category – III | 92% | 89% |
| Cottage Industries – Category IV | 92% | 89% |
| Irrigation and Agriculture – Category V | 80% | 72% |
| Public Lighting – Category VI | 64% | 50% |
| General Purpose – Category VII | 64% | 50% |
| Temporary – Category VIII | 64% | 50% |
| H.T Consumers |  |  |
| Industrial– Category I 11KV 33KV 220/132 KV | 84% | 86% |
|  | 90% | 91% |
|  | 90% | 95% |
| HT Others – Category II 11KV 33 KV 220/132 KV | 87% | 81% |
|  | 50% | 72% |
|  | 60% | 64% |
| Irrigation and Agriculture – Category IV 11KV 33KV 220/132 KV | 87% | 81% |
|  | 50% | 72% |
|  | 60% | 64% |
| Railway Traction – Category – V (132KV) | 100% | 74% |
| Temporary | 87.5% | 81% |
| Colony Consumption – 11KV | 87% | 81% |
| Colony Consumption – 33KV | 50% | 72% |

The DISCOM peak demands, both coincident and non-coincident are estimated using basic load shape synthesis model. Load shapes of different categories of consumers are constructed based on the Load Shapes data collected from the field. The following tabulation provides a derivation of the coincident peak demand, along with the assumptions for APCPDCL used in that derivation:

| CENTRAL | Energy (MU) | Coincident Demand (MW) |
| --- | --- | --- |
| Sales | 33153 | 4095 |
| Loss as % of input | 12.33% | 16.71% |
| Losses | 4664 | 822 |
| Sub Total | 37817 | 4917 |

The load factor and coincidence factor included in the Model for each category are assumed based on a review of the characteristics of the loads and load mix in APCPDCL. The system peak demand of APCPDCL is occurring during 21.00 hrs.

Expenditure Functionalization

The new model is developed keeping in view the unbundled nature of the power sector in A.P, hence the expenditure pertaining to CPDCL is taken as per the ARR in the financial input sheet.

- Power Purchase Cost - Transmission & SLDC Charges - Repairs and maintenance - Employee costs - Administration and general expenses - Depreciation - Interest and financial charges - Other expenses

Expenditure Classification

This section classifies the expenditure into demand, energy and customer related items.  The options with respect to classification are;

- Demand - Energy - 80% Demand , 20% Customer - Customer - Manual entry

The fixed costs in the power purchase are treated as demand related expense and the variable cost of power purchase is treated as energy related expense.

Entire transmission cost is considered to be a demand related expense. The O & M expenditure in distribution is classified into demand and customer related in the ratio of 80:20. The same has been arrived at based on subjective judgement, as it is felt that some portion of the assets and employee expenses are used for catering to the needs of the customer such as customer service/call centers. The other cost elements in distribution viz ROCE, depreciation and other costs have been fully considered under demand related costs.

Expenditure Allocation

The expenditures, which have been classified into, demand, energy and consumer related are apportioned to the individual customer categories.

Power Purchase Cost Allocation:

Demand related costs of Power Purchase are primarily driven by the system peak. Hence they are allocated to customer categories based on the Coincident Demand. Energy costs in Power Purchase are allocated based on the loss-adjusted category energy consumption.

Transmission Cost Allocation:

The transmission costs (including PGCIL and ULDC) are considered as demand related cost and the same is allocated to LT categories based on Non-coincident demand and contracted demand (CMD) for HT categories

Distribution Cost Allocation:

a) Operation and Maintenance Expenditure

The demand related portion of O & M expenses are allocated to LT consumer categories based on non -coincident demand and contracted demand (CMD) for the HT consumer categories.

The customer related costs are allocated to customer categories based on the number of customers in each category.

b) ROCE

Return on capital employed is driven by assets and it is fully considered as demand related expense. ROCE is allocated to LT consumer categories based on non -coincident demand and contracted capacity for the HT consumer categories.

c) Depreciation

Depreciation expense is driven by the level of fixed assets in the utility and is entirely considered under demand related expenses. Depreciation is allocated to LT consumer categories based on non -coincident demand and contracted capacity for the HT consumer categories.

d) Interest on Consumer Security Deposit

This is allocated to consumer categories based on the energy consumption grossed up for losses.

Summaries of the results of the model are the outputs and these are discussed in the next section and a comparison of revenues and costs by customers is made in this part of the computation.

# Results

The following tabulation summarizes the results of the process:

- APCPDCL needs to handle 37817 MU, which consist of sale of 33153 MU to its customers and losses of 4664 MU.

- Coincident  Peak demand required by APCPDCL is 4917 MW, which consist of 4095 MW to serve the customers, and 822 MW of losses in the system.

- The average unit cost of supplying the customers of APCPDCL is estimated at 3.65 Rs/kWh.

- The expected unit revenue from APCPDCL customers at current tariff is 3.08 Rs/kWh.

Table-3.1 compares the cost of service and revenue expected from current tariffs for the major categories and Table – 11 provides detailed results for each category:

- Revenue and unit revenue at current tariffs.

- Allocated cost and unit allocated cost

- Revenue to cost ratio

- Weightage of each category

Subsidies have not been considered.

| CONSUMER CATEGORIES | Revenue from Sale of Power | Non - Tariff Income | Allocated Expenditure | Total Revenue /Cost Comparison | Weightage |
| --- | --- | --- | --- | --- | --- |
| Low Tension Supply |  |  |  |  |  |
| Domestic - Category I | 1943 | 34 | 2,718.9 | 73% | 19% |
| Non-domestic Supply - Category II | 1300 | 5.7 | 899.2 | 145% | 6% |
| Industrial Supply - Category III | 620 | 1.8 | 491.1 | 127% | 4% |
| Cottage Industries - Category IV | 3.2 | 0.0 | 6.2 | 52% | 0% |
| Irrigation and Agriculture - Category V | 35 | 13.7 | 3,087.0 | 2% | 24% |
| Public Lighting - Category VI | 201 | 1.4 | 397.5 | 51% | 3% |
| General Purpose - Category VII | 30 | 0.2 | 32.1 | 94% | 0% |
| Temporary - Category VIII | 2 | 0.0 | 1.4 | 161% | 0% |
| Total Low Tension Supply | 4133 | 57 | 7,633 | 55% | 57% |
| High Tension Supply |  |  |  |  |  |
| Industrial - Cat- I | 1370 | 3.0 | 1,032.0 | 133% | 9% |
| Industrial Segregated - Cat- I (33KV) | 2110 | 5.2 | 1,538.4 | 138% | 16% |
| Indusl. Segregated - Cat-I (220/132KV) | 1256 | 3.2 | 934.6 | 135% | 10% |
| HT Others - Cat-II | 732 | 1.2 | 428.1 | 171% | 3% |
| Indusl. Non-Segregated - Cat- II (33KV) | 202 | 0.4 | 107.0 | 189% | 1% |
| Indusl. Non-Segre - Cat-II (220/132KV) | 61 | 0.1 | 34.6 | 178% | 0% |
| Irrigation and Agriculture - Cat-IV | 18 | 0.1 | 50.7 | 35% | 0% |
| Irrigation and Agriculture - Cat-IV (33KV) | 35 | 0.1 | 59.6 | 59% | 0% |
| Irrigation and Agriculture - Cat-IV (132KV) | 162 | 0.6 | 179.4 | 91% | 2% |
| Railway Traction - Cat V (132KV) | 86 | 0.2 | 61.8 | 140% | 1% |
| Colony Consumption (11KV) | 37.0 | 0.1 | 29.4 | 125% | 0% |
| Colony Consumption (33KV) | 6 | 0.0 | 5.2 | 120% | 0% |
| Temporary | 4 | 0.0 | 2.9 | 126% | 0% |
| RESCOS Cat VI | - | - | - | - | 0% |
| Total High Tension Supply | 6080 | 14 | 4,464 | 137% | 43% |
| TOTAL | 10213 | 71 | 12,097 | 85% | 100% |

| Category | Cost of Service (Rs/kwh) |
| --- | --- |
| Low Tension Supply |  |
| Domestic | 4.32 |
| Non-domestic | 4.43 |
| Industrial | 3.89 |
| Cottage Industries | 3.95 |
| Agriculture | 3.80 |
| Public Lighting &RWS | 4.25 |
| General Purpose | 4.29 |
| Temporary | 4.24 |
| Total Low Tension Supply | 4.07 |
| High Tension Supply |  |
| Industrial (11KV) | 3.57 |
| Industrial (33KV) | 2.88 |
| Industrial (EHT) | 2.74 |
| HT Others (11KV) | 3.76 |
| HT Others (33KV) | 2.97 |
| HT Others (EHT) | 2.79 |
| Irrigation and Agriculture (11KV) | 4.61 |
| Irrigation and Agriculture (33KV) | 3.95 |
| Irrigation and Agriculture - Cat-IV (132KV) | 2.88 |
| Railway Traction - Cat V (132KV) | 3.35 |
| Colony Consumption (11KV) | 3.73 |
| Colony Consumption (33KV) | 3.33 |
| Temporary | 5.15 |
| Total High Tension Supply | 3.10 |
| TOTAL | 3.65 |