# Annexure B - Power Procurement Cost for Current Year Second Half and Ensuing Year

## Basis of estimating the quantity and cost of Power Purchased

This section discusses the methodology and assumptions used for estimating the quantum and corresponding cost of power purchase of the Licensee for the second half of the financial year ending March 31, 2006 and for the financial year ending March 31, 2007.

With the implementation of Multi-Buyer Model (MBM) in the state from June 9, 2005, each discom has been allocated a certain share of the generating stations contracted by APTransco.  Non-conventional Energy sources have been allocated to the discoms based on their locations while the two mini-power plants LVS and Sri Vathsasa have been allocated to EPDCL. The allocation percentages for the different discoms are as follows:

| S.No. | Name of the Distribution Company | Allocation Percentage |
| --- | --- | --- |
| 1 | EPDCL | 16.70% |
| 2 | SPDCL | 22.90% |
| 3 | CPDCL | 43.48% |
| 4 | NPDCL | 16.92% |

In the following paragraphs the capacities and availabilities of all the generating sources have been described. The actual energy availability in MU for each discom is simply the total generation availability for each source (except NCEs and the mini-power plants) multiplied by the percentage allocation as per MBM. The energy availability of NCEs for each discom has been shown separately.

## Capacity break-up of major generating sources

APGenco

The table below shows the installed capacities of the thermal and hydel generating stations of APGenco as on 30-9-2005 including share in the interstate projects.  The Licensee purchases the entire generation of APGenco under the terms of the PPA with the generator.

| Source | Installed Capacity (MW) |
| --- | --- |
| - Thermal |  |
| Kothagudem-A | 240.0 |
| Kothagudem-B | 240.0 |
| Kothagudem-C | 240.0 |
| Kothagudem-D | 500.0 |
| Ramagundam-B | 62.5 |
| Vijaywada | 1,260.0 |
| RTPP | 420.0 |
| Total Thermal | 2,962.5 |
| Hydel |  |
| Interstate projects: |  |
| Machkund, Orissa (AP share 70%) | 84.0 |
| T.B. Station, Karnataka (AP share 80%) | 57.6 |
| State projects: |  |
| Donkarayi | 25.0 |
| Upper Sileru | 240.0 |
| Lower Sileru | 460.0 |
| Srisailam right bank PH | 770.0 |
| Srisailam left bank PH | 900.0 |
| Nagarjunsagar | 815.6 |
| Nagarjunsagar right canal PH | 90.0 |
| Nagarjunsagar left canal PH | 60.0 |
| Nizam Sagar | 10.0 |
| Pochampadu | 27.0 |
| Penna Ahobilam | 20.0 |
| Mini hydro | 12.6 |
| Wind | 2.0 |
| Singur | 15.0 |
| Total hydel | 3588.8 |
| Total APGenco | 6551.3 |

Central Generating Stations

Southern region:

The Licensee has Power Purchase Agreements with Central Generating Stations of Southern Region to purchase power from Ramagundam STPS (“RSTPS”), NTPC Ramagundam Stage-III, Neyveli Lignite Corporation Ltd (“NLC”), Madras Atomic Power Station (“MAPS”) and Talcher stage II. The allocated and unallocated share of the licensee in the total capacity of the stations is provided below:

| Name of the Station | Capacity | AP Share (including un-allocated power) |  |
| --- | --- | --- | --- |
|  | MW | MW | % |
| NTPC-RSTPS | 2100 | 677.25 | 32.25 |
| NTPC-RAMAGUNDAM STAGE-III | 500 | 160.70 | 32.14 |
| NTPC-TALCHER-II | 2000 | 450.00 | 22.50 |
| NLC TPS II Stage-I | 630 | 105.71 | 16.78 |
| NLC TPS II Stage-II | 840 | 191.52 | 22.80 |
| MAPS | 340 | 31.76 | 9.34 |
| TOTAL |  | 1616.94 |  |
| NTPC-SIMHADRI | 1000 | 1000.00 | 100.00 |
| GRAND-TOTAL (CGS) |  | 2616.94 |  |

IPPs

The following IPPs are under commercial operations in the State:

- 216MW gas-based plant at Jegurupadu by GVK Industries (“GVK”);

- 208MW gas-based plant at Kakinada by Spectrum Power Generation Ltd.,

- 351.49 MW gas-based plant at Vijaywada by Lanco Kondapalli Power Ltd (“Lanco Kondapalli”).  .

- 220 MW gas based plant at Samalkota, East Godavari District by M/s. Reliance Power Ltd. (formerly M/s.BSES).

AP Gas Power Corporation Ltd (“APGPCL”): Joint Sector

APGPCL is a joint sector gas-based power project. The allocation of power from this project is in proportion to the equity share capital of participating industries.  The total installed capacity of the project along with the Licensee’s share is as given below:

| Source | Installed Capacity (MW) | APTransco Share (MW) | APTransco Share (%) |
| --- | --- | --- | --- |
| Stage I | 100 | 16 | 16 |
| Stage II | 172 | 42.8 | 25.0 |
| Total | 272 | 58.8 | 21.62 |

Inter-State purchases

PTC & NTPC V V N L

The Licensee proposes to procure power from Power Trading Corporation, NTPC Vidyut Vyapara Nigam Ltd., Tata Power, Reliance Energy Trading Ltd. Adani etc., on need basis.

Non-Conventional Energy (NCE) Sources

The capacity installation of NCE projects in the state for FY 2006 and FY 2007 are as follows:

| Type of Project | FY 2006 (MW) | FY 2007 (MW) |
| --- | --- | --- |
| Bio Mass Power Projects | 168.58 | 180.86 |
| Bagasse Cogeneration Projects. | 107.83 | 135.14 |
| Wind Power Projects | 29.07 | 29.07 |
| Mini Hydel Power Projects | 34.30 | 46.68 |
| Industrial Waste Based Power Projects | 13.20 | 21.39 |
| Municipal Waste Based Power Projects | 11.60 60 | 11.60 |
| TOTAL | 364.57 | 427.74 |

Mini-Power Plants

APTransco had entered into adhoc agreement with LVS (36.8 MW) for purchase of power as per direction of Supreme Court and had a PPA with Sri Vathsa (17.202 MW) power plant. These have been allocated to EPDCL. APDISCOMs propose power purchase from CPPs of VSP, NBFAL, Sponge Iron India Ltd, Heavy Water Plant etc., during the year 2006-07 on need basis at the rates approved by the Commission.

### Expected major capacity additions during the FY 2007

| S. No. | Unit |  | Type | Capacity (MW) |
| --- | --- | --- | --- | --- |
| 1 | #### GVK Extension | #### IPP | #### Gas | 220 |
| 2 | Vemagiri | IPP | Gas | 370 |
| 3 | Konaseema EPS Oakwell Power Ltd. | IPP | Gas | 445 |
| 4 | Gouthami | IPP | Gas | 464 |
| 5 | RTPP-Stage II | APGenco | Coal | 420 |

The licensee expects new capacity additions of 1499 MW from GVK – Extension project, Vemagiri, Goutami and Konaseema during FY 2007.  Generation is expected to be available from the above projects from 7/2006 when additional gas of 1.18 MCMD is expected to be made available.  Considering the availability of additional gas, the expected additional capacity would be only 300 MW during FY 2007.

In addition two units of RTPP-Stage II of 210 MW each are expected to be commissioned in July and October of 2006.

### Basis of estimation of power availability for the FY 2007

#### APGenco

Thermal:

The power availability for the second half of FY 2006 has been estimated by the Licensee based on actual performance for the first half of FY 2006 and the expected plant performance and maintenance schedules of APGenco for the remaining six months of FY 2006.

| APGENCO Thermal ( Energy Availability- MUs) |  |  |  |
| --- | --- | --- | --- |
| Sl No | Station Name | H2 2005-06 | 2006-07 |
| 1 | VTPS- I | 1,606 | 2,781 |
| 2 | VTPS- II | 1,539 | 2,955 |
| 3 | VTPS- III | 1,473 | 3,046 |
| 4 | RTPP | 1,601 | 3,033 |
| 5 | RTPP Stage-II |  | 1,098 |
| 5 | KTPS-A | 854 | 1,692 |
| 6 | KTPS-B | 855 | 1,734 |
| 7 | KTPS-C | 913 | 1,734 |
| 8 | KTPS-D | 1,828 | 3,611 |
| 9 | RTS-B | 240 | 394 |
|  | Total | 10,908 | 22,079 |

Hydel:

The availability from hydel plants of APGenco for the second half of FY 2006 been estimated taking into consideration of inflows in the reservoirs of Srisailam and Nagarjuna Sagar owing to good rainfall in the Krishna catchment areas during the monsoon.

The projection for ensuing year is based on the expected reservoir levels at end of current year and the assumption of a normal rainfall in the ensuing year.

The following table shows the projected availability for FY 2006 and FY 2007:

| APGENCO Hydel ( Energy Availability-MUs) |  |  |  |
| --- | --- | --- | --- |
| Sl No | Station Name | H2 2005-06 | 2006-07 |
| 1 | MACHKUND PH AP Share | 277 | 400 |
| 2 | TUNGBHADRA PH AP Share | 114 | 128 |
| 3 | USL | 237 | 427 |
| 4 | LSR | 410 | 1301 |
| 5 | DONKARAYI | 58 | 87 |
| 6 | SSLM ( Right Bank) | 1059 | 1746 |
| 7 | NSPH | 889 | 2021 |
| 8 | NSRCPH | 116 | 138 |
| 9 | NSLCPH | 57 | 60 |
| 10 | POCHAMPAD PH | 82 | 83 |
| 11 | NIZAMSAGAR PH | 2 | 8 |
| 12 | PABM | 4 | 7 |
| 13 | MINI HYDRO&OTHERS | 7 | 12 |
| 14 | SINGUR | 14 | 5 |
| 15 | SSLM LCPH | 706 | 1164 |
|  | Total | 4031 | 7586 |

#### CGS

NTPC- Southern region: The estimate of availability from the CGS of the Southern Region for the second half of FY 2006 is based on the actual performance for the first half of FY 2006 and the expected performance for the second half of FY 2006. The 500 MW, NTPC Ramagundam Stage-III was under commercial operation w.e.f. 25-3-2005.

Talcher Stage 2: The last 500 MW unit in the 4 x 500 MW Talcher-II power station has been declared commercial operation w.e.f. 1-8-2005.  The allocation of power from Talcher-II STPS revised to 22.5% w.e.f 1-11-2005 by CEA.

The total power availability estimate from CGS for FY 06 and FY 07 is as stated below:

#### NTPC- Simhadri: Availability for second half of FY 06 and FY 07 is as declared by NTPC (Simhadri)

| Central Generating Stations ( Energy Availability-MUs ) |  |  |  |
| --- | --- | --- | --- |
| Sl No | Station Name | H2 2005-06 | 2006-07 |
| 1 | NTPC (SR) | 2,246 | 4,632 |
| 2 | NTPC- Simhadri | 3,439 | 7,400 |
| 3 | NTPC (SR) Stage III ( Simhadri Ext) | 512 | 1,115 |
| 4 | NLC TS-II - Stage I | 301 | 600 |
| 5 | NLC TS-II - Stage II | 512 | 1,087 |
| 6 | NPC-MAPS | 78 | 218 |
| 7 | Talcher - Stage 2 | 1,501 | 2,936 |
|  | Total | 8,589 | 17,988 |

#### APGPCL

The Licensee has not projected any power available from the unutilized capacity of other shareholders in APGPCL in the 2nd half of FY 2006 as it is in the nature of infirm power.

As per the agreement with APGPCL, unutilized energy in stage-I has to be purchased by AP Discoms.

| APGPCL Allocated Capacity (Energy Availability- MUs ) |  |  |  |
| --- | --- | --- | --- |
| Sl No | Station Name | H2 2005-06 | 2006-07 |
| 1 | APGPCL I - Allocated capacity | 51 | 106 |
| 2 | APGPCL II - Allocated capacity | 140 | 277 |
|  | Total | 191 | 383 |

#### IPPs

The availability of power from the generating stations of GVK, Spectrum, Lanco Kondapalli and BSES has been projected as declared by the IPPs.

| IPP's (Energy Availability-MUs ) |  |  |  |
| --- | --- | --- | --- |
| Sl No | Station Name | H2 2005-06 | 2006-07 |
| 1 | GVK | 682 | 1364 |
| 2 | Spectrum | 673 | 1346 |
| 3 | Kondapalli (Gas) | 1151 | 2303 |
| 4 | BSES | 469 | 938 |
|  | Total | 2976 | 5951 |

The projections for FY 2006 H2 and FY 2007 are based on existing gas supply position.

The availability of power from generating stations of GVK Extension, Vemagiri, Goutami and Konaseema has been projected based on the availability of expected additional gas of 1.18 MCMD from 1-7-2006 is considered for pro-rata allocation to the 4 upcoming gas based power projects.

| New IPPs | FY 2007 (projected) All figures in MU |
| --- | --- |
| #### GVK Extension | 394.86 |
| Vemagiri | 588.71 |
| Goutami | 420.69 |
| Konaseema EPS Oakwell Power Ltd. | 343.42 |

#### Availability from stations with Discom Specific Allocations

The NCE projects and the two mini power plants are not allocated on pro-rata basis but allocated to specific discoms based on their locations. The availability from these sources for each discom is as shown below:

##### Mini Power Plants:

| Mini-Power Plants Allocated to EPDCL ( Energy Availability-MUs) |  |  |  |
| --- | --- | --- | --- |
| Sl No | Station Name | H2 2005-06 | 2006-07 |
| 1 | Srivathsa | 60 | 110 |
| 2 | LVS | 0 | 0 |
|  | Total | 60 | 110 |

| NCE DISCOM-wise Allocation ( Energy Availability- MUs) H2 2005-06 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl No | Station Name | EPDCL | SPDCL | CPDCL | NPDCL | Total ( Type-wise) |
| 1 | NCE - Bio-Mass | 85 | 307 | 104 | 76 | 572 |
| 2 | NCE - Bagasse | 90 | 73 | 33 | 50 | 246 |
| 3 | NCE - Municipal Waste to Energy | 0 | 19 | 21 | 0 | 40 |
| 4 | NCE - Industrial Waste based power project | 33 | 0 | 0 | 0 | 33 |
| 5 | NCE - Wind Power | 0 | 2 | 17 | 0 | 19 |
| 6 | NCE - Mini Hydel | 5 | 10 | 31 | 6 | 51 |
| 7 | NCE - NCL Energy Ltd | 2 | 2 | 4 | 2 | 10 |
|  | Total Availability ( DISCOM-wise) | 214 | 413 | 210 | 133 | 970 |

##### Non-Conventional Energy (NCE) Sources

The availability of power from non-conventional sources is shown below:

| NCE DISCOM-wise Allocation ( Energy Availability- MUs) EY 2006-07 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl No | Station Name | EPDCL | SPDCL | CPDCL | NPDCL | Total ( Type-wise) |
| 1 | NCE - Bio-Mass | 171 | 666 | 220 | 153 | 1210 |
| 2 | NCE – Bagasse | 175 | 190 | 66 | 101 | 532 |
| 3 | NCE - Municipal Waste to Energy | 0 | 33 | 37 | 0 | 70 |
| 4 | NCE - Industrial Waste based power project | 92 | 0 | 14 | 0 | 107 |
| 5 | NCE - Wind Power | 0 | 4 | 29 | 0 | 33 |
| 6 | NCE - Mini Hydel | 12 | 36 | 62 | 12 | 122 |
| 7 | NCE - NCL Energy Ltd | 3 | 5 | 9 | 3 | 20 |
|  | Total Availability ( DISCOM-wise) | 454 | 935 | 437 | 269 | 2095 |

#### Losses external to APTransco system

The losses external to the APTransco system are estimated to be 3.96%. This is applicable for procurement of power from Central Generating Stations.

#### Summary

A summary of the source wise current estimate of availability for FY 2006 H2 and FY 2007 is presented below.

| Summary Energy Availability Table- MUs |  |  |  |
| --- | --- | --- | --- |
| Sl No | Sector | H2 2005-06 | 2006-07 |
| 1 | APGENCO - Thermal | 10,908 | 22,079 |
| 2 | APGENCO - Hydel | 4,031 | 7,586 |
| 3 | CGS | 8,589 | 17,988 |
| 4 | APGPCL | 191 | 383 |
| 5 | IPPs | 2,976 | 7,699 |
| 6 | NCE | 970 | 2,095 |
| 7 | Mini Power Plants | 60 | 110 |
|  | Total | 27,725 | 57,940 |

## Power purchase tariffs

### APGenco

A fixed cost of Rs. 1564.68 crores for  FY 2007 has been estimated by APPCC based on the proposal submitted by APGENCO and the same has been  considered in the ARR. In addition, Rs.141.49 crores is the fixed cost for RTPP Stage II. The total fixed cost is thus Rs.1706.17 crores.

The station-wise variable rates for APGENCO thermal plants that have been adopted for second half of FY 2005-06 and full year FY 2006-07 are as shown below. These are based on APGENCO FCA bill for the month of September 2005.  The variable cost of RTPP Stage II is Rs.118ps/kwh

| Station | Variable rate (Ps./kWh) |
| --- | --- |
| VTPS (I, II, III) | 124.414 |
| RTPP | 148.908 |
| RTPP Stage II | 118 |
| KTPS (A, B, C) | 103.564 |
| KTPS- D | 91.225 |
| RTS-B | 102.691 |

VARIABLE COSTS FOR SLBPH

For Srisailam Laft Bank power house, a weighted average variable cost of Rs. 1.156 / unit (Non-monsoon = Rs. 1.49, Monsoon = Rs. 0.91) has been adopted. The period of June-October has been considered as Monsoon and November-March has been considered as Non-Monsoon.

### Talcher Stage-2

CERC vide order dt.14-1-2005 approved provisional fixed charges of Rs.690.54 Crs. w.e.f. 1-11-2004 for Units-I, II and III.  The 500 MW has been declared commercial operation w.e.f. 1-8-2005.  CERC vide order dt.1-9-2005 approved provisional fixed charges of Rs.990.7265 Crs. w.e.f. 1-8-2005.  Fixed charges payable by APDISCOMs for 2006-07 are estimated based on revised allocation of 22.5% w.e.f. 1-11-2005.  The variable cost of September’ 2005 has been considered for projecting the cost of H2 FY 06 and FY 07.

| Fixed charges for Units 1,2 & 3 upto 31-10-05 | Rs.115.41 Crs. |
| --- | --- |
| Fixed charges for all 4 units for 5 months | Rs.92.88 Crs |
| Total Fixed Charges for FY 2006 | Rs.208.29 Crs |
| Fixed charges for fY 2007 | Rs.222.91 Crs. |
| Variable Charge | Rs. 0.8038/Unit |

### NTPC Ramagundam Stage-I, II & III.

The tariff applicable w e f 1.4.2005 is yet to be approved by CERC. Fixed cost for NTPC Ramagundam (2100 MW) is projected based on September 2005 bill. Fixed cost for Ramagundam Stg-III is projected based on CERC order dt.10-5-2005.

For both stations the variable rate charged for the month of September 2005 has been adopted for the second half of FY 2006 and for FY 2007.

NTPC Ramagundam STPS (2100 MW)

- Total capacity charges of 2100 MW Ramagundam station = Rs. 451.96 Crs/year.

- Capacity charges payable by APTRASCO for 677.25 MW entitlement=Rs.145.8 Crs/year.

- Variable charge = Rs.1.0022/Unit.

NTPC Ramagundam Stage-III  (500 MW)

- Total capacity charges of 500 MW Ramagundam Stg-III = Rs. 235.14 Crs

- Capacity charges payable by APTRANSCO for 160.7 MW entitlement = Rs.75.6 Crs.

- Variable charge = Rs.0.9747/unit.

NLC TPS II

CERC vide order dt.2-11-2005 approved provisional capacity charge and provisional energy charge which shall be computed on the basis of 80% of the lignite price in the respective years considered by the petitioner in the petition.

Based on CERC order projections for FY 06 and FY 07 are estimated as follows.

Tariff Estimates for FY 2005-06 and FY 2006-07

Stage –I

Fixed cost based on the CERC order = Rs. 114.03 Crs/year.

Fixed cost payable by APDISCOMs for 105.71 MW entitlement = Rs. 19.13 Crs/year

Variable charge 					=  114.08 Ps/Kwh

Stage –II

Fixed cost based on the CERC order = Rs. 218.16 Crs/year.

Fixed cost payable by APDiscoms for 191.52 MW entitlement = Rs. 49.74 Crs/year

Variable charge = 114.08 Ps/Kwh

MADRAS ATOMIC POWER STATION (MAPS):

Government of India, Department of Atomic Energy (Power Section) notified the tariff for supply of power from MAPS vide ‘Tariff Notification ‘dated 7.10.2003. As per the notification the tariff effective from 1.4.2001 is fixed at 210.51 Ps/Kwh. The tariff is subject to fuel and Heavy water adjustment charges. Taking into account the revisions subsequent to 1.4.2001 in insurance and in the rates of fuel and Heavy water, the tariff applicable for September 2005 is 202.91 Ps/Kwh. The tariff applicable for September 2005 has been projected for H2 of current year and ensuing year.

Other Costs for CGS (SR)

The Other Costs for NTPC (SR), NTPC(ER), NLC, PGCIL (SR) and PGCIL (ER) has been calculated based on the average cost that the Licensee has incurred during FY 2003, FY 2004 and FY 2005.

NTPC  (SR) incentive:

Incentive for CGS (SR) for 2005-06 and 2006-07 is arrived at based on CERC notification dt.26-3-04.

Total Incentive payable        :-	Incentive shall be payable at a flat rate of 25.0 Ps/kwh for ex-bus scheduled energy corresponding to scheduled generation in excess of ex-bus energy corresponding to target PLF (80%).

As per CERC orders, the incentive amount payable to NTPC is to be shared by the beneficiaries in the ratio of  energy scheduled beyond target PLF.

### NTPC- Simhadri

The fixed costs of NTPC (Simhadri) is projected based on CERC order dt.10-5-2005 and variable cost projects is based on September’ 2005 bill.

Fixed cost for FY 2006-07: Rs. 452 crores

Variable cost	: Rs. 1.0174/unit

### APGPCL

The cost structure for purchase from APGPCL as per existing Memorandum of Understanding is as follows:

| Cost component |  |
| --- | --- |
| Demand Charges (Rs./KVA) |  |
| Stage-I | 75 |
| Stage-II | - |
| Stage-I |  |
| Fixed cost (Rs per kWh) | 0.09 |
| Variable cost (Rs per kWh) | 1.58 |
| Stage –II |  |
| Fixed cost (Rs per kWh) | 0.58 |
| Variable cost (Rs per kWh) | 1.07 |

There are no demand charges for Stage II of the generating station under the terms of the Memorandum of Understanding of April 1997.

### IPPs

GVK: - Jegurupadu Power Project

The fixed cost is fully recoverable at 68.5% PLF. Based on a calorific value of gas of 9615.18 kcal/scm, a station heat rate of 2,000 kcal/kWh and auxiliary consumption of 3 percent, average gas cost of Rs 4839.99/1000 scm, the variable cost using the PPA formula is 103.788 Ps/kWh, which has been adopted for the cost computations.

The capital cost of this plant is Rs.816 crores. The fixed cost is Rs. 167.4 Crores per annum for FY 2006 and Rs.160.34 Crores for FY 2007. The fixed cost includes foreign exchange variations payable by APDISCOMs to the company as per the provisions of Power Purchase Agreement. The actual fixed cost as settled by the Licensee may be different from the estimates as presented above on account of the monthly Foreign Exchange Rate Variation (FERV).  The Licensee submits to the Hon’ble Commission to allow the Licensee to subsequently claim the change in fixed cost on account of FERV to the Discoms.

Deemed/Notional generation claims will be payable to the company up to 85% PLF as per the incentive formulae provided in the PPA.

Spectrum:

The fixed cost is fully recoverable at 68.5% PLF. A variable rate has been computed by the Licensee based on the information provided in the bill of September 2005 drawn by the generator.  Based on the calorific value of 9811 kcal/scm, station heat rate of 2,000kcal/kWh and auxiliary consumption of 3 percent, the variable cost works out to Rs 1.0347/kWh, which has been adopted for the cost computations, with cost of gas as Rs. 4923.57 /1000scm including transportation charges.

A fixed cost of Rs 164.65 Crores has been adopted for FY 2006 and Rs158.09 crores for FY 2007.  The fixed cost is inclusive of foreign exchange variations payable by APTRANSCO to the company as per the provisions of Power Purchase Agreement.

The actual fixed cost as settled by the Licensee may be different from the estimates as presented above on account of the monthly Foreign Exchange Rate Variation (FERV).  The Licensee submits to the Hon’ble Commission to allow the Licensee to subsequently claim the change in fixed cost on account of FERV from the Discoms.

Deemed/Notional generation claims will be payable to the company up to 85% PLF as per the incentive formulae provided in the PPA.

Incentives for GVK and Spectrum:

Computations of incentives for GVK and Spectrum have been carried out based on the formula provided in the PPAs.  The PLF used for estimating incentives is based on the generator’s projections of plant performance for FY 2006 and for FY 2007.

GVK

Incentive payment 	= Equity x (PLF - 68.5).x. 0.00525

Equity 	    	= Rs 244 .8 Crores

Projected incentive for FY 2007: Rs.21.21 Crs.

SPECTRUM

Incentive payment 	= Equity x (PLF-68.5)x0.004 (IF PLF > 68.5 < 80.5)

Incentive payment 	= Equity x (PLF - 68.5).x. 0.005 (if PLF > 80.5 and < 85.5 )

Incentive payment 	= Equity x (PLF - 68.5).x. 0.006 (if PLF > 85.5 )

Equity 		    	= Rs 117.92 Crores

Projected incentive for FY 2007 : Rs.9.73 Crs.

Lanco Kondapalli:

The fixed charges are fully recoverable at 80% PLF

For computation of variable charges for the station with gas, the weighted average cost of gas, including transportation charges, is Rs.6060.5 per 1000 SCM.    Based on the gross calorific  value of 9596 kcal/SCM, station heat rate of 1900 kcal/kWh and auxiliary consumption of 3%, the variable cost works out to Rs 1.2371/kWh, which has been adopted for the cost computations (for FY 2006 and FY 2007), using the formula

Estimated fixed cost for H2 of FY 2006	:	Rs.162.12 Crs.

Estimated fixed cost for FY 2007		:	Rs.310.88 Crs.

Reliance Power Ltd. (BSES)

The Fixed charge is fully recoverable at 85% PLF. Normative auxiliary consumption of 2.5% has been assumed under normal operations.  The computations are provided below: Fixed cost = FDSC+OFC=0.6 Cents+69.9 Ps =.0.975     ( @ 1US$=Rs.46.03)

Based on the GCV of 9826 K.cal/scm , Gas cost of Rs.5409.9 per 1000 scm, Station heat rate of 1900 K.cal/Kwh, Auxiliary consumption of 2.5%, the variable cost works out to Rs.1.073/ Kwh

Based on the formula provided in the PPA, the fixed cost works out to Rs. 83.95 Crs. for second half of FY 2005-06.  The fixed cost for FY2006-07 is calculated as Rs.158.406 Crs.

### LVS Power Ltd  (36.8 MW)

LVS is a Mini Power Plant originally contemplated for sale of power to third parties.  The Commission directed the developer to sell energy to APTransco.

However the company was directed to back down the plant from 3.4.2003 onwards due to increase in variable cost.  During the back down period fixed charges payable to the company from 11.10..04 to 10.10.2005 is Rs.31.759 Crs. and fixed charges payable from 11.10.2005 to 10.10.2006 is Rs.29.74 Crs.

### SRIVATHSA POWER PROJECTS LTD ( 17.202 MW)

It is also a Mini power plant originally contemplated for sale of power to third parties. However, as directed by the Commission, the developer has agreed to sell the energy to APTRANSCO withdrawing the court case against APERC order passed in O.P.No.70-A/2001, Dt. 4.5.2001 and after series of negotiations developer furnished their proposals with a capital cost of Rs.56.88 crores. Fixed cost for 3rd year (2005-06) is Rs.1.236 per unit and Variable cost during 3rd year is Rs.1.366 per unit, 80% PLF at generator terminals.  The fixed cost for 2nd half of 2005-06 is Rs. 7.43 Crores. For the year 2006-07, fixed cost of Rs.1.228/unit and variable cost of Rs.1.366/unit has been adopted.  The annual fixed charges work out to Rs.13.508 Crs for 2006-07.

### Non Conventional Energy (NCE) Sources:

The Commission as per its Order in R.P.84/2003 in O.P.-1075.2000 dated March 23, 2004 has announced the tariff applicable to NCE projects with effect from 1.4.2004. Subsequently, the Commission has revised the incentive in these projects for generation above threshold PLF and tariff for Mini Hydel projects in its orders on review petitions filed by NCE developers/associations against orders dt.20.3.2004.  The NCE developers approached Hon’ble High Court on the Commission’s orders. Hon’ble High Court issued interim directions allowing APTRANSCO to implement the revised tariff in addition to payment of 50% of the differential amount between the old rate and revised tariff. Hon’ble High Court in June’ 2005 disposed the cases filed by NCE developers, giving liberty to the Developers to file appeals before Appellate Tribunal for Electricity, New Delhi.  NCE developers filed appeals before Appellate Tribunal.  Hon’ble Appellate Tribunal passed interim orders directing licensees to continue to pay 50% differential amount between the old rate and revised tariff in addition to payment of tariff as fixed by APERC till disposal of appeals.  APTRANSCO and Discoms have filed counter affidavit in appeals.  The appeals are partly heard.  Hon’ble Tribunal posted the appeals to 7-2-2006 for further hearing.